3GPP TSG-RAN WG2 Meeting #117-e	R3-224462

E-meeting, 15 – 24 August 2022

Agenda item:	10.4

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN3 aspects of signalling based logged MDT override protection in Rel-18

Document for:	Discussion and Decision

# 1	Introduction

The agreed Rel-18 WI on further enhancement for SON/MDT in RP-221825 defines the following objective to support special handling in case of Signalling based MDT data collection:.

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

In this contribution we analyse the applicable scenario and identify aspects with relevance for RAN3.

# 2	NR Baseline

For Signaling based logged MDT override protection, Rel-17 introduced the information on logged MDT type into MDT configuration, which is stored in the UE variable including the other logged measurements information (VarLogMeasReport).

Observation 1: NR LoggedMeasurementConfiguration is extended with “Logged MDT type” IE (sigLoggedMeasType-r17), which explicitly indicates involvement in Signaling based MDT.

The UE provides the network back with the flag sigLogMeasConfigAvailable set to true to inform it that there is one prioritized MDT session configuration in force. The stored parameter is used by the UE to indicate the Signaling MDT configuration availability in RRCSetupComplete and RRCResumeComplete.

This extension facilitates the network decision to not configure the UE with Logged MDT configuration triggered by Management based MDT, even though the UE did not manage to collect any data yet.

Furthermore, the extension of RRC procedure is built on the following basic requirement (TS 37.320):

For Logged MDT, the configuration will always be done in cells of the same RAT type. However, measurements included in the logged MDT report comprises of measurements from the same RAT type (serving cell measurements, intra-frequency and inter-frequency neighbor cell measurements) and different RAT types (inter-RAT neighbor cell measurements).

Observation 2: LoggedMeasurementConfiguration signaled over RRC is RAT specific.

In addition, the UE maintains only one MDT configuration at time (e.g. following the legacy rules, the UE will replace the previous MDT configuration).

Observation 3: The UE maintains only one MDT configuration at a time.

# 3	Rel-18 enhancement scope

The target enhancement concerns the inter-RAT scenario. I.e.: how to protect signalling based Logged MDT configured in E-UTRAN, upon reselection of an NR cell by the UE.

From RAN3 perspective it can be understood that the protection mechanism introduced by RAN2 in Rel-17 corresponds informing the network about the presence of signalling based logged MDT configuration in the UE. In case of such presence the network shall consider the UE as not being eligible for management based logged MDT configuration. It would therefore in our vie be beneficial that also RAN3 analyses the scenarios applicable in Rel-17 (where RRC corrections might still be possible if strongly needed) and scenarios to be applicable in Rel-18.

Our understanding is:

- The Rel-17 feature applies to signalling based logged MDT configuration that was received by the UE while being served under NG-RAN, both under EUTRA and NR.

- Therefore the Rel-17 feature already applies in NG-RAN intra-system inter-RAT scenario, e.g. for the case where the s-based logged MDT configuration was received when the UE was served by an NG-RAN EUTRA Pcell and later reselects and reconnects to an NR Pcell.

- The Rel-17 feature applies irrespective of the PLMNs involved in providing the initial s-based logged MDT configuration and potentially aiming at configuring m-based logged MDT.

- However no restriction applies to configuration of s-based logged MDT, e.g. s-based logged MDT configured by PLMN A will be informed to PLMN B but not prevent PLMN B to configure the UE with a different s-based logged MDT configuration. From our side we observe that specifications generally prevent inter-PLMN transfer of configuration information.

- The Rel-18 feature is intended for intra-EUTRAN and inter-system scenarios, i.e. the presence of an s-based logged MDT configuration received while the UE is served by EUTRAN shall make the UE not eligible for m-based logged MDT in case of subsequent connections to EUTRAN and NG-RAN.

We propose that RAN3 discusses the above and informs RAN2 about the outcome of the discussion, who could take the information into account e.g. to update the detailed scope of the objective of the work and/or to provide additional clarifications on the feature (in stage 2 and/or stage 3).

Proposal: RAN3 to discuss and confirm scenarios for signalling-based logged MDT protection.

# 4		Conclusion

In this contribution we made the following proposal:

Proposal: RAN3 to discuss and confirm scenarios for signalling-based logged MDT protection.