3GPP TSG-RAN WG3 #117-e	R3-224477

# Online, 15th - 24th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0245 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the example message flow for multicast MBS Context establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon Wireless, AT&T, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The example message flow for multicast MBS Context establishment is not aligned with the current status of specifications and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The example message flow for multicast MBS Context establishment was devided into three scenarios: a) a first UE joining an inactive multicast MBS session as the first UE in a gNB b) multicast session activation c) MRB type configuration change for a UE and establishment of an F1-U ptp retransmission tunnel with current stage 2/3 specification aligned. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it corrects the stage 2 specification of the multicast MBS function in NG-RAN. The impact can be considered isolated because the change affects only the stage 2 specification of the multicast MBS function in NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specification of the multicast MBS function in NG-RAN would be incomplete, misleading and contradicating other stage 2/3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figures 8.15.1.2-1, 8.15.1.2-2 and 8.15.1.2-3 illustrate an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment.

A UE joins an inactive multicast MBS Session as first UE in a gNB:

Figure 8.15.1.2-1: Multicast MBS Session Context establishment - 
a UE joins an inactive multicast MBS Session as first UE in a gNB

0.	For a certain multicast MBS session (TMGI), which is currently inactive, no multicast MBS session context is yet established in the gNB.

1.	The gNB(-CU-CP) receives for a UE within the NGAP PDU Session Modification Request message the information that the UE has joined a multicast MBS session. An multicast MBS session context is established within the gNB.

2/3. If not yet existing, the gNB-CU-CP establishes the multicast MBS session context at the gNB-CU-UP, in order to retrieve for unicast NG-U transport the GTP DL TEID, a shared resource address (GTP DL TEID).

4/5.	The UE context in the gNB-DU is modified to include the information that the UE has joined the multicast MBS session.

6./7. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

8./9. The NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the DU side F1-U TNL address and the 5GC NG-U TNL address to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

10.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

11.	The gNB successfully terminates the NGAP PDU Session Modification procedure.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU-CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

The multicast MBS session is activated:

Figure 8.15.1.2-2: Multicast MBS Session Context establishment - 
the multicast MBS session is activated

1.	The multicast MBS session is activated by the 5GC.

2./3.	The gNB-CU-CP establishes the multicast MBS session context at the DU, providing MRB configuration. It may contain MBS Area Session ID information.

4.	The gNB-DU triggers the establishment of an F1-U tunnel by means of the F1AP Multicast Distribution Setup procedure, which is established either per DU or per cell or, if applicable, per MBS Area Session ID. The decision to establish per cell or per MBS Area Session ID may depend on the joined UEs current location.

5./6.	The receiving gNB-CU-CP may need to fetch a gNB side NG-U TNL address information for the gNB-CU-UP by means of a E1AP MC Bearer Context Modification procedure.

7. The gNB-CU CP terminates the F1AP Multicast Distribution Setup procedure.

8.	The gNB-CU-CP configures MRB resources on a per UE basis.

9.	The multicast MBS media stream is provided to the UEs.

Change of the MRB type configuration with F1-U ptp retransmission tunnel establishment:

Figure 8.15.1.2-2: Multicast MBS Session Context establishment - 
change of the MRB type configuration with F1-U ptp retransmission tunnel establishment

1.	The gNB-DU decides to change the MRB type configuration of a UE and provides the new configuration to the gNB-CU-CP containing a per-MRB indication regarding the MRB type change.

2.	The gNB-CU-CP confirms the modification request and, if it decides to establish an F1-U ptp retransmission tunnel in order for the UE to perform PDCP Status Report and receive retransmitted PDCP PDUs for the reconfigured MRBs, it indicates this decision to the gNB-DU.

4. - 7.	The gNB-DU requests the establishment of an F1-U tunnel by means of triggering the F1AP Multicast Distribution Setup procedure, which in turn triggers the E1AP MC Bearer Context Modification procedure to exchange F1-U tunnel end-point addresses.

8.	The RRC Reconfiguration procedure is performed

9.	PDCP Status Report and PDCP PDU retransmission can take place via the established F1-U ptp retransmission tunnels.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>