3GPP TSG-RAN3 #117-e	R3-224488

15th August – 24th August 2022

Online

Title: 	AI/ML Network Energy Saving

Source: 	Ericsson

Agenda item:	12.2

Document Type:	Discussion and Decision

# Introduction

In RP-220635 it was agreed to start normative work on AI/ML for NG-RAN based on the outcome of the study for AI enabled RAN as documented in TR37.817.

The objectives of the WI as detailed in RP-220635 are as follows:

Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

The conclusions in TR 37.817 state the following:

The AI/ML functionality and the following use cases are recommended by RAN3 to be specified in Rel-18 normative phase:

-	AI/ML-based Network Energy Saving

-	AI/ML-based Load Balancing

-	AI/ML-based Mobility Optimization

Recommendations for each use case take the section of “Solutions and standard impacts” for each use case as basis. The high-level principles captured in section 4.1 of TR37.817 shall remain valid during normative phase, while the functional framework captured in section 4.2 of TR37.817 should be used as a guideline in normative phase.

This paper discusses inputs, outputs, and feedback of the AI/ML-based Network Energy Saving use case captured in TR37.817, regarding the standard impacts arising thereof.

# AI/ML-based Network Energy Saving use case in TR37.817

In TR37.817 the Network Energy Saving use case was analyzed. Following a general description of the use case, solutions and standard impacts were presented. Regarding solutions, relevant information was identified for the following:

- Information needed as input data to NG-RAN for AI/ML-based network energy saving in order to predict the optimized network energy saving decisions

- Information that can be generated as output by an AI/ML-based network energy saving model

- Information that can be collected from NG-RAN nodes as feedback in order to optimize the performance of AI/ML-based network energy saving model

In the following we will analyze in detail the input, output and feedback information proposed in TR37.817, but firstly we will highlight some of this information that we consider deserve more discussion.

  - Discussion on signaling of new information

### Network energy efficiency signaling between NG-RAN nodes

Current and predicted energy efficiency information from NG-RAN nodes is believed to be of paramount importance for AI/ML-based prediction of optimized network energy saving decisions. Hence, this energy efficiency information is captured in TR37.817 as input, output, and feedback of AI/ML-based Network Energy Saving. Therefore, the said information must be signaled between NG-RAN nodes via the Xn interface, as also recognized in TR37.817, to enable training as well as inference of an AI/ML model for making optimal network-level energy saving decisions. This information is currently not signaled in Xn, hence there is a need to introduce new signaling in Xn for collecting AI/ML energy saving related information. Based on that we proposed to use the following two new procedures in XnAP, as analyzed in R3-224494, to signal Network Energy Efficiency parameters between NG/RAN nodes:

- a new non-UE associated procedure, for handling the subscription mechanism. A tentative name for this procedure can be AIML Assistance Data Reporting Initiation.

- a new non-UE associated procedure, for handling the collection of subscribed information. A tentative name for this procedure can be AIML Assistance Data Reporting.

Proposal 1:  Use two new procedures in XnAP, the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, to signal Network Energy Efficiency parameters between NG-RAN nodes.

For instance, the Network Energy Efficiency parameters can be requested as shown in the table below, which only addresses the energy saving use case:

| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the assistance information is required. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| Report Assistance Information | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(64)) | Each position in the bitmap indicates assistance information the NG-RAN node2 is requested to report. First Bit = Predicted radio resources Periodic, Second Bit = Predicted Energy Efficiency Periodic, Third Bit = Current Energy Efficiency Periodic, Fourth Bit =UE Performance Indicator Periodic, Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting. | YES | ignore |

Another issue concerns the way that the energy efficiency can be expressed. In principle, the energy efficiency of an NG-RAN node can be expressed in several ways. However, absolute energy consumption values are difficult to interpret and constitute rather sensitive information. Similarly, energy efficiency in terms of Joule/bit (or bit/Joule) is highly volatile depending, among other things, on signaling conditions and traffic characteristics of served UEs and therefore misleading. For this reason, it is proposed that an energy efficiency score is used instead. This metric represents the energy efficiency as a relative score, e.g., a percentage, subject to the energy levels measured over a certain period. This approach is valid for both prediction and actual values of the energy efficiency score.

As example of how Energy Efficiency may be specified is shown below:

| Energy Effieciency |  |  | INTEGER (0..100) | Value 1 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Energy Efficiency |  |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |

Proposal 2: It is proposed that the energy efficiency of NG-RAN nodes is signaled as an energy efficiency score, where the “energy efficiency score” is an index representing the energy efficiency average over a given time period as a percentage.

### Predicted resource status from neighboring NG-RAN node(s)

Predicted Resource Status Information of neighboring NG-RAN nodes is one of the potential AI/ML assistance information considered in the TR 37.817 also for AI/ML-based Network Energy Saving.

Reusing legacy signaling has its disadvantages. Indeed, adding new features to existing procedures would not follow the principles of modular design. The addition of new requirements to existing procedures for AI/ML-related functionality may lead to a more error-prone system that is harder to analyze and debug. For this reason, we propose the use of the new dedicated procedures proposed in Proposal 1 to assist AI/ML in RAN in requesting and reporting predicted Resource Status Information of neighbouring NG-RAN nodes.

Proposal 3:  Use two new procedures in XnAP, the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, to request and report Predicted Resource Status Information of neighbouring NG-RAN nodes.

As discussed during the SI, the “predicted radio resources” mirrors the information contained in the Resource Status Update. Hence, the simplest way to define this information in the new procedures would be to define the Predicted Radio Resources IE as a sequence of predicted values for the IEs reported in the Xn: Resource Status Update message. One example of how the Predicted Radio Resources IE may look like is shown below:

#### 9.2.3.xx	 Predicted Radio Resources

This IE provides prediction values for Radio Resource parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | –- |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

### UE performance feedback

According to TR37.817, the feedback of AI/ML-based Network Energy Saving comprises the UE performance affected by an energy saving action, e.g., for handed-over UEs. Nevertheless, in many cases there are UEs that were not handed over due to the energy saving action but whose performance is impacted by the energy saving action as well. For example, if the output of an AI/ML model is a certain cell deactivation, there may be many UEs in the coverage area of the cell (that is to be deactivated) but not in RRC Connected mode at the time. In response to cell deactivation, those UEs would camp on and perhaps connect to another cell later. Similarly, there may be UEs moving into the potential coverage area of the deactivated cell later and connecting to the other cell. Further, there may be UEs already connected to the other (still active) cell, which may be affected by the cell deactivation due to load shift from the deactivated cell to the still active cell. In the examples, the UEs are just as affected by the energy saving action as the handed-over UEs (if any).

Observation 1: There are UEs not handed over due to an energy saving action whose performance is affected by the energy saving action.

An AI/ML-based solution for optimizing the RAN energy performance should not degrade the UE performance beyond a certain level. This cannot be guaranteed when only taking into account the performance of a subset of UEs affected by an energy saving action. Hence, it is vitally important to consider the performance of all UEs affected by the action. For this reason, it is proposed that the UE performance feedback comprises performance feedback for any UE affected by the action, including the UEs that were not handed over due to the action, or immediately involved in the action.

Proposal 4: It is proposed that the UE performance feedback comprises performance feedback for any UE affected by an energy saving action, including UEs not handed over or immediately involved in the action.

Besides, such UEs may not have been served by the NG-RAN node deciding on an action and receiving feedback for an action, i.e., the NG-RAN node perhaps never had these UE contexts and may not be aware of these UEs. It is thus not opportune to signal UE performance feedback using a UE associated procedure. It is proposed that UE performance feedback is signaled over Xn using a non-UE associated procedure to facilitate the inclusion of performance feedback for UEs that were affected by the action but not handed over due to the action. For UEs whose context is known, the UE Xn AP ID of the node requesting feedback will be included when reporting the UE performance feedback, allowing the node receiving the feedback to associate the UE performance measurement to the UE context.

Proposal 5: It is proposed that UE performance feedback is signaled over Xn using a non-UE associated procedure including the UE Xn AP ID for the UEs whose UE context is known.

In order to quantify how the UE performance feedback should be expressed, it is plausible to assume that the basic metrics could consist of QoS parameters such as bitrate, latency, packet error rate.

However, a certain degradation in UE performance regarding bitrate, latency, reliability, etc. does not necessarily lead to a degradation in user satisfaction. It may not be necessary that UEs achieve a similar quality of service at the neighbor NG-RAN nodes, since UEs may have been over-provisioned with respect to the applications or services in use, e.g., in case of DASH streaming. In order to facilitate best possible energy saving strategies, it is proposed that the UE performance feedback further includes RAN Visible Quality of Experience (RVQoE) measurements, when applicable.

Proposal 6: It is proposed that the UE performance feedback includes RAN Visible Quality of Experience (RVQoE) measurements.

In order to apply AI/ML techniques at the RAN level to determine the best possible energy saving configuration, it is crucial to introduce tools to evaluate the impact on the UE energy consumption. An AI/ML-based optimization of the RAN energy performance should not come at the expense of bad UE energy performance. Therefore, to support AI/ML-based Network Energy Saving solutions, it is proposed to introduce new feedback from the UE on the experienced energy efficiency regarding the current energy saving strategy. The network can learn from this feedback how a certain configuration or strategy affects the UE energy efficiency. For example, the UE energy consumption may increase significantly when turning off a certain capacity cell.

The UE could report the energy consumption in absolute value or relative to a reference value or as delta changes in energy consumption. In one alternative, the UE could report the energy efficiency within a predefined range, e.g., an integer in [0, N], where 0 indicates a low or the lowest energy efficiency and N indicates a high or the highest energy efficiency. For instance, if all UEs experience a low energy efficiency, it could motivate activating a capacity cell.

Proposal 7: A UE should indicate its current energy efficiency, which should be used as input to the AI/ML model and as feedback to train the AI/ML model.

## Inputs, outputs, and feedback captured in TR38.817

### Input of AI/ML-based Network Energy Saving

The following information is captured in TR37.817 as input of AI/ML-based Network Energy Saving:

Input information from local node:

- UE mobility/trajectory prediction

- Current/predicted energy efficiency

- Current/predicted resource status

Input information from UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurements, etc.), including cell level and beam level UE measurements

Input information from neighbouring NG-RAN nodes:

- Current/predicted energy efficiency

- Current/predicted resource status

- Current energy state (e.g., active, high, low, inactive)

We now discuss each one of the above input information:

- UE mobility/trajectory prediction: As captured in TR37.817, UE mobility/trajectory prediction is input information from the local node, i.e., gNB-CU, that can be derived at the gNB-CU, e.g., using an AI/ML model. It is node-internal information at the gNB-CU and has no impact on RAN3 specification work.

- Current/predicted energy efficiency (from the local node): The local node, i.e., gNB-CU, is aware of the current energy efficiency. With that information, the gNB-CU can predict the energy efficiency with respect to a time interval in the future, e.g., using an AI/ML model, and then use it as input of the AI/ML Network Energy Saving model. In case of disaggregated gNB, the gNB-CU can estimate the energy efficiency of connected gNB-DUs based on available information from existing F1 signaling, e.g., load metrics. The gNB-CU is aware of mobility, UE measurements, neighbour cell status, so it has the overall picture of current status and also of how load will change. Hence, the gNB-CU remains the best node to calculate energy efficiency changes.

- Current/predicted resource status (from the local node):  In case of a disaggregated gNB, the existing F1AP signaling Resource Status Reporting Initiation and Resource Status Reporting already allows a gNB-CU to obtain resource measurements from the connected gNB-DU(s). Furthermore, the gNB-CU can make resource status predictions with the available information. Hence no signaling enhancements are foreseen for this input.

- UE measurement report: The UE can be configured to measure and report the listed UE measurements, including cell-level and beam-level RSRP, RSRQ, SINR, etc., through legacy signaling. As discussed in Section 2.1.3, RAN Visible Quality of Experience (RVQoE) measurements are crucial for AI/ML-based Network Energy Saving approaches. It should thus be regarded as an input of AI/ML Network Energy Saving. In addition, as elaborated in Section 2.1.4, it is proposed that the UE energy efficiency is considered as input.

- Current/predicted energy efficiency (from neighboring NG-RAN nodes): As discussed in Section 2.1.1, it is proposed that the current and predicted energy efficiency of a neighboring NG-RAN node can be requested and received from that neighboring NG-RAN node as an energy efficiency score over the Xn interface.

- Current/predicted resource status (from neighboring NG-RAN nodes): For the case of current resource status information, this information is already available using the XnAP signaling Resource Status Reporting Initiation and Resource Status Reporting. It is possible to extend this signaling to include predicted values, but as we discuss in Section 2.1, we suggest introducing new dedicated procedures for collecting information to assist the AI/ML-based use case: AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting. Therefore, support for this input requires signaling enhancements.

- Current energy state: Since there is no definition of the energy states of NG-RAN nodes, e.g., active, high, low, or inactive, there is no clear benefit of signaling the current energy state between NG-RAN nodes. In addition, the energy state can just as well be expressed by means of the proposed energy efficiency score, e.g., (active, high, low, inactive) and can just be signaled as (95, 75, 25, 5). The energy state provides no additional information on top of the energy efficiency, i.e., there is no need to signal energy state between NG-RAN nodes in addition to energy efficiency.

Proposal 8: It is proposed not to signal the (current or predicted) energy state of an NG-RAN node over Xn because this information is included in the Energy Efficiency.

### Output of AI/ML-based Network Energy Saving

The following information is captured in TR37.817 as output of AI/ML-based Network Energy Saving:

- Energy saving strategy, such as recommended cell activation/deactivation

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy efficiency

- Predicted energy state (e.g., active, high, low, inactive)

- Model output validity time will be discussed during R18 normative work per inference output.

We now discuss each one of the above output information:

- Energy saving strategy: If the output of an AI/ML model is an energy saving strategy, such as recommended cell activation or deactivation, the output is considered as internal output only and not signaled as such. That is supported by the fact that signaling of strategies generated by an AI/ML-based Network Energy Saving model is not listed as potential standard impact in TR37.817. However, actions triggered due to such output will be signaled between NG-RAN nodes when opportune. To this end, the existing procedures defined in the XnAP, such as the NG-RAN node Configuration Update procedure and the Cell Activation procedure, can be reused and enhanced as necessary, e.g., by adding an optional pre-change notification for certain IEs.

- Handover strategy: Like the above, if the output of an AI/ML model is a handover strategy, such as recommended candidate cell(s) for taking over the traffic, the output is regarded as internal output. Based on the output, the NG-RAN node will then either trigger a regular handover or a conditional handover.

- Predicted energy efficiency: As discussed above, the energy efficiency of an NG-RAN node is paramount for AI/ML-based Network Energy Saving. Thus, the predicted energy efficiency can be the output of an AI/ML model, which can then be used as input at the local NG-RAN node as well as at neighboring NG-RAN nodes. It is thus proposed that the predicted energy efficiency of a neighboring NG-RAN node can be requested and received from that NG-RAN node over the Xn interface.

- Predicted energy state: The predicted energy state may be the output of an AI/ML model based on AI/ML model implementation. But, as described above, there is no clear benefit of signaling the energy state between NG-RAN nodes in addition to the energy efficiency, i.e., the output is considered as internal output only and not signaled to another NG-RAN node.

- Model output validity time: The output validity time discussion is depending on the type of output. If the output is an energy saving strategy or a handover strategy, the output is used by the local NG-RAN node to decide on appropriate action(s) and initiate corresponding procedure(s) as needed. Since this output is not signaled as such, there is no need to determine and signal the output validity time between NG-RAN nodes for an energy saving strategy. If the output is a predicted energy efficiency, on the other hand, the output validity time may be defined as a time window, such as the prediction horizon. Nevertheless, it is generally assumed that information received from a neighboring NG-RAN node over the Xn interface can be considered valid until new, updated information is received from that NG-RAN node. Since the benefit of signaling the output validity time between NG-RAN nodes is unclear, it is proposed not to signal the output validity time.

Proposal 9: It is proposed not to signal the output validity time and the predicted energy state to other NG-RAN nodes over Xn.

### Feedback of AI/ML-based Network Energy Saving

The following information is captured in TR37.817 as feedback of AI/ML-based Network Energy Saving:

- Resource status of neighbouring NG-RAN nodes

- Energy efficiency

- UE performance affected by the energy saving action (e.g., handed-over UEs), including bitrate, packet loss, latency

- System KPIs (e.g., throughput, delay, RLF of current and neighbouring NG-RAN node)

We now discuss each one of the above feedback information:

- Resource status of neighbouring NG-RAN nodes: For the case of current resource status information, this information is already available using the XnAP signalling Resource Status Reporting Initiation and Resource Status Reporting.

- Energy efficiency: As discussed before, the energy efficiency of an NG-RAN node is essential feedback. An NG-RAN node should be able to request and receive energy efficiency feedback, e.g., for a previous energy saving action, from a neighboring NG-RAN node over Xn. It is therefore proposed that energy efficiency feedback from a neighboring NG-RAN node can be requested and received as energy efficiency score.

- UE performance affected by the energy saving action: As discussed in the dedicated Section 2.1.3, the UE performance feedback should be signaled over Xn using a non-UE associated procedure to include performance feedback for any UEs affected by the energy saving action to which it corresponds to, and also include RAN Visible Quality of Experience (RVQoE) measurements, when applicable.

# Reporting of AI/ML information for Energy Saving

In light of the analysis above an example is given of how a signaling message could be structured to report all the AI/ML information relative to the Energy Saving use case.

| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.3.xx | Indicates the predicted values of reported resource measurements |  |  |
| >>Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| >>Predicted Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |
| >>UE Performance Indicator List | O | 1 .. < maxnoofFeedbackUEs > |  |  |  |  |
| >>>UE Performance Measurements |  |  | 9.2.3.yy |  |  |  |
| >>>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

#### 9.2.3.xx	 Predicted Radio Resources

This IE provides prediction values for Radio Resource parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | –- |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

#### 9.2.3.yy 	UE Performance Measurement

The UE Performance Measurements IE indicates performance measurements for a UE .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | 9.2.3.4 | Average overall user plane UE throughput in DL |
| Average UE Throughput DL | O |  | 9.2.3.4 | Average overall user plane UE throughput in DL |
| Average Packet Delay | O |  | 9.3.1.80 | Average value for the delay that a packet may experience. |
| Average Packet Error Rate | O |  | 9.3.1.81 | Average Packet Error Rate |
| Average RAN Visible QoE | O |  | FFS | Average RAN Visible QoE experienced by the UE on all the active services for which QoE is collected at the time of reporting |
| UE Energy Consumption | O |  | FFS | The energy consumption experienced by the UE at the time of reporting |

While the above tabulars are not definitive and need to be discussed in RAN3 and refined, they can be taken as a baseline for further work.

Proposal 10: Take as baseline the tabulars in sections 2 and 3 for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the Xn Resource Status Request Initiation and Resource Status Reporting procedures

# Conclusion

In this paper, we discussed inputs, outputs, and feedback of the AI/ML-based Network Energy Saving use case with respect to the standard impacts. The corresponding proposals are listed below.

Proposal 1:  Use two new procedures in XnAP, the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, to signal Network Energy Efficiency parameters between NG-RAN nodes.

Proposal 2: It is proposed that the energy efficiency of NG-RAN nodes is signaled as an energy efficiency score, where the “energy efficiency score” is an index representing the energy efficiency average over a given time period as a percentage.

Proposal 3:  Use two new procedures in XnAP, the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, to request and report Predicted Resource Status Information of neighbouring NG-RAN nodes.

Proposal 4: It is proposed that the UE performance feedback comprises performance feedback for any UE affected by an energy saving action, including UEs not handed over or immediately involved in the action.

Proposal 5: It is proposed that UE performance feedback is signaled over Xn using a non-UE associated procedure including the UE Xn AP ID for the UEs whose UE context is known.

Proposal 6: It is proposed that the UE performance feedback includes RAN Visible Quality of Experience (RVQoE) measurements.

Proposal 7: A UE should indicate its current energy efficiency, which should be used as input to the AI/ML model and as feedback to train the AI/ML model.

Proposal 8: It is proposed not to signal the (current or predicted) energy state of an NG-RAN node over Xn because this information is included in the Energy Efficiency.

Proposal 9: It is proposed not to signal the output validity time and the predicted energy state to other NG-RAN nodes over Xn.

Proposal 10: Take as baseline the tabulars in sections 2 and 3 for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the Xn Resource Status Request Initiation and Resource Status Reporting procedures

A CR to TS38.423, mirroring the proposals above, is available in R3-224491