3GPP TSG-RAN WG3 Meeting #117-e	R3-224497

# Online, August 15th – 24th 2022

Agenda Item:	13.3

Source:	Ericsson

Title:	Enhancements for IAB-Node Mobility

Document for:	Agreement

# Introduction

This paper discusses the following objective from the Rel-18 IAB WID:

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

# Pre-storing of configurations at mIAB-nodes

The migration procedure for mIAB-nodes will include at least F1AP and BAP reconfigurations, as often as at every HO of the mIAB-MT. Both the delivery and the application of the new configurations at the mIAB-node may take prohibitively long if not done early, causing prolonged service interruption. The normative work should hence investigate the ways to minimize service interruption caused by reconfigurations related to IAB-node mobility.

A flagship scenario for mIAB is the one involving mIAB-nodes mounted onboard public transport vehicles, such as city buses and trains. In such scenarios, the routes of public transport vehicles are usually predetermined and known in advance. This determinism can be exploited in design of enhancements for mIAB-node mobility, to reduce service interruption.

One way to exploit this determinism is to deliver and pre-store the F1 and BAP configurations at the mIAB-node sufficiently (long?) before the mIAB-node reaches connects to the area where these settings are to be applied. These configurations can be activated once the mIAB-MT connects to the corresponding cell.

Figure 1 exemplifies a journey of an mIAB-node involving inter-donor handovers (HOs) and how the pre-storing of configurations could be used. An mIAB-node travels along a route that is covered by 4 different parent IAB nodes (Parent 1, 2, 3, 4). The parent nodes backhaul their traffic through 2 donor nodes (Donor X and Donor Y). According to the figure, when the mIAB-node is in position A or B (still served by the Donor X), it receives from Donor X the F1 and BAP configurations that it should apply once it connects to the IAB Parent 3 under Donor Y.

Observation: The fact that the routes of public transport vehicles are usually pre-determined, can be used for reducing service interruption by pre-storing the F1 and BAP configurations at the mIAB-node.

Figure 1: A journey of an mIAB-node involving inter-donor handovers

Meanwhile, Figure 2 shows a possible signalling flow.

Figure 2: An illustration of pre-storing of F1 and BAP configurations at the mIAB-node.

- Proposal 1: RAN3 to study the solutions for pre-storing the F1 and BAP configurations at the mIAB-node.

# Handover signalling for UEs onboard

Once the mIAB-DU is handed over from one donor CU to another, all the onboard UEs will also needed to be handed over. As of today, the HO signalling messages pertain to individual UEs, which means that, at mIAB-DU HO, a separate HO procedure would need to be triggered for each UE. There are two obvious options:

- Option 1: Reuse legacy signalling, with individual HO procedure triggered for each UE.

- Option 2: Specify group signalling, where handover information pertaining to multiple UEs may be included in one message.

Besides the obvious drawbacks of applying the legacy solution (Option 1), such as increased signalling and processing load due to a larger number of messages, we also note that the target donor CU would not know how many UEs the mIAB-node brings along to the target, until the very last HO request for the very last UE has been received. This may, in turn, lead to the situation where the target donor CU would be able to accept only some of the onboard UEs. Not being able to accept a UE is, of course, perfectly legitimate, but the problem is that the source donor CU would realize this right in the middle of the group HO process, so it is unclear how to handle the UEs that cannot be handed over. One option would be to trigger the HO of these UEs to a stationary IAB cell, but this would postpone the tearing up the F1AP connection between the mIAB-DU and the source donor CU. On the other hand, with group signalling (Option 2), it would become apparent to the source donor CU right away whether the target donor CU will be able to accept all the onboard UEs.

Proposal 2: RAN3 to discuss signalling for group handover of UEs served by an mIAB-node, where handover information pertaining to multiple UEs may be included in one message.

# Conclusion

This paper discusses the Rel-18 IAB WID objectives related to enhancements for mobility of an IAB-node together with its served UEs. The following is observed and proposed:

Observation: The fact that the routes of public transport vehicles are usually pre-determined, can be used for reducing service interruption by pre-storing the F1 and BAP configurations at the mIAB-node.

Proposal 1: RAN3 to study the solutions for pre-storing the F1 and BAP configurations at the mIAB-node.

Proposal 2: RAN3 to discuss signalling for group handover of UEs served by an mIAB-node, where handover information pertaining to multiple UEs may be included in one message.