3GPP TSG-RAN WG3 Meeting #117 electronic                                              R3-224504

Online, August 15 – 26, 2022

Agenda item: 		13.2

Source: 		Qualcomm Incorporated

Title: 	Topology adaptation for mobile IAB

Document for:	Discussion

# 1	Introduction

The WID on Rel-18 mobile IAB holds the following objective [1]:

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2]

This contribution discusses procedures to support mobile IAB in support of this objective.

# 2	Discussion

## 2.1	Inter-donor migration (full migration)

In Rel-17, full migration was discussed for stationary IAB. In Rel-18, the discussion on full migration can continue for mobile IAB where it left off in Rel-17.

Figure 1: UE handover between cells pertaining to different logical IAB-DUs connected to separate CUs (based on [2])

In Rel-17, RAN3 assumed that during full migration, the IAB-node concurrently supports two logical IAB-DUs, where one logical IAB-DU has F1AP association with CU1 and the other logical IAB-DU as F1AP association with CU2.

The support of two concurrent logical IAB-DUs allows UEs to be handed over a cell supported by one IAB-DU to a cell supported by the other [2]. One reason for this handover is that the UE has to perform a security update since it migrates to a different CU.

These assumptions from Rel-17 can be carried over to mobile IAB.

Proposal 1: During full migration, the mobile IAB-node to concurrently support two logical DUs, referred to as source and target IAB-DUs, which have F1AP associations with CU1 and CU2, respectively.

Proposal 2: The UEs connected to the mobile IAB-node to be handed over between a cell on the source logical DU and a cell on the target logical DU.

During Rel-17, RAN3 further considered two implementation alternatives:

- Alt1: the two logical DUs use separate physical cell resources

- Alt2: the two logical DUs use the same physical cell resources

- These implementation alternatives were liaised to RAN1, RAN2 and RAN4 [2], where the discussion on separate vs. same physical resources created confusion. RAN2, for instance, raised the question if separate physical cell resources implied that the cells would use separate frequencies or if they could also be multiplexed in the time domain using the same frequency [3]. Based on this discussion, RAN2 agreed [3]:

R2 assumes that the UE need to be able to treat the separate resources as different cells on L1.

This agreement should be confirmed by RAN3 as it leaves the discussion on resource sharing to RAN1. In this manner, RAN2 and RAN3 can make progress on full migration without discussing resource sharing. RAN1 should be liaised on this agreement.

Proposal 3: RAN3 assumes that the UE needs to be able to see the cell on the source logical DU and the cell on the target logical DU as different cells on layer-1.

Proposal 4: An LS to be sent to RAN1 to discuss the resource sharing between the layer-1 source and target cells on the mobile IAB-node.

In Rel-17, RAN3 further assumed that partial migration would be the first step of full migration. This implies that the F1AP associations are carried via the mobile IAB-MT’s target path. Alternatively, it would be possible to establish the two F1AP associations via the source path before partial migration.

As the baseline, it may be the easier to assume that partial IAB-MT migration is performed as the first step of full migration. This has the advantage that full migration can be avoided in case the IAB-node moves back and forth between the two donors.

Proposal 5: As the baseline, partial migration to be considered the first step of full migration.

Further, for UE handover between the logical IAB-DUs on the mobile IAB-node, legacy procedures should be used as the baseline.

Proposal 6: UE handover between logical IAB-DUs on the mobile IAB-node to use legacy procedures as the baseline.

## 2.2	Other procedures for mobile IAB-node

Rel-17 partial migration allows the mobile IAB-MT to migrate to a new CU while the collocated mobile IAB-DU remains connected with the original CU. In some use cases, where the mobile IAB-node moves across multiple CUs within a local vicinity, it may be beneficial to allow for multiple consecutive partial migrations by the mobile IAB-MT without full migration.

Proposal 7: The mobile IAB-MT can perform multiple consecutive partial migration procedures.

Rel-17 supports CHO for the IAB-MT. The support of CHO therefore also applies to the mobile IAB-node.

Proposal 8: Rel-17 CHO for IAB is also supported for the mobile IAB-node.

Rel-17 spent a lot of time to discuss the use of “DAPS-like” solutions for IAB. The term “DAPS-like” caused confusion across RAN3 and RAN2. Further, RAN3 realized during that discussion that any “DAPS-like” solution for IAB should have to be defined by RAN2. While DAPS may have benefits for mobile IAB, it would still be RAN2 that should take the initiative to define DAPS for IAB.

Proposal 9: Any discussion on using DAPS for mobile IAB should be initiated by RAN2.

## 2.3	Topology adaptation in absence of Xn

Rel-16 and Rel-17 topology adaptation procedures assume that Xn available. For partial migration, Rel-17 IAB further assumes that IP routability is supported between the F1-terminating CU and the IAB-donor-DU2 of the non-F1-terminating CU. While these assumptions are reasonable for stationary IAB deployments, they become questionable for mobile IAB use cases, where the IAB-node traverse large distances (e.g., long distance trains or busses).

RAN3 should therefore discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability.

Proposal 10: RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability.

In case inter-donor IP routability is not available, F1-C could for instance be transported via NGAP analogue to the transport of F1-C over XnAP or X2AP as used for CP/UP separation or ENDC, respectively.

Proposal 11: For inter-donor topology adaptation in absence of inter-donor IP routability, RAN3 to consider F1-C transport over NGAP.

# Conclusion

This contribution discussed procedures to for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration. The following proposals have been made:

Proposal 1: During full migration, the mobile IAB-node to concurrently support two logical DUs, referred to as source and target IAB-DUs, which have F1AP associations with CU1 and CU2, respectively.

Proposal 2: The UEs connected to the mobile IAB-node to be handed over between a cell on the source logical DU and a cell on the target logical DU.

Proposal 3: RAN3 assumes that the UE needs to be able to see the cell on the source logical DU and the cell on the target logical DU as different cells on layer-1.

Proposal 4: An LS to be sent to RAN1 to discuss the resource sharing between the layer-1 source and target cells on the mobile IAB-node.

Proposal 5: As the baseline, partial migration to be considered the first step of full migration.

Proposal 6: UE handover between logical IAB-DUs on the mobile IAB-node to use legacy procedures as the baseline.

Proposal 7: The mobile IAB-MT can perform multiple consecutive partial migration procedures.

Proposal 8: Rel-17 CHO for IAB is also supported for the mobile IAB-node.

Proposal 9: Any discussion on using DAPS for mobile IAB should be initiated by RAN2.

Proposal 10: RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability.

Proposal 11: For inter-donor topology adaptation in absence of inter-donor IP routability, RAN3 to consider F1-C transport over NGAP.

# References

- RP-221815: Revised WID for NR_mobile_IAB; TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

- R3-212981: LS on inter-donor migration, 3GPP TSG-RAN WG3 Meeting #112-e, E-meeting, May 2021.

[3]  R3-214690: Reply LS on Inter-donor migration, 3GPP TSG-RAN3 Meeting #114-e, E-meeting, November 2021.