3GPP TSG-RAN WG3 Meeting #117 electronic                                              R3-224505

Online, August 15 – 26, 2022

Agenda item: 		13.3

Source: 		Qualcomm Incorporated

Title: 	Enhancements for mobility of IAB-node and its served UEs

Document for:	Discussion

# 1	Introduction

The WID on Rel-18 mobile IAB holds the following objective [1]:

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

This contribution discusses support for group mobility, user location information for the UEs camping on/connected to the mobile IAB-node, and coverage of onboard UEs.

# 2	Discussion

## 2.1	Support for group mobility

The term group mobility relates to enhancements that bundle the signaling for the handover of a group of UEs into a single message.

For mobile IAB, group mobility enhancements can be applied to full migration where all UEs connected to a cell on one logical IAB-DU are migrated to a cell on another logical IAB-DU.

The bundling of UE-associated information for a whole group of UEs can also be applied in other contexts. For instance, in Rel-16, the F1AP IAB UP Configuration Update messages allow bundling of UE associated FTEID information for multiple UEs.

Group mobility enhancements can be applied to F1, E1, Xn and NG. It cannot be easily applied to RRC.

Proposal 1: Group mobility enhancements refer to the bundling of UE-specific information for a group of UEs into a common message.

For the bundling of UE-associated information for UE groups, limitations due to the maximum message size need to be taken into account. Due to these limitations, partitioning of UE information into UE sub-groups may be necessary.

Proposal 2: For group mobility enhancements, the limitations of the maximum message size to be taken into account.

Group mobility enhancements should be considered after progress has been made with the baseline procedures.

Proposal 3: Group mobility enhancements should be considered after progress has been made with the baseline procedures.

## 2.2	User Location Information

The UE’s user location information includes the NCGI of the cell the UE is connected to, as well as the TAC, which is broadcasted by this cell via SIB1. The user location information is sent by the UE’s CU to the UE’s AMF.

For an IDLE or INACTIVE UE, the user location information is updated during the UE’s registration update, which occurs (1) periodically, (2) when the UE enters a new RAN Notification Are, or  (3) when the UE enters a new Tracking Area that is not contained in its registration area.

The Rel-18 SA2 Study on Architecture Enhancements for Vehicular Mounted Relay identified key issues #5 and #6 for the user location information (see Annex below). The following aspects of the issues identified by SA2 are relevant for this RAN WI, and they are within RAN3 scope:

- The NCGI and TAC reported by the CU in the user location should reflect the physical location of the UE.

- For a UE connected to a mobile IAB-DU, the NCGI in the user location may not properly reflect the UE’s physical location when the IAB-node moves across the CU coverage area. It certainly reflects an incorrect location when the mobile IAB-DU moves to another CU without updating the NCGI, as it is the case for partial migration.

- For a UE connected to a mobile IAB-DU, the TAC reported in the user location may not properly reflect the UE’s physical location if the mobile IAB-DU’s TAC broadcast in SIB1 is not geographically aligned with the TAC broadcast by the surrounding network.

Proposal 4: For a UE, that is connected to mobile IAB-node or a camping on a mobile IAB-node, RAN3 to discuss how NCGI and TAC reported in the user location to the AMF can reflect the user’s physical location.

RAN3 may consider the following options:

Option 1: The mobile IAB-node’s inter-donor migration always applies the full migration procedure. Partial migration is only used as a transitory state to avoid a potentially signalling storm from full migration. The mobile IAB-DU further broadcasts the same TAC as the surrounding stationary network. This option has the following issues:

  - The physical location of the IAB-DU’s NCGI may still vary when it travels within the CU’s coverage area.

  - The signaling load for the handover of connected UEs may be large in deployments with many CUs with small coverage area.

  - Full migration with TAC change will cause a signaling burst due to highly correlated registration updates by all IDLE and INACTIVE UEs that are camping on the IAB-DU’s cells.

Option 2: The mobile IAB-node’s inter-donor migration always applies the full migration procedure. The mobile IAB-DU broadcasts a dedicated TAC, which does not change with IAB-node mobility. During full migration, the target CU updates the AMF with the addition of the mobile IAB-DU’s dedicated NCGI/TAC, while the source CU updates the AMF with the release of this mobile IAB-DU’s dedicated NCGI/TAC. This can be done via legacy NGAP signaling. This option has the following issues:

  - The physical location of the IAB-DU’s NCGI may still vary when it travels within the CU’s coverage area.

  - The source and target CUs have to know all AMFs used by IDLE/INACTIVE UEs camping on the mobile IAB-DU.

  - This approach, even though it uses legacy signaling, needs to be verified by SA2.

Option 3: The CU includes information about the mobile IAB-node’s location to the user location. This information could, for instance, be the NCGI and TAC seen by the mobile IAB-MT on the stationary network. This option has the following issues:

  - Such an enhancement has impact on the CN and needs further discussion by SA2.

Proposal 5: RAN3 to consider the following options:

- Option 1: The mobile IAB-DU’s NCGI/TAC are the same as used by the surrounding network.

- Option 2: The mobile IAB-DU uses a dedicated TAC, and the CUs dynamically update all relevant AMFs with the mobile IAB-DU’s current NCGI/TAC.

- Option 3: The CU amends the user location information with additional information on the mobile IAB-node’s location.

## 2.3	Coverage of onboard UEs

According to the Rel-18 WIE, mobile IAB supports use cases, where the mobile IAB-node cell aims to cover onboard UEs as well as outside UEs. The WID, however, precludes optimizations for the coverage of outside UEs. For that purpose, it is necessary that the network (i.e., the CU) knows if a the IAB-node cell aims to target onboard UEs so that potential optimizations related to mobile IAB can be applied.

One example for the optimization relates to the policies applied for UE handover between mobile-IAB cells targeting onboard UEs and cells of the stationary network. In case the CU know that the mobile IAB-node moves at high speed, it may apply more stringent criteria for UE handover between cells with onboard coverage.

Proposal 6: To allow for mobile-IAB-specific optimizations, the IAB-DU to indicate if a cells aims to cover onboard users.

# Conclusion

This contribution discusses support for group mobility and user location information for the UEs camping on/connected to the mobile IAB-node. The following observations and proposals have been made:

Proposal 1: Group mobility enhancements refer to the bundling of UE-specific information for a group of UEs into a common message.

Proposal 2: For group mobility enhancements, the limitations of the maximum message size to be taken into account.

Proposal 3: Group mobility enhancements should be considered after progress has been made with the baseline procedures.

Proposal 4: For a UE, that is connected to mobile IAB-node or a camping on a mobile IAB-node, RAN3 to discuss how NCGI and TAC reported in the user location to the AMF can reflect the user’s physical location.

Proposal 5: RAN3 to consider the following options:

- Option 1: The mobile IAB-DU’s NCGI/TAC are the same as used by the surrounding network.

- Option 2: The mobile IAB-DU uses a dedicated TAC, and the CUs dynamically update all relevant AMFs with the mobile IAB-DU’s current NCGI/TAC.

- Option 3: The CU amends the user location information with additional information on the mobile IAB-node’s location.

Proposal 6: To allow for mobile-IAB-specific optimizations, the IAB-DU to indicate if a cells aims to cover onboard users.

# References

- RP-221815: Revised WID for NR_mobile_IAB; TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

- TR 23.700-05 v0.3.0 (2022-05) – Rel-18 Study on Architecture Enhancements for Vehicle-mounted Relays

# Annex

The Rel-18 SA2 Study on Architecture Enhancements for Vehicular Mounted Relay identified the following key issues related to UE location information for UEs camping on or connected to the mobile base station realy  [2]:

Key Issue #5: Support of location services for UEs accessing via a mobile base station relay Based on the requirements of TS 22.261 [3], the 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay. However, when the mobile base station relay (i.e. IAB-node) is present, the location of the cell/IAB-node serving the UEs may change, and this may affect not only positioning procedures but also regulatory services needs UE location. Therefore, this key issue needs to address: - How to support location services for the UEs served by a mobile base station relay that moves (with or without changing IAB-donor gNBs); or roams to VPLMN; NOTE: For this key issue, this study should not seek to change the LCS framework, and the coordination with the study on FS_eLCS_PH3 may be needed.

Key Issue #6: Provide cell ID/TAC of mobile base station relay for services When the mobile base station relay (i.e. IAB-node) is present, the physical location of the cell/IAB-node serving the UEs may change even without changing its IAB-donor node. Therefore, the serving cell ID/TAC of the UE may not always accurately reflect the location of the UE. This may affect services, e.g. regulatory services, tariff notifications, and etc. that rely on cell ID/TAC as location reference. Therefore, this key issue needs to address: - How to provide appropriate cell ID/TACs information (and possible its corresponding geographic area) of mobile base station relay that moves when such cell ID/TACs are needed, e.g. to assist route an emergency call to the correct PSAP, to assist to handle Lawful Interception, to assist the Public Warning System, etc. NOTE 1: For mobile base station relay that moves, whether its cell information in the System Information Broadcast (e.g. Cell ID, TAC) changes or not due to its movement has RAN dependency. NOTE 2: For mobile base station relay that moves, even the cell information in the System Information Broadcast (e.g. Cell ID, TAC) does not change, it may represents different geographic area due to the movement.