3GPP TSG-RAN WG3 Meeting # 117-e	R3-224508

# Online,  Aug 15th 2022 - Aug 24th 2022

Agenda Item:	10.2

Source:	Ericsson

Title:	NR-U enhancements for UL MLB and MRO

Document for:	Discussion, Approval

# Introduction

One of the objectives agreed in the Rel-18 WI on further enhancements of data collection for SON/MDT (RP-221825) is to support NR-U enhancements in relation to MRO and UL MLB.

In this paper we discuss some initial proposals for these topics.

# Support of NR-U for UL MLB

## Energy Detection Threshold

In Rel-17, in the support of NR-U for DL MLB, it has been agreed to signal the Energy Detection Threshold IE as part of the RESOURCE STATUS UPDATE:

| >>>>Energy Detection Threshold | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | – |
| --- | --- | --- | --- | --- | --- |

The objectives of the Rel-18 WI on SON/MDT enhancements include support of NR-U for UL MLB, so we would like to discuss whether and how to add a similar Energy Detection Threshold in UL.

At RAN3#116-e, it has been brought up to the attention of the group that the parameter ul-toDL-COT-SharingED-Threshold could be relevant for both downlink and uplink channel access and because of that, no additional EDT parameter would be needed. We would like to investigate further, to clarify whether the support of a new parameter for EDT in UL is really needed or the existing parameter for DL is enough.

The description of the ul-toDL-COT-SharingED-Threshold IE in TS 38.331 is the following:

ul-toDL-COT-SharingED-Threshold Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission as specified in TS 37.213 [48], clause 4.1.3 for downlink channel access and clause 4.2.3 for uplink channel access. This field is not applicable in semi-static channel access mode.

From the last sentence of the IE description, we observe that the IE is not relevant in case of “semi-static channel access mode” (described in TS 37.213, section 4.3).

Observation 1: The parameter ul-toDL-COT-SharingED-Threshold is not applicable in semi-static channel access mode.

If we then consider the TS 37.213, clause 4.2.3, it describes a few parameters that can be sent by a gNB to a UE, so that a UE can understand the maximum value for EDT in UL it can use (XThresh_max). Those parameters are e.g.: maxEnergyDetectionThreshold, energyDetectionThresholdOffset, ul-toDL-COT-SharingED-Threshold, ChannelAccessMode. The same clause also indicates that a UE accessing a channel on which UL transmissions are performed, shall set the Energy Detection Threshold to be less than or equal to XThresh_max. It is also clear, from to the same clause, that different UEs can use different values for EDT in UL, as long as this value is within the corresponding maximum EDT value.

So, there is more than just one parameter one could take into account to indicate the EDT for UL. However, we think that for Load Balancing purposes, a simple indication would be sufficient, and it can be the average of the maximum values of the ED Thresholds configured for the UEs. This value can be used as an indication of the average ED Threshold used for UL channel sensing.

In summary, we think a separate ED Threshold in UL is needed, and it can be defined as the average of the maximum ED Threshold used for UL channel sensing. A possible name for the parameter can be Energy Detection Threshold UL.

Therefore, we suggest to:

- Improve the current name of Energy Detection Threshold and specify that it is an Energy Detection Threshold DL.

- To support NR-U for UL MLB, add a new IE to signal the average of the maximum ED Threshold used for UL channel sensing. The proposed name for the new IE is Energy Detection Threshold UL.

Proposal 1: Change the name of Energy Detection Threshold IE into Energy Detection Threshold DL.

Proposal 2: To support NR-U for UL MLB, add a new IE Energy Detection Threshold UL to signal the average of the maximum ED Threshold used for UL channel sensing.

Since the EDT regulates the UE/gNB behavior at PHY layer is it known by the DU. Specifically for the UL, the configuration of the EDT to be used by a UE is included in the ChannelAccessConfig-r16 IE (within the ServingCellConfig IE). The configuration for the UE is actually sent by the CU to the UE, but from a functional point of view, the ServingCellConfig IE is prepared by the DU and the CU is not required to understand it.

Therefore, the DU needs to signal the Energy Detection Threshold UL to the CU, and this can be done adding the new IE in the existing F1AP RESOURCE STATUS UPDATE message.

Proposal 3: Add to F1AP RESOURCE STATUS UPDATE message a new IE Energy Detection Threshold UL.

Going back to the use of the Energy Detection Threshold UL for Load Balancing: if a node (e.g. gNB 2) receives from one of its neighbors (e.g. gNB 1) the Energy Detection Threshold UL, the gNB2 can use this information to better understand how easy/difficult would be for a UE to transmit data once it is handed over to the gNB1 after a load balancing action. To achieve this goal, the gNB1 can signal the Energy Detection Threshold UL as a new IE in the existing XnAP RESOURCE STATUS UPDATE message.

Proposal 4: Add to XnAP RESOURCE STATUS UPDATE message a new IE Energy Detection Threshold UL.

## Channel Occupancy

In Rel-17, in the support of NR-U for DL MLB, it has been agreed to signal the Channel occupancy time percentage DL as part of the RESOURCE STATUS UPDATE:

| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

As part of the Rel-18 enhancements we would like to discuss the adding of a similar Channel Occupancy percentage time in UL.

As for the case of the EDT in UL, a node (e.g. gNB 2) that receives from one of its neighbors (e.g. gNB 1) the Channel Occupancy percentage time in UL, can use the information to better understand how a UE would be served by the peer node after a load balancing action.

In split deployment, since the DU receives the UL transmission from the UE, it is the DU that is aware of the percentage of time for which the channel resources have been utilised for UL traffic. Therefore, to support Channel Occupancy percentage time in UL we need to:

- add a new IE over F1AP to signal the Channel occupancy percentage time in UL from DU to CU

- add a new IE over XnAP to signal the Channel occupancy percentage time in UL from one gNB to another gNB

Proposal 5: Add to F1AP RESOURCE STATUS UPDATE message an IE Channel occupancy time percentage UL.

Proposal 6: Add to XnAP RESOURCE STATUS UPDATE message an IE Channel occupancy time percentage UL.

A draft CR mirroring the proposals for F1AP is in Appendix A.

A draft CR mirroring the proposals for XnAP is in Appendix B.

# NR-U enhancements for MRO

During Rel-17, some initial discussions were taken, suggesting that new UE measurements are required to support NR-U in MRO, and work was postponed to Rel-18.

To detect and potentially correct connection failures due to mobility, RLF report is an essential tool for the network. If a UE declares a radio link failure due to consistent uplink LBT failures, this is captured in the current RLF report by setting the cause rlf-Cause-r16 to lbtFailure-r16.

After RLF reports are fetched from the UE, they can be sent from one gNB to another gNB to perform the root cause analysis. In case of consistent LBT failure detected by the UE, the node that has configured the UE for accessing the NR-U Channel can decide some NR-U related optimization for the detected problem during mobility, e.g. adjusting the ED Threshold used for the UE. This becomes possible if the UE can add in the RLF report:

- the value of the configured ED Threshold for the UE

- the measured RSSI

Proposal 7: RAN3 to send an LS to RAN2 for extending the RLF report with:

- the value of the configured ED Threshold for the UE

- the measured RSSI

# Conclusion

This paper focused on NR-U enhancements in relation to MRO and UL MLB and the following proposals were derived:

Observation 1: The parameter ul-toDL-COT-SharingED-Threshold is not applicable in semi-static channel acces mode.

Proposal 1: Change the name of Energy Detection Threshold IE into Energy Detection Threshold DL.

Proposal 2: To support NR-U for UL MLB, add a new IE Energy Detection Threshold UL to signal the average of the maximum ED Threshold used for UL channel sensing.

Proposal 3: Add to F1AP RESOURCE STATUS UPDATE message a new IE Energy Detection Threshold UL.

Proposal 4: Add to XnAP RESOURCE STATUS UPDATE message a new IE Energy Detection Threshold UL.

Proposal 5: Add to F1AP RESOURCE STATUS UPDATE message an IE Channel occupancy time percentage UL.

Proposal 6: Add to XnAP RESOURCE STATUS UPDATE message an IE Channel occupancy time percentage UL.

Proposal 7: RAN3 to send an LS to RAN2 for extending the RLF report with:

- the value of the configured ED Threshold for the UE

- the measured RSSI

# Appendix A – Draft CR for F1AP

3GPP TSG-RAN WG3 Meeting #117-e	R3-22xxxx

# Online, August 15th – 24th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1AP NR-U load metrics in UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support NR-U load metrics in UL: - add a new IE Energy Detection Threshold UL. - add a new IE Channel occupancy time percentage UL The existing IE Energy Detection Threshold IE is renamed as Energy Detection Threshold DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new IE Energy Detection Threshold UL. Existing IE Energy Detection Threshold is renamed as Energy Detection Threshold DL. Introduce a new IE Channel occupancy time percentage UL. ASN.1 updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Mobility Load Balancing for NR-U in UL. Impact analysis - Support for NR-U metrics in UL in introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------Start of changes-----------------------------------------------

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | - |  |
| >>>>Channel occupancy time percentage UL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold UL | M |  | INTEGER (-100..-50,…) | Average of the maximum ED Threshold used for UL channel sensing. Value is in dBm. | - |  |

-----------------------------------------------Next change-----------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

(skip unchanged)

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-energyDetectionThresholdUL,

id-ChannelOccupancyTimePercentageUL,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

-----------------------------------------------Next change-----------------------------------------------

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThresholdDL				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-energyDetectionThresholdUL	CRITICALITY ignore	EXTENSION EnergyDetectionThreshold	PRESENCE optional }|

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore	EXTENSION ChannelOccupancyTimePercentage	PRESENCE optional },

...

}

-----------------------------------------------Next change-----------------------------------------------

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType								ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-energyDetectionThresholdUL						ProtocolIE-ID ::= xx1

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= xx2

END

-- ASN1STOP

-----------------------------------------------End of changes-----------------------------------------------

# Appendix B – Draft CR for XnAP

3GPP TSG-RAN WG3 Meeting #117-e	R3-22xxxx

# Online, August 15th – 24th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XnAP NR-U load metrics in UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support NR-U load metrics in UL: - add a new IE Energy Detection Threshold UL. - add a new IE Channel occupancy time percentage UL The existing IE Energy Detection Threshold IE is renamed as Energy Detection Threshold DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new IE Energy Detection Threshold UL. Existing IE Energy Detection Threshold is renamed as Energy Detection Threshold DL. Introduce a new IE Channel occupancy time percentage UL. ASN.1 updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of Mobility Load Balancing for NR-U in UL. Impact analysis - Support for NR-U metrics in UL in introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------Start of changes-----------------------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | – |  |
| >>>>Channel occupancy time percentage UL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold UL | M |  | INTEGER (-100..-50,…) | Average of the maximum ED Threshold used for UL channel sensing. Value is in dBm. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |

-----------------------------------------------Next change-----------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

(skip unchanged)

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-energyDetectionThresholdUL,

id-ChannelOccupancyTimePercentageUL,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

-----------------------------------------------Next change-----------------------------------------------

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						NR-U-ChannelID,

channelOccupancyTimePercentageDL	ChannelOccupancyTimePercentage,

energyDetectionThresholdDL			EnergyDetectionThreshold,

iE-Extension		ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-energyDetectionThresholdUL	CRITICALITY ignore	EXTENSION EnergyDetectionThreshold	PRESENCE optional }|

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore	EXTENSION ChannelOccupancyTimePercentage	PRESENCE optional },

...

}

-----------------------------------------------Next change-----------------------------------------------

id-SDTDataForwardingDRBList										ProtocolIE-ID ::= 355

id-PagingCause													ProtocolIE-ID ::= 356

id-PEIPSassistanceInformation									ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList									ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR										ProtocolIE-ID ::= 359

id-PositioningInformation										ProtocolIE-ID ::= 360

id-UEAssistantIdentifier										ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList						ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator								ProtocolIE-ID ::= 363

id-energyDetectionThresholdUL									ProtocolIE-ID ::= xx1

id-ChannelOccupancyTimePercentageUL								ProtocolIE-ID ::= xx2

END

-- ASN1STOP

-----------------------------------------------End of changes-----------------------------------------------