3GPP TSG-RAN3 #117-e	R3-224518

15th August – 24th August 2022

Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0247 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signaling flows for support of L1/L2 based inter-cell mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to fulfil the objectives of the WID in RP-221799 (NR_Mob_enh2) a number of changes are required. This CR implements the signaling flows among UE, gNB-DU, and gNB-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce signaling flows to support for L1/L2 based inter-cell mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1/L2 based inter-cell mobility based procedures cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< Start of Changes >>>>>>

## 8.xx	L1/L2 based inter-cell Mobility

Figure 8.xx-1. Procedure for L1/L2 based inter-cell mobility in a distributed RAN architecture

- The UE sends a MeasurementReport message to the serving gNB-DU.

- The serving DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received MeasurementReport message.

- The gNB-CU initiates configuration of a candidate target cell for L1/L2 mobility and transmits a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU to create a UE context and to request a configuration of L1/L2 mobility for a candidate target cell.

- The candidate gNB-DU responds with a UE CONTEXT SETUP RESPONSE message to the gNB-CU. The message includes a L1/L2 mobility candidate target cell configuration.

- The gNB-CU transmits a DL RRC MESSAGE transfer to the serving gNB-DU including an RRCReconfiguration message, including a L1/L2 mobility candidate target cell configuration.

- The serving gNB-DU forwards the RRCReconfiguration message to the UE.

- The UE stores the configuration of a L1/L2 mobility candidate target cell and transmits a RRCReconfigurationComplete message to the serving gNB-DU.

- The serving gNB-DU transmits an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message.

- The UE transmits lower-layer (L1 or MAC, FFS) report(s) to the serving gNB-DU on L1/L2 mobility candidate target cells.

- The network decides to trigger a L1/L2 mobility serving cell change to a candidate target cell.

Editor’s note: from this step onwards, which node, e.g., serving DU or candidate DU or CU, to make decision on execution of a L1/L2 inter-cell mobilty remains FFS.

- The serving gNB-DU transmits a lower layer signal to the UE to trigger the L1/L2 mobility serving cell change. The signal indicates a target cell for L1/L2 mobility.

Editor’s note: the order of  step 10 and 11 is not defined.

- The serving gNB-DU stops transmitting DL data and transmits a DL data delivery status frame to the gNB-CU, to inform about the unsuccessfully transmitted downlink data.

- The UE stops transmitting uplink packets, changes to the target cell and starts to monitor beams in the target cell. The UE also applies/switches to the corresponding L1/L2 mobility candidate target cell configuration used to operate with the target cell.

- The UE transmits an uplink signal to the target gNB-DU indicating its C-RNTI for target cell. The gNB-DU is now able to schedule the UE in the target cell and the UE starts to transmit uplink packets.

- The target gNB-DU transmits a DL data delivery status frame, to inform the gNB-CU. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the UE through the target gNB-DU.

<<<<<< End of Changes >>>>>>