3GPP TSG-RAN WG3 #117-e	 R3-224530

Online, 15th - 24th   August 2022

Agenda Item:	23.2

Source:	Ericsson

Title:		Study on Rel-18 NR Sidelink Positioning

Document for:	Discussion

# Introduction

The following RAN3-related objective is listed in the approved Rel-18 NR Positioning SID [1]:

- Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required].

In this paper we will review the current sidelink mechanisms in NR and provide some initial observations for guidance in this SI.

# Discussion

## NG-RAN Architecture aspects

In the initial study of the positioning architecture to support Sidelink positioning and Ranging, covering both UE-based positioning and network-based positioning, we consider that the main change to the network architecture is only on the UE side, where a UE will exchange positioning information via PC5 with another UE:

Fig.1 Overall positioning architecture to support Sidelink Positioning and Ranging

The RAN1 scenarios considered in the SI are “in coverage”, “partial coverage” and “out of coverage” scenarios.

For “in-coverage” and “partial coverage” scenarios, both PC5-based and Uu(network)-based signalling can be used to support Sidelink Positioning/Ranging. In our view, the gNBs shall not be precluded from the study in all coverage use cases and scenarios of relative and absolute positioning. In fact, gNBs are necessary for providing accurate and improved positioning datum. In case cellular coverage is available, hybrid ranging/positioning (sidelink+TRPs) would improve ranging between devices and give the best results. We propose therefore to re-use the current NG-RAN architecture as much as possible for all coverage scenarios.

Observation 1: For all coverage (“in-coverage” and “partial coverage”) scenarios, network-based signalling can be used to support Sidelink Positioning/Ranging along with PC5

Proposal 1: The NG-RAN architecture is re-used as much as possible for all coverage scenarios

New methods for Sidelink positioning along with new dedicated measurements will be discussed by RAN1, namely: SL-AoA, SL-AoD, SL-TDoA and SL-SRS-RSRP, and others if needed.

These methods will be used for the calculation of a target’s UE location by another UE using Ranging technologies. In case the UE Sidelink measurements need to be relayed from one out-of-coverage UE to an in-coverage UE into the LMF, or in case of hybrid PC5+Uu positioning, it is not clear if the new Sidelink positioning measurements need to be reported to LMF via NRPPa. RAN3 can ask RAN1 if this should be supported, since F1AP and NRPPA could be impacted. RAN2 can be cc-ed in the LS for the LPP impacts.

A draft LS is proposed in the annex.

Proposal 2: RAN3 to send a LS to RAN1 (cc RAN2) asking if the new SL positioning measurements (e.g. for SL-AoA, SL-AoD, SL-TDoA and SL-SRS-RSRP) need to be reported to LMF when UE measurements are relayed from out-of-coverage to in-coverage or in hybrid PC5+Uu positioning scenarios.

## Service Authorization aspects

In order to avoid creating radio interference for other non-V2X UEs in the same cell, a V2X capable UE needs authorization from the serving gNB before transmitting in the sidelink. Currently, the authorization mechanism for NR V2X is supported in RAN3 through the NR V2X Authorized, the NR UE Sidelink Aggregate Maximum Bit Rate and the PC5 QoS Parameters IEs; all signaled over NG, F1 and Xn interfaces.

The NR V2X Authorized indication sent to target NG-RAN denotes that the UE is authorized to use V2X communication over PC5 as Vehicle UE, Pedestrian UE or both. The structure of the IE is as below.

This IE provides information on the authorization status of the UE to use the NR sidelink for V2X services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Vehicle UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Vehicle UE. Pedestrian UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Pedestrian UE.

Table 1 NR V2X Services Authorized IE.

The NR UE Sidelink Aggregate Maximum Bit Rate IE sent to target NG-RAN is to limit the data rate for sidelink. The structure of the IE is as below.

This IE provides information on the Aggregate Maximum Bitrate of the UE’s sidelink communication. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NR UE Sidelink Aggregate Maximum Bit Rate; M; Bit Rate 9.3.1.4; Value 0 is not valid, and considered as a logical error by the receiving NG-RAN node..

Table 2 NR UE Sidelink Aggregate Maximum Bit Rate IE.

The PC5 QoS Parameters IE is sent to target NG-RAN for the resource management of UE's PC5 transmission for V2X in network scheduled mode. The structure of the IE is as below.

This IE provides information on the PC5 QoS parameters of the UE’s sidelink communication for NR PC5. IE/Group Name; Presence; Range; IE type and reference; Semantics description. PC5 QoS Flow List; 1. >PC5 QoS Flow Item; 1..<maxnoofPC5QoSFlows>. >>PQI; M; INTEGER (0..255, …); PQI is a special 5QI as specified in TS 23.501 [9].. >>PC5 Flow Bit Rates; 0..1; Only applies for GBR QoS Flows.. >>>Guaranteed Flow Bit Rate; M; Bit Rate 9.3.1.4; Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9].. >>>Maximum Flow Bit Rate; M; Bit Rate 9.3.1.4; Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9].. >>Range; O; ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …); Only applies for groupcast.. PC5 Link Aggregate Bit Rates; O; Bit Rate 9.3.1.4; Only applies for non-GBR QoS Flows.. Range bound; Explanation. maxnoofPC5QoSFlows; Maximum no. of PC5 QoS flows allowed towards one UE. Value is 2048..

Table 3 PC5 QoS parameters IE.

For Proximity Service (ProSe), the same mechanism of Service Authorization is in place where AMF sends the 5G ProSe Authorized, 5G ProSe UE PC5 Aggregate Maximum Bit Rate (coded as the NR UE Sidelink Aggregate Maximum Bit Rate IE) and 5G ProSe PC5 QoS Parameters IEs to NG-RAN. This is needed to allow device discovery and service operation procedures between two UEs operating over PC5 reference point in the cell.

The structure of the IEs is as below:

This IE provides information on the authorization status of the UE to use the 5G ProSe services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. 5G ProSe Direct Discovery; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized for 5G ProSe Direct Discovery. 5G ProSe Direct Communication; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized for 5G ProSe Direct Communication. 5G ProSe Layer-2 UE-to-Network Relay; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay. 5G ProSe Layer-3 UE-to-Network Relay; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay. 5G ProSe Layer-2 Remote UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE.

Table 4 5G ProSe Authorized IE.

This IE provides information on the 5G ProSe PC5 QoS parameters of the UE’s sidelink communication for 5G ProSe services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. 5G ProSe PC5 QoS Flow List; 1. >5G ProSe PC5 QoS Flow Item; 1..<maxnoofPC5QoSFlows>. >>PQI; M; INTEGER (0..255, …); PQI is a special 5QI as specified in TS 23.501 [9].. >>5G ProSe PC5 Flow Bit Rates; 0..1; Only applies for GBR QoS flows.. >>>Guaranteed Flow Bit Rate; M; Bit Rate 9.3.1.4; Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9].. >>>Maximum Flow Bit Rate; M; Bit Rate 9.3.1.4; Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9].. >>Range; O; ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …); Only applies for groupcast.. 5G ProSe PC5 Link Aggregate Bit Rates; O; Bit Rate 9.3.1.4; Only applies for Non-GBR QoS flows.. Range bound; Explanation. maxnoofPC5QoSFlows; Maximum no. of PC5 QoS flows allowed towards one UE. Value is 2048..

Table 5 5G ProSe PC5 QoS Parameters IE.

Taking NG-AP as example, the V2X and ProSe service authorization IEs are signalled in the following messages:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

Since a Sidelink positioning UE will communicate over PC5, it needs to get the service authorization and assistance data for Sidelink positioning communication. Therefore, an equivalent authorization mechanism as for NR V2X is needed in the protocols.

Likewise, a UE that will use Ranging technology for positioning needs to get the ProSe service authorization from the serving gNB in order to support discovery of other UEs supporting Ranging for positioning.

Proposal 3: Similarly to NR V2X and ProSe, a UE shall use Sidelink positioning/Ranging only if authorized by the serving gNB based on the authorization indications received from AMF; such service authorization indications are signaled over NGAP, F1AP and XnAP.

According to the SID [1], Sidelink Positioning is targeted for V2X, public safety and commercial use cases. Thus, it can be assumed that Sidelink Positioning may always be used when the V2X service is authorized by 5GCN in the PLMN.

Similarly, Ranging for positioning may always be used when the ProSe Discovery service is authorized by the 5GCN in the PLMN.

Observation 2: It can be assumed that Sidelink Positioning and Ranging may always be used when the V2X and ProSe services, respectively, are authorized by 5GCN in the PLMN

Hence, instead of defining new authorization IEs for both Sidelink positioning and Ranging, RAN3 can discuss if the existing ones for NR V2X and ProSe in NG-AP, F1AP and XnAP can be re-used for the purpose of signalling Sidelink Positioning and Ranging Service Authorizations. This can considerably reduce the RAN3 specification impacts.

Proposal 4: RAN3 to discuss if the existing mechanism of NR V2X and ProSe Service Authorization to NG-RAN can be re-used to support of Sidelink positioning / Ranging authorization, to limit the specification impacts.

# Conclusions and Proposals

Our proposals are summarized below.

Observation 1: For all coverage (“in-coverage” and “partial coverage”) scenarios, network-based signalling can be used to support Sidelink Positioning/Ranging along with PC5

Proposal 1: The NG-RAN architecture is re-used as much as possible for all coverage scenarios

Proposal 2: RAN3 to send a LS to RAN1 (cc RAN2) asking if the new SL positioning measurements (e.g. for SL-AoA, SL-AoD, SL-TDoA and SL-SRS-RSRP) need to be reported to LMF when UE measurements are relayed from out-of-coverage to in-coverage or in hybrid PC5+Uu positioning scenarios.

Proposal 3: Similarly to NR V2X and ProSe, a UE shall use Sidelink positioning/Ranging only if authorized by the serving gNB based on the authorization indications received from AMF; such service authorization indications are signaled over NGAP, F1AP and XnAP.

Observation 2: It can be assumed that Sidelink Positioning and Ranging may always be used when the V2X and ProSe services, respectively, are authorized by 5GCN in the PLMN

Proposal 4: RAN3 to discuss if the existing mechanism of NR V2X and ProSe Service Authorization to NG-RAN can be re-used to support of Sidelink positioning / Ranging authorization, to limit the specification impacts.

# References

RP-213588, Revised SID on Study on expanded and improved NR positioning, TSG RAN Meeting #94e, Dec. 6 - 17, 2021.

# Annex – LS to RAN1

Title:	[DRAFT] LS on Reporting of Positioning sidelink measurement

Reply to:

Release:	Release 18

Work Item:	FS_NR_pos_enh2

Source:	Ericsson (to be RAN3)

To:	RAN WG1,

Cc:	RAN WG2

Contact Person:

#### Name:	Yazid Lyazidi

E-mail Address:	Yazid.lyazidi@ericsson.com

Attachments:	n/a

1. Overall Description:

RAN3 have started the discussion on Rel-18 Positioning Study Item.

RAN3 have remarked that new measurements for Sidelink positioning along with new dedicated measurements will be discussed by RAN1, namely: SL-AoA, SL-AoD, SL-TDoA and SL-SRS-RSRP, and others if needed.

In case the UE Sidelink measurements need to be relayed from one out-of-coverage UE to an in-coverage UE into the LMF, or in case of hybrid PC5+Uu positioning, RAN3 questions if the new Sidelink positioning measurements need to be reported to LMF via NRPPa.

RAN3 would like to ask RAN1 if NRPPA support should be considered in this regard.

2. Actions:

To RAN1 and RAN4:

ACTION: RAN3 kindly requests RAN1 to feedback on the above question.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#117bis-e	       10 - 18 October, 	2022	Online

RAN3#118	       14 - 18 November, 	2022	Canada