3GPP TSG-RAN WG3 Meeting #117-e 	R3-224563

E-meeting, 15 – 24 August 2022

Agenda item:	12.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Feedback Collection after an AI/ML Action

Document for:	Discussion and Agreement

# 1	Introduction

In the TR 37.817, input, output and feedback information for an AI/ML-based function is identified. In particular, the following feedback information can be collected from an NG-RAN node for AI/ML Network Energy Saving use case:

UE performance affected by the energy saving action (e.g., handed-over UEs), including bitrate, packet loss, latency.

Similarly, the following feedback information can be collected from an NG-RAN node for AI/ML Load Balancing use case:

UE performance information from target NG-RAN (for those Ues handed over from the source NG-RAN node)

The following feedback information can be collected from an NG-RAN node for AI/ML Mobility Enhancements use case:

QoS parameters such as throughput, packet delay of the handed-over UE, etc.

In this contribution, we discuss our views on some issues related to the calculation of this Feedback information.

# 2	Providing Feedback information for AI/ML

Feedback information is necessary for AI/ML actions in order to derive further training data, inference data or to monitor AI/ML Model Performance and its impact to the network through updating of KPIs and performance counters. As illustrated in Figure 1, feedback is sent after an AI/ML action is taken (step 10 in the figure below).

Figure 1 Feedback illustration for AI/ML Mobility Optimization example where AI/ML Model Training and AI/ML Model Inference are both located in a NG-RAN node

Some of the feedback information described in TR 37.817, involves UE performance measurements of handed over UEs. This performance information includes latency, packet loss rate, packet delay, throughput, etc., depending on the use case. These performance measurements may be signalled from a target NG-RAN node after a Handover is completed. However, calculating those performance measurements will require some time for their calculation at the target NG-RAN node since they involve average values. For example, a typical time window over which a counter is calculated can be around 15 mins. Depending on the different received samples, calculating UE performance measurements may take a considerable amount of time.

Observation 1: UE performance measurements of handed over UEs need to be calculated over a certain period of time to allow calculation of the average to converge.

However, during a Handover the UE context ceases to exist when a Handover is completed.

Observation 2: UE Context is typically more short-lived compared to the time needed for UE performance measurements to be calculated at a node.

Since there is no permanent or unique UE ID in the RAN, after UE context is released a source node is unable to correlate the performance information of the handed over UEs to the UEs that actually performed the handover.

Observation 3: Since there is no permanent or unique UE ID in the RAN, after UE context is released a source node is unable to correlate the performance information of the handed over UEs to the UEs that actually performed the handover.

In the lack of this correlation a source node cannot correlate feedback information to the corresponding action and hence it is unable to train or retrain an AI/ML Model accordingly.

Observation 4: A source node cannot correlate feedback information to the corresponding AI/ML action taken and hence it cannot use this feedback information to train or retrain an AI/ML Model.

One option is to send those performance measurements at UE context release to the source NG-RAN node, when the target node sends to the source node information about the completion of a handover. However, at this time it may be too early to collect the needed feedback information since a throughput or delay metric needs to be averaged over a period of time after a handover is completed at the target NG-RAN node.

Observation 5: Sending UE performance information at UE context release to the source NG-RAN node may be too early in the sense that the needed UE performance information is not yet calculated by the target NG-RAN node.

So, there is a need for a new UE context, surviving longer than a handover, and which can characterize a UE or a group of UEs being affected by an AI/ML action during AI/ML operation.

Proposal 1: RAN3 should discuss methods to allow UE context survival beyond a handover for the purpose of AI/ML Training Data Collection.

In addition, in TR 37.817 we have agreed that Feedback is sent from Target NG-RAN node where the Handover is executed to the Source NG-RAN node.

Specifically in CHO, a source NG-RAN node prepares one or more candidate Target cells and the Target cells will reserve resources. The following AI/ML Outputs for Mobility Optimization are agreed in the study:

- Predicted handover target node, candidate cells in CHO, together with the confidence of the predication

- Priority, handover execution timing, predicted resource reservation time window for CHO

However, in the agreed Feedback information there is no mechanism to control (and possibly update) the output related to the candidate cells in CHO. In particular, the number of prepared target cells (and not used by the UE) may be large which would lead to reserving a large number of resources unnecessarily. Also, CHO preparations at different NG-RAN nodes may have different effects on the nodes, e.g., the impact of an unnecessary preparation at a “highly” loaded NG-RAN node may be higher than the impact at a NG-RAN node that is “less” loaded. Therefore, feedback also from candidate cells in CHO can be useful to the Source NG-RAN node in order to better determine how many, which cells and when to prepare those for a given UE to reduce inefficient usage of resources.

Proposal 2: Feedback information from all candidate cells in CHO may be beneficial at the Source NG-RAN node to identify how many and which CHO cells as well as when to prepare those for a given UE.

# 3 	Conclusion

In this paper we make the following observations and proposals:

Observation 1: UE performance measurements of handed over UEs need to be calculated over a certain period of time to allow calculation of the average to converge.

Observation 2: UE Context is typically more short-lived compared to the time needed for UE performance measurements to be calculated at a node.

Observation 3: Since there is no permanent or unique UE ID in the RAN, after UE context is released a source node is unable to correlate the performance information of the handed over UEs to the UEs that actually performed the handover.

Observation 4: A source node cannot correlate feedback information to the corresponding AI/ML action taken and hence it cannot use this feedback information to train or retrain an AI/ML Model.

Observation 5: Sending UE performance information at UE context release to the source NG-RAN node may be too early in the sense that the needed UE performance information is not yet calculated by the target NG-RAN node.

Proposal 1: RAN3 should discuss methods to allow UE context survival beyond a handover for the purpose of AI/ML Training Data Collection.

Proposal 2: Feedback information from all candidate cells in CHO may be beneficial at the Source NG-RAN node to identify how many and which CHO cells as well as when to prepare those for a given UE.