3GPP TSG-RAN WG3 Meeting #117-e	R3-224581

# Online, 15th - 25th August 2022

Agenda Item:	15.3

Source:	Ericsson

Title:	Support of multicast reception in RRC_INACTIVE state - the bigger picture

Document for:	Discussions & Approval

# 1	Introduction

This document transposes discussions led during Rel-17 timeframe into the new promising era of Rel-18 discussions.

Those discussions have been revolving around two dimensions of 5GS limitations supporting multicast MBS services which will become more relevant with growing numbers of UEs joining the multicast MBS session:

-	static 5GS limitations

-	dynamic 5GS limitations

That transposition will place the intended Rel-18 support of multicast reception by UEs in RRC_INACTIVE state (see [1]) into the bigger picture of the overall NR MBS puzzle (or riddle) to be solved in order to achieve a final 5GS specification status that allows deployment of high performing networks, from multicast MBS session context creation throughout MBS session activation until the MBS session is released, also in case of challenging service requirements - which might be tough ones, especially for public safety applications.

# 2	Discussion

## 2.1	On 5G system limitations for multicast MBS sessions

1. Static 5GS limitations

5G System Resources are limited in general. If system resources would be not limited, 3GPP standardization work would be much less challenging. From an NG-RAN perspective, a perspective that has proven to provide most valuable input on system functions dealing with resource limitations, the following aspects can be brought forward:

1a. Limitation of number UEs in CM-CONNECTED in a cell/NG-RAN node:
An NG-RAN node cannot admit endless UEs, regardless the state the UE is held by the NG-RAN node (RRC_CONNECTED/RRC_INACTIVE). Applying a certain statistical model for the admitted UEs, the RAN has to provide a certain minimum service for the UEs expected to request resources for (individual) data transmission, and a certain statistical model regarding the effort to keep the UE in RRC_CONNECTED (mobility, measurements, link control etc.)

1b. Limitation of number Data Radio Bearers in a cell/NG-RAN node :
UEs in RRC_CONNECTED are provided with a DRB if a PDU Session Resource is established for them.

2. dynamic 5GS limitations

Dynamic 5G system limitations become mainly relevant at multicast MBS session activation, where system resources for a Multicast MBS Sessions need to be setup in a limited amount of time, hence the time interval from the application-level trigger until all UEs and system resources are configured with multicast MBS Session Resources is an indicator of the responsiveness of the system. Requirements on the responsiveness for public safety and similar services is very short and specified in TS 22.261. The following aspects that determine the responsiveness at session activation can be brought forward:

2a. Generation and Processing information required to determine the area within which UEs in CM-IDLE or RRC_INACTIVE are Group Paged at Session Activation.

2b. Communication among system entities to establish Associated PDU Session Resources for providing NG-RAN information which MBS Sessions the UE has joined.

2c. State Change of UEs from CM-IDLE or RRC_INACTIVE to RRC_CONNECTED, given the limitation of the Random Access Channels (RACH) in cells with high UE populations.

2d. Improvements in terms of Group Notification RAN resource utilization.

## 2.2	5GS limitations versus support of multicast reception by UEs in RRC_INACTIVE state

1. 5GS limitations mitigated by support of multicast reception by UEs in RRC_INACTIVE state

If NG-RAN is limited with keeping UEs in RRC_CONNECTED, respective static limitations can be mitigated by releasing UEs to RRC_INACTIVE (see 1a above) - accompanied by the fact that UEs kept in RRC_INACTIVE would not require DRB resources to be allocated (see 1b), as for UEs in RRC_CONNECTED.

If the final solution supports the reception of multicast data by UEs in RRC_INACTIVE without the UEs in RRC_INACTIVE having to change to RRC_CONNECTED at session activation first, limitations of RACH resources (see 2c) can be efficiently mitigated, and also aspects on generating/processing information for group paging of UEs in RRC_INACTIVE (see 2a) can be stretched over time with probably prioritising certain UEs to be sent to RRC_CONNECTED as quick as possible.

2. 5GS limitations still remaining with support of multicast reception by UEs in RRC_INACTIVE state

Aspect 1a, limitations regarding UEs able to be kept in RRC_INACTIVE within a gNB or a cell, would still exist and would need to be considered for public safety services with respective requirements. It is worth repeating from aspect 1a, that admission of UEs being served in CM-CONNECTED by NG-RAN and kept in RRC_INACTIVE still have to consider a certain likelihood for those UEs to return to RRC_CONNECTED for unicast data transmission, i.e. admission control will have to set a realistic limit for the overall number of UEs admitted in CM-CONNECTED.

Further, although it can be assumed, that RRC/CM state-control for UEs will be adapted with the introduction of NR MBS, it can be also assumed that realistic implementations will send UEs to RRC_/CM-IDLE after a certain amount of inactivity time (for both, unicast and multicast traffic) - at least implementations should be supported that do not require to keep joined UEs endlessly in CM-CONNECTED, which shows, that for rough service requirements, even with the introduction of support of multicast reception by UEs in RRC_INACTIVE state, basically all system limitation aspects listed above as still in place.

# 3	Conclusion and Prospect for obviously remaining issues

We have tried to paint a bigger picture for support of multicast reception in RRC_INACTIVE state and give a first analysis based on discussions already led during Rel-17 timeframe and during Rel-18 WID preparation.

It became clear that this sub-task of the WID in [1] provides some mitigation to scalability issues for realisation of MBS services with large UE populations by means of multicast MBS sessions, but this does not solve all issues.

From a system point of view, RACH congestion and limitations in terms of admitting UEs in CM-CONNECTED are the most important additional topics that should be looked at in Rel-18 with the prospect of aiming a truly implementable and deployable overall system concept for NR MBS.

Proposal:	Regardless the RAN WG where the main discussions are led and respective protocol solutions are developed - it is proposed to include into discussions for support of multicast reception in RRC_INACTIVE state considerations on RACH congestion handling and limitations of gNBs in terms of admitting UEs in CM-CONNECTED.

# 4	References

[1]	RP-213568 "New WID: Enhancements of NR Multicast and Broadcast Services" approved at RAN#94-e