3GPP TSG-RAN WG3 Meeting #117-e	R3-224595

E-meeting, 15 Aug – 24 Aug, 2022

Title: 	UE location verification

Source: 	Huawei

Agenda item:	17.3

Document Type:	Discussion

1. Introduction

In the Rel-18 WID [1] for NR NTN enhancements, the objective on network verified UE location is included as following:

Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]. RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

And recommendations in the SI [2] are as follows:

TS 38.882 Section 5 The verification should be performed independently from the location information reported by UE. The UE location information for the study is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing). The solution should not impact significantly the latency of the targeted services nor infringe privacy requirements that apply to the UE location. The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects: - The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. - Multiple satellite (or HAPS) in view by the UE may be considered if time allows - Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning - Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded - When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded. - Solutions using existing NG-RAN architecture and procedures shall be considered.

In this paper, we provide some initial considerations on this objective.

2. Discussion

Before considering any enhancements in RAN3, we note that there were some discussions regarding the location verification in other WGs. For example, in R17, RAN2 discussed the UE location reporting and received the LS [3] from SA3LI which also cc RAN3. According to the LS, the network needs to verify the reported UE location and SA3li suggests to verify the UE location using the LCS procedures.

Question 2: RAN2 would like to ask SA3 and SA3-LI to confirm whether A-GNSS based UE location information, i.e. computed at network using A-GNSS based measurements provided by UE, or computed by UE, can be considered reliable e.g. for lawful interception. SA3LI notes that any method which relies solely on UE-generated location information is unlikely to be considered reliable for network selection purposes. Therefore, a method such as GNSS/A-GNSS cannot be considered as reliable or trusted unless the information provided by the UE can be verified by the network. In the event that the available location information is insufficient for the AMF to determine the UE location with comparable accuracy and reliability to terrestrial networks, SA3LI considers that invocation of LCS procedures via the LMF may be necessary to fulfil regulatory obligation.

Observation 1: SA3LI suggests to use the LCS procedures to verify the UE location

In LCS procedures, there are two main steps.

-	signal measurements; and

-	position estimate and optional velocity computation based on the measurements.

The signal measurements may be made by the UE or by the serving gNB. The position estimate computation may be made by the LMF

Also according to the recommendations in the SI [2], the solutions using existing NG-RAN architecture and procedures shall be considered and the existing 3GPP defined RAT dependent positioning methods shall be considered as baseline.

Therefore we suggest that it is the LMF to verify the UE location. In the meanwhile, we understand RAN3 cannot decide a solution, it is suggested to discuss the potential way forwards, and wait for SA2 and RAN2’s progress.

Proposal 1: It is suggested to let LMF to verify the UE location using the LCS procedures, RAN3 should discuss the potential way forwards, and wait for SA2 and RAN2’s progress.

In TN, RAT-dependent positioning methods, including NR E-CID, multi-RTT, DL-AoD, UL-AOA, DL-TDOA, UL-TDOA, are introduced to determine the position of UEs under the accuracy requirement of meters or even centimeters [4]. To reach such accuracy, multiple radio measurements, such as RSRP, RSRQ, TDOA and Rx-Tx time difference, are generally required to be performed and transferred between UE, gNB, and LMF.

In our understanding, although RAT-dependent positioning methods can be applied to verify the UE reported location, some NTN specific issues may limit their usage and performance.

Firstly, due to the possibly large cell coverage of satellites, solely utilizing cell ID is not a viable solution even under such a low requirement for accuracy.

Secondly, methods like NR E-CID, which depend on signal level and quality, are also not directly applicable as measurements such as RSRP and RSRQ are less correlated with geographical positions due to the near-far effect.

Thirdly, angle-based positioning methods needs extra work in 3GPP. Angle-based positioning methods are employed to estimate the UE location via angle-of-arrival (AoA) measurements at multiple RPs, where the antenna configuration at RPs plays a key factor. However in transparent mode NTN, only a typical reflector antenna at satellite is studied and there is no discussion about the linear array or phased array antennas. Therefore the AOA is not suitable.

Fourth, methods such as DL-TDOA and UL-TDOA relying on TDOA are also not directly applicable. DL-TDOA and UL-TDOA methods in general make use of the DL RSTD of downlink signals (e.g., PRS) received from multiple TPs and the UL-RTOA at multiple RPs of uplink signals (e.g., SRS) transmitted from UE, respectively. According to the recommendations in the SI [2], the scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. Therefore the DL-TDOA and UL-TDOA are not suitable.

Finally, we find that method such as multi-RTT positioning can be potentially used. The UE measures Rx-Tx time difference of downlink signals from the satellite at different time instants, and the satellite measures the corresponding Rx-Tx time difference of uplink signals transmitted from the UE.

Proposal 2: Multi-RTT positioning method should be studied for verification of the UE reported location.

3. Conclusion

In this contribution, we discussed the network verified UE location and have the following observation and proposals:

Observation 1: SA3LI suggests to use the LCS procedures to verify the UE location

Proposal 1: It is suggested to let LMF to verify the UE location using the LCS procedures, RAN3 should discuss the potential way forwards, and wait for SA2 and RAN2’s progress

Proposal 2: Multi-RTT positioning method should be studied for verification of the UE reported location.

4. Reference

- RP-221819, NR NTN (Non-Terrestrial Networks) enhancements

- TS 38.882, Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR.

- R2-2102679, Reply LS on UE location aspects in NTN

- TS 38.305, User Equipment (UE) positioning in NG-RAN