3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224600

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	16.2

Source:	Qualcomm Inc.

Title:	Authorization for multi-path and U2U relays

Document for:	Discussion and Decision

# Introduction

In this paper, we look at SA2 progress on the authorization aspects for multi-path and U2U relays in Rel-18 and look at the potential signalling support needed in RAN3 if SA2 concludes to support authorization.

# Discussion

  - Authorization for Multi-Path relays

SA2 in 23.700-33 has captured the following objectives as part of their Key Issue to support authorization for multi-path relays:

Whether and how the network authorizes and the triggers for connection establishment for multi-path transmission, including:

- Whether and how to authorise a Remote UE to use the multi-path transmission for specific ProSe service(s).

- What information is required for and how does a Remote UE or UE-to Network Relay or the network trigger the multi-path connection establishment.

- How to provide/update the rules for multiple-path transmission.

Solution #26 proposed for the above key issue has the following impacts relevant to RAN3. We think the solution makes sense and we can have the AMF indicating to NG-RAN whether a UE is authorized to use multi-path transmission via a L2 U2N relay.

| Solution #26: Multi-path transmission via Layer-2 UE-to-Network Relay | NG-RAN: - Multi-path connection establishment and the transfer of data via both paths, taking into account authorization from AMF. AMF: - Provide authorization to NG-RAN. |
| --- | --- |

Proposal 1: if SA2 concludes that AMF should indicate to NG-RAN whether a UE is authorized to use multi-path transmission via Layer-2 UE-to-Network Relay, RAN3 agrees to provide signaling support for the same.

  - Authorization for U2U relays

SA2 in 23.700-33 has captured the following objectives as part of their Key Issue to support authorization for U2U relays:

Whether and how the network can control UE-to-UE Relay operation, at least including how to:

- Authorize the UE-to-UE Relay, e.g., authorize a UE as UE-to-UE Relay.

- Authorize Source/Target UEs to use a UE-to-UE Relay.

- Provisioning policy and parameters for UE-to-UE Relay service.

| Solution #8: Authorization for 5G ProSe UE-to-UE Relay Service | AMF: - Determine whether UE is authorized to be a 5G ProSe Remote UE/UE-to-UE Relay; |
| --- | --- |

As captured in Solution #8, we think that it is sufficient for AMF to determine whether a UE is authorized to be a U2U relay and there is no need for NG-RAN to be aware whether a U2U relay is being used.

Proposal 2: There is no need for NG-RAN to be aware whether a U2U relay is being used and hence there is no separate authorization needed at NG-RAN for U2U relay operation

# Conclusion

Proposal 1: if SA2 concludes that AMF should indicate to NG-RAN whether a UE is authorized to use multi-path transmission via Layer-2 UE-to-Network Relay, RAN3 agrees to provide signaling support for the same.

Proposal 2: There is no need for NG-RAN to be aware whether a U2U relay is being used and hence there is no separate authorization needed at NG-RAN for U2U relay operation

# 4. References