3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224607

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	10.2

Source:	Qualcomm Inc.

Title:	SON enhancements for NR-U and RACH optimization enhancements

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues and provide proposals for the same.

# Discussion

  - SON enhancements for NR-U

### MLB for NR-U

The gNB performs channel access mode procedures as described in TS 37.213. The gNB and the UE may apply Listen-Before-Talk (LBT) before performing a transmission on a cell configured with shared spectrum channel access. When LBT is applied, the transmitter listens to/senses the channel to determine whether the channel is free or busy and performs transmission only if the channel is sensed free.

Observation 1: gNB performs LBT before DL transmissions and UE performs LBT before UL transmissions

Rel-17 already supports MLB for NR-U in DL to some extent e.g., by exchanging Channel Occupancy percentage (the percentage of time for which the channel resources have been utilized for DL traffic served by the corresponding cell) and Energy Detection Threshold per NR-U channel ID in Resource status update procedure.

Observation 2: Rel-17 already supports MLB for NR-U in DL by exchanging Channel Occupancy percentage and Energy Detection Threshold per NR-U channel ID in Resource status update procedure.

Proposal 1: RAN3 should identify load metrics that are needed to be exchanged between NG-RAN nodes to support MLB for NR-U in UL.

### Optimizing Consistent LBT failures

Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity.

When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell than the SCell(s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE.

When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select.

Observation 3: gNB-DU is already aware of consistent UL LBT failures on SpCell or SCell(s) via the reception of LBT failure MAC CE from UE, which includes the cell ID(s) where the consistent UL LBT failures happened

For PSCell, if consistent uplink LBT failures are detected on all the UL BWPs with configured RACH resources, the UE declares SCG RLF and reports the failure to the MN via SCGFailureInformation.

For PCell, if the uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF.

Observation 4: Rel-17 already supports UE indicating appropriate cause values for RLF and SCG failures due to consistent UL LBT failures in RLF Report and SCGFailureInformation respectively

Proposal 2: Rel-18 can study solutions to optimize RACH failures, connection establishment failures and handover failures due to consistent UL LBT failures

  - RACH optimization enhancements

In Rel-17, RACH optimization is supported by UE reported information made available at the NG RAN node and by PRACH parameters exchange between NG RAN nodes.

### Feature combination specific RACH configurations

In Rel-17, 3GPP introduce RACH resource partitioning based on feature combinations. The network can associate a set of RACH resources with feature(s) applicable to a RA procedure: Network Slicing, RedCap, SDT, and NR coverage enhancement.

Observation 5: 3GPP introduced feature combination specific RACH configurations in Rel-17, which enables the network to associate a set of RACH resources with feature(s) applicable to a RA procedure: Network Slicing, RedCap, SDT, and NR coverage enhancement.

A set of RACH resources associated with a feature is only valid for random access procedures applicable to at least that feature; and a set of RACH resources associated with several features is only valid for random access procedures having at least all of these features.

The UE selects the set(s) of applicable RACH resources, after uplink carrier (i.e., NUL or SUL) and BWP selection and before selecting the RA type

The following is from TS 38.331, which highlights the feature or feature-combination specific RACH configurations in addition to the already existing rach-ConfigCommon and msgA-ConfigCommon

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

…

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                  OPTIONAL,   -- Need M

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }              OPTIONAL    -- Cond SpCellOnly2

[[

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }       OPTIONAL, -- Cond SpCellOnly2

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                     OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                 OPTIONAL,  -- Need R

...

}

maxAdditionalRACH-r17                    INTEGER ::= 256      -- Maximum number of additional RACH configurations

additionalRACH-ConfigList

List of feature or feature combination-specific RACH configurations, i.e., the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to a feature or feature combination.

Observation 6: There can be up to 256 additional RACH configurations (feature or feature-combination specific RACH configurations) which can be configured in BWP-UplinkCommon, but only a maximum of 16 PRACH configurations (per UL and SUL) can indicated over F1/Xn

Proposal 3: Exchange the additional RACH configurations (feature and feature-combination specific RACH configurations) over Xn and F1

### RA Report enhancements

In Rel-17, the contents of the RA Report from UE comprise of the following:

- Contention detection indication per RACH attempt

- Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts

- Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold per RACH attempt

- TAC of the cell in which the UE performs the RA procedure

- Frequency location related information of the RA resources used by the UE

- For 2-step RA, the following information can be additionally included:

  - The measured RSRP of DL pathloss reference obtained just before performing RACH procedure (per RA procedure)

  - Indication that fallback from 2-step RA to 4-step RA was performed by the UE, as per RACH attempt

  - Indication of RA switching point (as defined by the field msgA-TransMax in TS 38.331)

  - The payload size available in the UE buffer at the time of initiating the 2-step RA procedure, without considering the padding (per RA procedure)

  - MSGA PUSCH resources for 2-step RACH can be included in case the UE uses random access resources configured with CFRA

Considering the introduction of feature-combination specific RACH resources, it is proposed to study potential optimizations in RA report in Rel-18 e.g., UE can indicate the feature combination of the RA resource used in RA report.

Proposal 4: RAN3 should study whether any enhancements to RA report are needed to optimize feature combination specific RACH resources and LS RAN2 in future meetings if there are any enhancements identified

### RACH indication from gNB-DU to gNB-CU

Currently there is no RA report availability indication over Uu and gNB-CU retrieves the RA report only based on its implementation. There are also some RACH causes e.g., beam failure recovery failure, no UL synchronization which are not aware of the gNB-CU. Hence gNB-CU might fail to retrieve the RA report in time e.g., the maximum of 8 RA report entries might already got occupied or would have to depend on network signaling (Xn) to retrieve the RA reports from neighbor nodes and then filter the appropriate reports.

This issue was discussed in Rel-16/Rel-17 as well and was deprioritized to Rel-18. We therefore propose to specify support for gNB-DU to send a RACH indication to gNB-CU over F1AP.

Proposal 5: gNB-DU can send a RACH indication to gNB-CU over F1AP

# Conclusion

Observation 1: gNB performs LBT before DL transmissions and UE performs LBT before UL transmissions

Observation 2: Rel-17 already supports MLB for NR-U in DL by exchanging Channel Occupancy percentage and Energy Detection Threshold per NR-U channel ID in Resource status update procedure.

Proposal 1: RAN3 should identify load metrics that are needed to be exchanged between NG-RAN nodes to support MLB for NR-U in UL.

Observation 3: gNB-DU is already aware of consistent UL LBT failures on SpCell or SCell(s) via the reception of LBT failure MAC CE from UE, which includes the cell ID(s) where the consistent UL LBT failures happened

Observation 4: Rel-17 already supports UE indicating appropriate cause values for RLF and SCG failures due to consistent UL LBT failures in RLF Report and SCGFailureInformation respectively

Proposal 2: Rel-18 can study solutions to optimize RACH failures, connection establishment failures and handover failures due to consistent UL LBT failures

Observation 5: 3GPP introduced feature combination specific RACH configurations in Rel-17, which enables the network to associate a set of RACH resources with feature(s) applicable to a RA procedure: Network Slicing, RedCap, SDT, and NR coverage enhancement.

Observation 6: There can be up to 256 additional RACH configurations (feature or feature-combination specific RACH configurations) which can be configured in BWP-UplinkCommon, but only a maximum of 16 PRACH configurations (per UL and SUL) can indicated over F1/Xn

Proposal 3: Exchange the additional RACH configurations (feature and feature-combination specific RACH configurations) over Xn and F1

Proposal 4: RAN3 should study whether any enhancements to RA report are needed to optimize feature combination specific RACH resources and LS RAN2 in future meetings if there are any enhancements identified

Proposal 5: gNB-DU can send a RACH indication to gNB-CU over F1AP

# 4. References