3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224610

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	11.2

Source:	Qualcomm Inc.

Title:	QMC for new service types and high mobility scenarios

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss how to support QoE Measurement Collection (QMC) for new service types and high mobility scenarios in Rel-18

# Discussion

  - QMC for new service types

In Rel-17, QMC was only supported for DASH, VR and MSTI. In Rel-18, QMC can be extended to other service types such as AR, MR, MBS depending on SA4 support.

Observation 1: SA4 is studying QoE metrics for XR (e.g., AR/MR) in Rel-18 FS_ARMRQoE Study Item and specifying QoE metrics for AR Media in Rel-18 MeCar Work Item

Observation 2: SA4 has already defined QoE metrics for MBMS in TS 26.346 but has not defined any QoE metrics for MBS till Rel-17. SA4 might study QoE metrics for MBS in Rel-18

Proposal 1: Wait for SA4 progress before supporting QoE for AR, MR and MBS

RVQoE for new service types and existing service types

Rel-17 supported RAN Visible QoE (RVQoE) for only DASH and VR services and defined RVQoE metrics – buffer level and Playout delay

Proposal 2: RAN3 to identify if there are any RVQoE metrics of interest for AR, MR and MBS once SA4 defines the QoE metrics

  - QMC in high mobility scenarios

One of the objectives in the Rel-18 QoE WID is to support QMC in high mobility scenarios. It is not clear on the scenarios to be considered under high mobility. Potential scenarios could include high mobility scenarios determined by the mobility state of the UE e.g., high speed trains or studying how to support QMC in High-Speed Dedicated Networks (HSDN).

Proposal 3: RAN3 should discuss which of the following scenarios should be considered under high mobility scenarios:

- Case 1: UE is in high mobility state

- Case 2: UE is camped on a HSDN cell

- Case 3: UE is in high mobility state and camped on a HSDN cell

Proposal 4: RAN3 should discuss whether there are solutions needed to:

- Restrict QoE configuration to UEs in high mobility scenarios

- Restrict QoE measurement collection from UEs in high mobility scenarios

- Restrict QoE reporting from UEs in high mobility scenarios

Proposal 5: RAN3 should also discuss whether enhancements to QoE report e.g., a mobility state indicator (high/medium/normal mobility) are needed to better optimize QoE in high mobility scenarios

# Conclusion

Observation 1: SA4 is studying QoE metrics for XR (e.g., AR/MR) in Rel-18 FS_ARMRQoE Study Item and specifying QoE metrics for AR Media in Rel-18 MeCar Work Item

Observation 2: SA4 has already defined QoE metrics for MBMS in TS 26.346 but has not defined any QoE metrics for MBS till Rel-17. SA4 might study QoE metrics for MBS in Rel-18

Proposal 1: Wait for SA4 progress before supporting QoE for AR, MR and MBS

Proposal 2: RAN3 to identify if there are any RVQoE metrics of interest for AR, MR and MBS once SA4 defines the QoE metrics

Proposal 3: RAN3 should discuss which of the following scenarios should be considered under high mobility scenarios:

- Case 1: UE is in high mobility state

- Case 2: UE is camped on a HSDN cell

- Case 3: UE is in high mobility state and camped on a HSDN cell

Proposal 4: RAN3 should discuss whether there are solutions needed to:

- Restrict QoE configuration to UEs in high mobility scenarios

- Restrict QoE measurement collection from UEs in high mobility scenarios

- Restrict QoE reporting from UEs in high mobility scenarios

Proposal 5: RAN3 should also discuss whether enhancements to QoE report e.g., a mobility state indicator (high/medium/normal mobility) are needed to better optimize QoE in high mobility scenarios

# 4. References