3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224611

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	11.2

Source:	Qualcomm Inc.

Title:	QoE in RRC_IDLE and RRC_INACTIVE for MBS broadcast service

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss how to support QoE measurement collection in RRC_IDLE and RRC_INACTIVE for MBS broadcast service.

# Discussion

  - QoE configuration in RRC_IDLE and RRC_INACTIVE

SA4 has defined QoE metrics for MBMS in TS 26.346 but has not defined any QoE metrics for MBS till Rel-17. It is our understanding that SA4 plans to study QoE metrics for MBS in Rel-18 and RAN2/RAN3 can discuss mechanisms to support QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS (at least for broadcast service), conditional on the fact SA4 will define QoE metrics for MBS in Rel-18.

Proposal 1:  RAN3 should discuss mechanisms to support QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS broadcast service, assuming that SA4 will define QoE metrics for MBS in Rel-18

In Rel-17, a UE in RRC_CONNECTED can be provided a QoE configuration via dedicated RRC signaling (RRCReconfiguration). With the objective to support QoE measurement collection (QMC) in RRC_IDLE and RRC_INACTIVE for broadcast service in Rel-18, how to define and provide the QoE configuration for RRC_IDLE and RRC_INACTIVE is to be discussed. One option is have a different configuration for RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE similar to immediate MDT for UEs in RRC_CONNECTED and logged MDT for UEs in RRC_IDLE/RRC_INACTIVE. But unlike MDT which have different optimization objectives cases in different RRC states, QoE is application layer measurement and should be agnostic of RRC state. Hence a common QoE configuration should be defined for all RRC states.

Proposal 2: A common QoE configuration should be defined for all RRC states for MBS i.e., there is no need to define a separate QoE configuration for RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE for MBS broadcast service.

Proposal 3: It is up to RAN2 to decide how to signal this common QoE configuration for MBS to the UE e.g., via MCCH and/or dedicated RRC signaling. LS RAN2 seeking signaling support for this common QoE configuration for MBS.

  - Identifying QoE configuration upon reconnecting from RRC_IDLE

In Rel-17, the UE releases all the QoE configurations upon entering RRC_IDLE state. However, this release of QoE configuration should not be done for MBS broadcast service upon entering RRC_IDLE

Proposal 4: UE should not release the QoE configuration for MBS broadcast service upon entering RRC_IDLE state. RAN2 should provide corresponding support in the specifications

Also, the gNB releases the UE context upon going to RRC_IDLE and hence the new gNB to which UE reconnects after transitioning from RRC_IDLE has no knowledge of the MBS broadcast QoE configuration (including measConfigAppLayerID, QoE Reference and MCE IP address) configured by the old gNB. Therefore, even if UE doesn’t release the MBS broadcast QoE configuration upon entering RRC_IDLE and continues to collect QoE measurements in RRC_IDLE, the new gNB would be unable to forward the QoE reports to MCE.

Observation 1: Upon releasing the UE to RRC_IDLE, gNB releases all UE context and hence has no more knowledge of the QoE configuration

Proposal 5: RAN3 should discuss solutions on how the new gNB to which UE reconnects after transitioning from RRC_IDLE can forward the QoE reports to MCE in case QoE reports associated to the old gNB's MBS broadcast QoE configuration are received in the new gNB

One solution would be to define an MCE-ID (similar to TCE-ID in case of logged MDT) which can be mapped uniquely to an MCE IP address and this mapping can be configured by OAM at all NG-RAN nodes. This MCE-ID can be provided to the UE in the common QoE configuration for MBS broadcast service and UE can add the MCE-ID together with the QoE report sent for the MBS broadcast service

Proposal 6: An identifier for MCE which can be uniquely mapped to an MCE IP address should be defined and OAM should configure this mapping to each NG-RAN node. LS SA5 to check if such an identifier and mapping is feasible.

Proposal 7: NG-RAN should include at least the MCE ID in the common QoE configuration for MBS broadcast service sent to the UE

Proposal 8: UE should indicate the MCE ID in the QoE report associated to the MBS broadcast QoE configuration sent after transitioning from RRC_IDLE to RRC_CONNECTED

  - Reporting QoE measurements collected in RRC_INACTIVE/RRC_IDLE

Upon collecting QoE measurements in RRC_IDLE and RRC_INACTIVE, UE APP forwards the QoE report to UE AS either periodically or some event trigger being met at UE APP. Now, the question is whether UE should trigger a service request or resume upon the arrival of QoE reports at UE AS or whether the QoE reports be simply stored at UE and should be reported only after transitioning to RRC_CONNECTED for some other reason. Reporting QoE measurements is not a high priority and worth requesting UE to transition to RRC_CONNECTED.

Proposal 9: Reuse Push-based reporting via SRB4 for QoE measurements collected in RRC_IDLE and RRC_INACTIVE i.e., UE should send QoE report upon reception from APP layer and if SRB4 is configured and should not trigger a connection establishment just to report QoE

Proposal 10: QoE measurements collected in RRC_IDLE/RRC_INACTIVE should be stored at the UE and should be reported once the UE is back to RRC_CONNECTED

Proposal 11: It is up to RAN2 to decide whether to store the QoE measurements collected in RRC_IDLE/RRC_INACTIVE in AS layer or APP layer.

If the new gNB to which UE reconnects after transitioning from RRC_IDLE is unaware that a QoE for MBS broadcast is configured at the UE, it might ending up not configuring a SRB4 bearer which is used to collect the QoE reports. RAN3 should therefore discuss how the new gNB is made aware that an MBS broadcast QoE configuration was provided to the UE prior moving to RRC_IDLE.

One solution would be for the UE to indicate the availability of MBS broadcast QoE measurements to the reconnected NG-RAN node so that it can configure SRB4 for retrieving the stored MBS broadcast QoE reports and also avoid overwriting with a new MBS QoE configuration

Proposal 12: UE can indicate the availability of MBS broadcast QoE measurements to the reconnected NG-RAN node so that it can configure SRB4 for retrieving the stored MBS broadcast QoE reports and avoid overwriting with a new MBS broadcast QoE configuration

  - Area scope handling for QMC in RRC_INACTIVE/RRC_IDLE

NG-RAN has no knowledge of the UE location in RRC_IDLE and hence won’t be able to release the QoE configuration upon UE moving outside the area scope for QMC. If so, UE ends up collecting QoE measurements even outside the area scope of QMC, which is detrimental both from a UE power and potentially legal perspective (e.g., if UE measures QoE in restricted areas). Hence, an area configuration for QMC in RRC_IDLE/RRC_INACTIVE should be provided to the UE and UE should perform the area scope check.

Proposal 13: An area scope for QMC in RRC_IDLE/RRC_INACTIVE should be provided to the UE and UE should collect QoE measurement in RRC_IDLE/RRC_INACTIVE only if it is within the configured area scope. Whether this area scope check should be done by UE AS or UE APP is to be decided by RAN2.

  - MDT-QoE alignment in RRC_IDLE and RRC_INACTIVE

In Rel-17, alignment between immediate MDT and QoE measurements is made possible by UE providing a session start or session stop indication to NG-RAN. To achieve alignment, NG-RAN can configure the immediate MDT only upon receiving session start indication and can deactivate immediate MDT upon the reception of session stop indication.

In Rel-18, we can further discuss how to achieve alignment between logged MDT and QoE configuration of MBS broadcast service.

Proposal 14: RAN3 agrees to study how to achieve alignment between logged MDT and QoE configuration of MBS broadcast service in Rel-18

One solution would be to keep logged MDT suspended at the UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer

Proposal 15: Logged MDT is kept suspended at UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer

# Conclusion

Proposal 1:  RAN3 should discuss mechanisms to support QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS broadcast service, assuming that SA4 will define QoE metrics for MBS in Rel-18

Proposal 2: A common QoE configuration should be defined for all RRC states for MBS i.e., there is no need to define a separate QoE configuration for RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE for MBS broadcast service.

Proposal 3: It is up to RAN2 to decide how to signal this common QoE configuration for MBS to the UE e.g., via MCCH and/or dedicated RRC signaling. LS RAN2 seeking signaling support for this common QoE configuration for MBS.

Proposal 4: UE should not release the QoE configuration for MBS broadcast service upon entering RRC_IDLE state. RAN2 should provide corresponding support in the specifications

Observation 1: Upon releasing the UE to RRC_IDLE, gNB releases all UE context and hence has no more knowledge of the QoE configuration

Proposal 5: RAN3 should discuss solutions on how the new gNB to which UE reconnects after transitioning from RRC_IDLE can forward the QoE reports to MCE in case QoE reports associated to the old gNB's MBS broadcast QoE configuration are received in the new gNB

Proposal 6: An identifier for MCE which can be uniquely mapped to an MCE IP address should be defined and OAM should configure this mapping to each NG-RAN node. LS SA5 to check if such an identifier and mapping is feasible.

Proposal 7: NG-RAN should include at least the MCE ID in the common QoE configuration for MBS broadcast service sent to the UE

Proposal 8: UE should indicate the MCE ID in the QoE report associated to the MBS broadcast QoE configuration sent after transitioning from RRC_IDLE to RRC_CONNECTED

Proposal 9: Reuse Push-based reporting via SRB4 for QoE measurements collected in RRC_IDLE and RRC_INACTIVE i.e., UE should send QoE report upon reception from APP layer and if SRB4 is configured and should not trigger a connection establishment just to report QoE

Proposal 10: QoE measurements collected in RRC_IDLE/RRC_INACTIVE should be stored at the UE and should be reported once the UE is back to RRC_CONNECTED

Proposal 11: It is up to RAN2 to decide whether to store the QoE measurements collected in RRC_IDLE/RRC_INACTIVE in AS layer or APP layer.

Proposal 12: UE can indicate the availability of MBS broadcast QoE measurements to the reconnected NG-RAN node so that it can configure SRB4 for retrieving the stored MBS broadcast QoE reports and avoid overwriting with a new MBS broadcast QoE configuration

Proposal 13: An area scope for QMC in RRC_IDLE/RRC_INACTIVE should be provided to the UE and UE should collect QoE measurement in RRC_IDLE/RRC_INACTIVE only if it is within the configured area scope. Whether this area scope check should be done by UE AS or UE APP is to be decided by RAN2.

Proposal 14: RAN3 agrees to study how to achieve alignment between logged MDT and QoE configuration of MBS broadcast service in Rel-18

Proposal 15: Logged MDT is kept suspended at UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer

# 4. References