3GPP TSG-RAN WG3 Meeting #117-e                                                      	R3-224612

E-Meeting: Aug 15th – Aug 24th, 2022

Agenda item:	11.3

Source:	Qualcomm Inc.

Title:	Support for QoE in NR-DC

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss how to support QoE measurement configuration, collection and reporting in NR-DC scenarios for the following objective in Rel-18 QoE WID

Specify to support for QoE in NR-DC, e.g., enable QoE reporting via SN [RAN3, RAN2].

- Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

- Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

# Discussion

Rel-17 already supports signaling and management based QoE for NR standalone deployments. In Rel-18, we should look to support both signaling and management based QoE in NR-DC.

Proposal 1: Rel-18 should support both signaling and management based QoE in NR-DC.

  - Signaling based QoE in NR-DC

QoE measurements collected at the UE are sent to MCE for optimizing application performance and there are no obvious benefits to configure a separate QoE configuration from both MN and SN i.e., it is sufficient to send a single QoE measurement configuration towards MN. Therefore, we propose that AMF provides a QoE configuration towards MN for signalling based QoE activation in MR-DC and no QoE configuration for SN is provided.

Proposal 2: For signalling based QoE activation in NR-DC, AMF provides the signaling based QoE configuration to MN and no QoE configuration for SN is provided.

  - Management based QoE in MR-DC

For management-based QoE measurement activation, the OAM sends one or more QoE measurement configurations directly to the gNB. The gNB selects UE(s) that meet the required QoE measurement capability, Area Scope and Slice Scope.

Similar to management-based trace activation, OAM should be able provide the management based QoE configuration to both MN and SN independently in case of NR-DC, but we propose that only MN should be able to configure the management based QoE to the UE. In case of Management Based QoE Activation towards a SN, the SN forwards the management based QoE configuration to the MN which performs area scope check and UE capability check and decides whether to configure the UE with this QoE configuration

Proposal 3: In case of management-based QoE activation in NR-DC, OAM can provide management based QoE configuration to MN and SN independently, but only MN should be able to configure the UE with management based QoE

Proposal 4: In case a management based QoE is received at a SN, the SN forwards the management based QoE configuration to the MN which performs area scope check and UE capability check and decides whether to configure the UE with this QoE configuration

  - Handling QMC session by SN

It is not clear to us whether there are scenarios that necessitate the need for SN handling a QMC session e.g., to collect QoE reports directly from UE or forward the received QoE reports to MN/MCE. If the intention is to provide QoE reports to SN during MN overload, we are not sure if this is really necessary as QoE reporting can be paused by MN before overload and can be resumed once the overload is complete as defined in Rel-17. Also, reporting QoE reports directly to SN would require a SCG only bearer and also potentially switching between QoE reporting via MCG only bearer and SCG only bearer. The conditions to switch should be defined appropriately.

Observation 1: SN handling QoE reports from UE during MN overload is not very critical as QoE reporting can be paused by MN before overload and can be resumed once the overload is complete as defined in Rel-17.

Proposal 5: RAN3 should discuss if there are scenarios under which it is beneficial for SN to handle a QMC session e.g., to collect QoE reports directly from UE or forward the received QoE reports to MN/MCE.

Proposal 6: If scenarios are identified in Proposal 5, MN can request SN to handle a QMC session for the UE by propagating the complete or portion (e.g., QoE Reference, MCE IP address) of the QoE configuration to SN so that the SN can identify the QoE configuration upon receiving QoE reports and will know how to forward the QoE reports

Proposal 7: If scenarios are identified in Proposal 5, RAN3 should discuss the following options for SN handling of QoE reports

- Option 1: SN can forward the QoE reports to MCE directly

- Option 2: SN forwards the QoE reports to MN and MN then sends it to MCE

Proposal 8: If scenarios are identified in Proposal 5, a UE should report QoE measurements directly to SN upon certain conditions being met. LS RAN2 to check the feasibility

  - RAN visible QoE in NR-DC

Rel-17 supported RVQoE in NR standalone deployments. In Rel-18, it needs to be discussed how to configure, collect and report RVQoE measurements in NR-DC.

RVQoE configuration is a node-specific configuration that is generated to collect certain RVQoE measurements from UE at a certain periodicity and can be utilized by the gNB for network optimization. In case of NR-DC, it is possible that MN and SN generates their own RVQoE configuration, but this needs to be coordinated to avoid duplicate RVQoE configuration at the UE. Also if MN and SN uses their own measConfigAppLayerID without coordination, this could lead to overwriting of each other’s RVQoE configuration.

Observation 2: If MN and SN generate their own RVQoE configuration without any coordination, this might lead to duplicate RVQoE configurations at the UE and overwriting of each other’s RVQoE configurations (e.g., if same measConfigAppLayerID is used by MN and SN)

MN forwards the list of available RVQoE metrics received from OAM or AMF to SN over XnAP during SN addition. One option is that SN provides the list of interested RVQoE metrics and reporting periodicity back to MN. MN then configures a common RVQoE configuration for MN and SN to the UE. Another option is that SN generates a SN-specific RVQoE configuration and provides it to the UE indirectly via MN and SRB1 or directly via SRB3.

Proposal 9: MN forwards the list of available RVQoE metrics received from AMF to SN over XnAP during SN addition

Proposal 10: RAN3 should discuss how to configure RVQoE in NR-DC and down select among the following options:

- Option 1: Common RVQoE configuration for MN and SN (after SN provides a list of interested RVQoE metrics to MN)

- Option 2: Independent RVQoE configuration for MN and SN

Regardng the reporting of RVQoE measurements in NR-DC, there could be again two options. One option is that UE reports RVQoE measurements to only MN and optionally indicate the interface information (e.g., bearer ID, PDU session ID, QoS flow ID) in the RVQoE report. MN can further determine whether the received RVQoE report is for MN or SN, and can forward it to SN if needed. Other option is that UE can send RVQoE report to MN and SN independently, either using a dedicated SCG bearer for SN or via a container to MN which forward it to SN.

Proposal 11: RAN3 should discuss how UE should report RVQoE in NR-DC:

- Option 1: UE reports RVQoE to only MN. MN can forward the RVQoE reports to SN if needed

- Option 2: UE can report RVQoE to MN and SN independently

# Conclusion

Proposal 1: Rel-18 should support both signaling and management based QoE in NR-DC.

Proposal 2: For signalling based QoE activation in NR-DC, AMF provides the signaling based QoE configuration to MN and no QoE configuration for SN is provided.

Proposal 3: In case of management-based QoE activation in NR-DC, OAM can provide management based QoE configuration to MN and SN independently, but only MN should be able to configure the UE with management based QoE

Proposal 4: In case a management based QoE is received at a SN, the SN forwards the management based QoE configuration to the MN which performs area scope check and UE capability check and decides whether to configure the UE with this QoE configuration

Observation 1: SN handling QoE reports from UE during MN overload is not very critical as QoE reporting can be paused by MN before overload and can be resumed once the overload is complete as defined in Rel-17.

Proposal 5: RAN3 should discuss if there are scenarios under which it is beneficial for SN to handle a QMC session e.g., to collect QoE reports directly from UE or forward the received QoE reports to MN/MCE.

Proposal 6: If scenarios are identified in Proposal 5, MN can request SN to handle a QMC session for the UE by propagating the complete or portion (e.g., QoE Reference, MCE IP address) of the QoE configuration to SN so that the SN can identify the QoE configuration upon receiving QoE reports and will know how to forward the QoE reports

Proposal 7: If scenarios are identified in Proposal 5, RAN3 should discuss the following options for SN handling of QoE reports

- Option 1: SN can forward the QoE reports to MCE directly

- Option 2: SN forwards the QoE reports to MN and MN then sends it to MCE

Proposal 8: If scenarios are identified in Proposal 5, a UE should report QoE measurements directly to SN upon certain conditions being met. LS RAN2 to check the feasibility

Observation 2: If MN and SN generate their own RVQoE configuration without any coordination, this might lead to duplicate RVQoE configurations at the UE and overwriting of each other’s RVQoE configurations (e.g., if same measConfigAppLayerID is used by MN and SN)

Proposal 9: MN forwards the list of available RVQoE metrics received from AMF to SN over XnAP during SN addition

Proposal 10: RAN3 should discuss how to configure RVQoE in NR-DC and down select among the following options:

- Option 1: Common RVQoE configuration for MN and SN (after SN provides a list of interested RVQoE metrics to MN)

- Option 2: Independent RVQoE configuration for MN and SN

Proposal 11: RAN3 should discuss how UE should report RVQoE in NR-DC:

- Option 1: UE reports RVQoE to only MN. MN can forward the RVQoE reports to SN if needed

- Option 2: UE can report RVQoE to MN and SN independently

# 4. References