3GPP TSG-RAN WG3 #117-e	R3-224643

E-meeting, 15th – 24th August 2022

Source:	CATT

Title:	Discussion on Discontinuous Coverage

Agenda Item:	18. 2

Document for:	Discussion and decision

Introduction

SA2 provides 16 candidate solutions for mobility management and/or power saving in discontinuous coverage. The solutions can be divided into CN determined, UE determined and jointly determined. In this contribution, we simply analyse the CN determined and UE determined case provided in SA2 from the perspective of RAN3.

Discussion

In CN determined case, most solutions indicate that RAN needs to provide satellite assistance information to MME (solutions #1, #4, #5, #9, #10 and #11). The satellite assistance information can be performed as different form, but the purpose is similar for knowing the duration of none coverage and determines the parameters for power saving and/or perform paging optimisation. The form of satellite assistance information may be satellite ephemeris (solution #4 and #5). For MME, only transfer satellite ephemeris seems not enough to inference the satellite coverage, which information should be included in satellite assistance information and transfer to MME can be further study.

Proposal 1: RAN need to study which assistant information should be provide to MME in CN determined case.

In UE determined case, those solutions rely upon the UE knowing its mobility and being able to provide accurate information to the CN (Solutions #8 and #16). Similar, RAN2 has defined a new SIB32 to broadcasts satellite information to UE in REL-17.The maximum number of ephemeris broadcast is set to 4. Due to the incomplete satellite ephemeris, the UE still hard to know the accurate unavailability period, especially for the UE with strong mobility. In addition, UE determined case may base on an assumption that UE reported location is always believable, which is not always established in REL-18.

Observation 1: In Rel-17, RAN2 has defined a new SIB32 to broadcasts satellite information to UE.

Proposal 2: UE determined case should be further checked combination with UE location verification.

Conclusion

Proposal 1: RAN need to study which assistant information should be provide to MME in CN determined case.

Observation 1: In Rel-17, RAN2 has defined a new SIB32 to broadcasts satellite information to UE.

Proposal 2: UE determined case may be further check combination with UE location verification.

Reference

[1] 3GPP TR 23.700-28  Study on Integration of satellite components in the 5G architecture  Phase 2(Release 18)

[2] RP-211806  Revised WID on IoT NTN enhancements  MediaTek Inc

[3] R2-2206693  Addition of missing functionalities and corrections to support of Non-Terrestrial Network in NB-IoT and eMTC  Huawei, HiSilicon