3GPP RAN WG3 Meeting #117-e	R3-224646

# Online, 15th – 25th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0870 | rev | 0 | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of abnormal handling for Multicast session deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently,AMF does not always keep the activation result of multicast MBS session and AMF may even does not know the activation result in the NG-RAN node,e.g.for the scenario that the shared tunnel is established after the MBS session is activated. Thereby,AMF may send Multicast Session Deactivation message to NG-RAN node even the multicast MBS session is not succeessfully activated in this NG-RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add abnormal description on when NG-RAN node receives Multicast Session Deactivation message corresponding to a MBS session which is not active in NG-RAN node. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has impact on multicast session deactivation procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour of NG-RAN node is unclear when NG-RAN node receives a Multicast Session Deactivation message corresponding to a multicast session which is not active. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////Start of change///////////////////////////////////////////////////////////////////////

8.18.4	Multicast Session Deactivation

8.18.4.1	General

The purpose of the Multicast Session Deactivation procedure is to request a NG-RAN node to deactivate the MBS resources of one MBS session. The procedure uses non-UE-associated signalling.

8.18.4.2	Successful Operation

Figure 8.18.4.2-1: Multicast Session Deactivation, successful operation.

The AMF initiates the procedure by sending a MULTICAST SESSION DEACTIVATION REQUEST message to the NG-RAN node.

Upon receipt of this message, the NG-RAN node shall deactivate the MBS resources corresponding to the MBS session indicated in the MULTICAST SESSION DEACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message for which MBS session the request was fulfilled.

8.18.4.3	Unsuccessful Operation

Not applicable.

8.18.4.4	Abnormal Conditions

Upon receipt of this message, if there is no active MBS resources corresponding to the MBS session indicated in the MULTICAST SESSION DEACTIVATION REQUEST message, the NG-RAN node shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message for which MBS session the MBS resources were already deactivated.

///////////////////////////////////////////////////////////////////////End of change///////////////////////////////////////////////////////////////////////