3GPP TSG-RAN WG3 #117-e	R3-224657

E-meeting, 15th – 24th August 2022

Source:	CATT

Title:	TP on TS 38.423 for AI/ML

Agenda Item:	12.2.2

Document for:	Discussion and decision

- Introduction

This is a TP based on the discussion paper R3-224655.

- TP for TS 38.423

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Node Data Collection Initiation | NODE DATA COLLECTION REQUEST | NODE DATA COLLECTION RESPONSE | NODE DATA COLLECTION FAILURE |
| UE Data Collection Initiation | UE DATA COLLECTION REQUEST | UE DATA COLLECTION RESPONSE | UE DATA COLLECTION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Node Data Collection Reporting | NODE DATA COLLECTION REPORT |
| UE Data Collection Reporting | UE DATA COLLECTION REPORT |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 8.2.b1	UE Data Collection Initiation

#### 8.2.b1.1	General

This procedure is used by an NG-RAN node to request the collection of UE-associated data to another NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.b1.2	Successful Operation

Figure 8.2.b1.2-1: UE Data Collection Initiation, successful operation

[FFS]

#### 8.2.b1.3	Unsuccessful Operation

Figure 8.2.b1.3-1: UE Data Collection Initiation, unsuccessful operation

[FFS]

#### 8.2.b1.4	Abnormal Conditions

[FFS]

### 8.2.b2	UE Data Collection Reporting

#### 8.2.b2.1	General

This procedure is initiated by an NG-RAN node to report the result of UE-associated data collection admitted by the NG-RAN node following a successful UE Data Collection Initiation procedure.

The procedure uses UE-associated signalling.

#### 8.2.b2.2	Successful Operation

Figure 8.2.b2.2-1: UE Data Collection Reporting, successful operation

[FFS]

#### 8.2.b2.3	Unsuccessful Operation

Not applicable.

#### 8.2.b2.4	Abnormal Conditions

Void

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 8.4.a1	Node Data Collection Initiation

#### 8.4.a1.1	General

This procedure is used by an NG-RAN node to request the collection of non-UE-associated data to another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.a1.2	Successful Operation

Figure 8.4.a1.2-1: Node Data Collection Initiation, successful operation

[FFS]

#### 8.4.a1.3	Unsuccessful Operation

Figure 8.4.a1.3-1: Node Data Collection Initiation, unsuccessful operation

[FFS]

#### 8.4.a1.4	Abnormal Conditions

[FFS]

### 8.4.a2	Node Data Collection Reporting

#### 8.4.a2.1	General

This procedure is initiated by an NG-RAN node to report the result of non-UE-associated data collection admitted by the NG-RAN node following a successful Node Data Collection Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.a2.2	Successful Operation

Figure 8.4.a2.2-1: Node Data Collection Reporting, successful operation

[FFS]

#### 8.4.a2.3	Unsuccessful Operation

Not applicable.

#### 8.4.a2.4	Abnormal Conditions

Void

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Analytics Pushing | O |  | UE Analytics 9.2.3.z6 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.1.1.x1	UE DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested data collection according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| UE Analytics Request | O |  | 9.2.3.z4 |  | YES | ignore |

#### 9.1.1.x2	UE DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that at least one of the requested data collection is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| UE Analytics Response | O |  | 9.2.3.z5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.1.x3	UE DATA COLLECTION FAILURE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that for none of the requested data collection can be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.1.x4	UE DATA COLLECTION REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested data collection.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| UE Analytics | O |  | 9.2.3.z6 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node or the source SN. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Analytics Pushing | O |  | UE Analytics 9.2.3.z6 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.1.3	Messages for Global Procedures

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.1.3.y1	NODE DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested data collection according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 Data Collection ID [FFS whether needed] | O |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Node Analytics Request | O |  | 9.2.3.z1 |  | YES | ignore |

#### 9.1.3.y2	NODE DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that at least one of the requested data collection is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Analytics Response | O |  | 9.2.3.z2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.y3	NODE DATA COLLECTION FAILURE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that for none of the requested data collection can be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.y4	NODE DATA COLLECTION REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested data collection.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN Node1 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN Node2 Data Collection ID [FFS whether needed] | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Analytics | O |  | 9.2.3.z3 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.1	Container and List IE definitions

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.2.1.w1	Node RRM Status Request

This IE is used to request collecting statistics of RRM events.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of RRC Reestablishment Request [FFS] | O |  | FFS |  |

#### 9.2.1.w2	Node RRM Status Response

This IE is used to indicate the acceptation or rejection of collecting statistics of RRM events.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of RRC Reestablishment Response [FFS] | O |  | FFS |  |

#### 9.2.1.w3	Node RRM Status

This IE contains statistics of RRM events.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of RRC Reestablishment [FFS] | O |  | FFS |  |

#### 9.2.1.w4	Node Traffic Status Request

This IE is used to request collecting statistics of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Status Request | O |  | FFS |  |

#### 9.2.1.w5	Node Traffic Status Response

This IE is used to indicate the acceptation or rejection of collecting statistics of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Status Response | O |  | FFS |  |

#### 9.2.1.w6	Node Traffic Status

This IE contains statistics of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Status | O |  | FFS |  |

#### 9.2.1.w7	Node Traffic Prediction Request

This IE is used to request prediction of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Prediction Request | O |  | FFS |  |

#### 9.2.1.w8	Node Traffic Prediction Response

This IE is used to indicate the acceptation or rejection of prediction of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Prediction Response | O |  | FFS |  |

#### 9.2.1.w9	Node Traffic Prediction

This IE contains prediction of e.g. data volume.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Volume Prediction | O |  | FFS |  |

#### 9.2.1.w10	Node Air Interface Status Request

This IE is used to request collecting statistics about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Status Request | O |  | FFS |  |
| Cell Throughput Status Request | O |  | FFS |  |
| Air Interface Delay Status Request | O |  | FFS |  |

#### 9.2.1.w11	Node Air Interface Status Response

This IE is used to indicate the acceptation or rejection of collecting statistics about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Status Response | O |  | FFS |  |
| Cell Throughput Status Response | O |  | FFS |  |
| Air Interface Delay Status Response | O |  | FFS |  |

#### 9.2.1.w12	Node Air Interface Status

This IE contains statistics about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Status | O |  | FFS |  |
| Cell Throughput Status | O |  | FFS |  |
| Air Interface Delay Status | O |  | FFS |  |

#### 9.2.1.w13	Node Air Interface Prediction Request

This IE is used to request prediction about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Prediction Request | O |  | FFS |  |
| Cell Load Prediction Request | O |  | FFS |  |

#### 9.2.1.w14	Node Air Interface Prediction Response

This IE is used to indicate the acceptation or rejection of prediction about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Prediction Response | O |  | FFS |  |
| Cell Load Prediction Response | O |  | FFS |  |

#### 9.2.1.w15	Node Air Interface Prediction

This IE contains prediction about air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Consumption Prediction | O |  | FFS |  |
| Cell Load Prediction | O |  | FFS |  |

#### 9.2.1.w16	UE RRM Prediction Request

This IE is used to request prediction about UE RRM, e.g. UE location.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Prediction Request | O |  | FFS |  |

#### 9.2.1.w17	UE RRM Prediction Response

This IE is used to indicate the acceptation or rejection of prediction about UE RRM, e.g. UE location.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Prediction Response | O |  | FFS |  |

#### 9.2.1.w18	UE RRM Prediction

This IE contains prediction about UE RRM, e.g. UE location.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Prediction | O |  | FFS |  |

#### 9.2.1.w19	UE Traffic Status Request

This IE is used to request collecting statistics of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Status Request | O |  | FFS |  |

#### 9.2.1.w20	UE Traffic Status Response

This IE is used to indicate the acceptation or rejection of collecting statistics of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Status Response | O |  | FFS |  |

#### 9.2.1.w21	UE Traffic Status

This IE contains statistics of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Status | O |  | FFS |  |

#### 9.2.1.w22	UE Traffic Prediction Request

This IE is used to request prediction of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Prediction Request | O |  | FFS |  |

#### 9.2.1.w23	UE Traffic Prediction Response

This IE is used to indicate the acceptation or rejection of prediction of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Prediction Response | O |  | FFS |  |

#### 9.2.1.w24	UE Traffic Prediction

This IE contains prediction of traffic for a UE.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Prediction | O |  | FFS |  |

#### 9.2.1.w25	UE Air Interface Status Request

This IE is used to request collecting UE-associated information acquired at the air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Sub-cell Status Request | O |  | FFS |  |

#### 9.2.1.w26	UE Air Interface Status Response

This IE is used to indicate the acceptation or rejection of collecting UE-associated information acquired at the air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Sub-cell Status Response | O |  | FFS |  |

#### 9.2.1.w27	UE Air Interface Status

This IE contains UE-associated information acquired at the air interface.

It may be delivered transparently over interfaces.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Location Sub-cell Status | O |  | FFS |  |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.3	General IE definitions

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.2.3.z1	Node Analytics Request

This IE is used to request analytics not associated to a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Node RRM Status Request | O |  | OCTET STRING | Containing 9.2.1.w1 Node RRM Status Request. |
| Node Traffic Status Request | O |  | OCTET STRING | Containing 9.2.1.w4 Node Traffic Status Request. |
| Node Traffic Prediction Request | O |  | OCTET STRING | Containing 9.2.1.w7 Node Traffic Prediction Request. |
| Node Air Interface Status Request | O |  | OCTET STRING | Containing 9.2.1.w10 Node Air Interface Status Request. |
| Node Air Interface Prediction Request | O |  | OCTET STRING | Containing 9.2.1.w13 Node Air Interface Prediction Request. |

#### 9.2.3.z2	Node Analytics Response

This IE is used to indicate the acceptation or rejection of each request on analytics not associated to a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Node RRM Status Response | O |  | OCTET STRING | Containing 9.2.1.w2 Node RRM Status Response. |
| Node Traffic Status Response | O |  | OCTET STRING | Containing 9.2.1.w5 Node Traffic Status Response. |
| Node Traffic Prediction Response | O |  | OCTET STRING | Containing 9.2.1.w8 Node Traffic Prediction Response. |
| Node Air Interface Status Response | O |  | OCTET STRING | Containing 9.2.1.w11 Node Air Interface Status Response. |
| Node Air Interface Prediction Response | O |  | OCTET STRING | Containing 9.2.1.w14 Node Air Interface Prediction Response. |

#### 9.2.3.z3	Node Analytics

This IE contains analytics not associated to a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Node RRM Status | O |  | OCTET STRING | Containing 9.2.1.w3 Node RRM Status. |
| Node Traffic Status | O |  | OCTET STRING | Containing 9.2.1.w6 Node Traffic Status. |
| Node Traffic Prediction | O |  | OCTET STRING | Containing 9.2.1.w9 Node Traffic Prediction. |
| Node Air Interface Status | O |  | OCTET STRING | Containing 9.2.1.w12 Node Air Interface Status. |
| Node Air Interface Prediction | O |  | OCTET STRING | Containing 9.2.1.w15 Node Air Interface Prediction. |

#### 9.2.3.z4	UE Analytics Request

This IE is used to request analytics for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RRM Prediction Request [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w16 UE RRM Prediction Request. |
| UE Traffic Status Request | O |  | OCTET STRING | Containing 9.2.1.w19 UE Traffic Status Request. |
| UE Traffic Prediction Request [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w22 UE Traffic Prediction Request. |
| UE Air Interface Status Request [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w25 UE Air Interface Status Request. |

#### 9.2.3.z5	UE Analytics Response

This IE is used to indicate the acceptation or rejection of each request on analytics for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RRM Prediction Response [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w17 UE RRM Prediction Response. |
| UE Traffic Status Response | O |  | OCTET STRING | Containing 9.2.1.w20 UE Traffic Status Response. |
| UE Traffic Prediction Response [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w23 UE Traffic Prediction Response. |
| UE Air Interface Status Response [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w26 UE Air Interface Status Response. |

#### 9.2.3.z6	UE Analytics

This IE contains analytics for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RRM Prediction [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w18 UE RRM Prediction. |
| UE Traffic Status | O |  | OCTET STRING | Containing 9.2.1.w21 UE Traffic Status. |
| UE Traffic Prediction [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w24 UE Traffic Prediction. |
| UE Air Interface Status [FFS whether present] | O |  | OCTET STRING | Containing 9.2.1.w27 UE Air Interface Status. |

///////////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////////