3GPP TSG-RAN WG3 Meeting #117-e	R3-224663

E-meeting, 15 – 24 August 2022

Title: 	(TP to TR 38.867) NR network-controlled repeater identification and authorization

Source: 	Huawei

Agenda item:	22.2

Document Type:	pCR

# 1. Introduction

In RAN#94e meeting, the SI on NR network-controlled repeaters was approved in [1], with the objectives given as follows.

- Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

- Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

- Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

In this paper, we give the TP for TR 38.867 for capturing our proposals about the network-controlled repeater management in [2].

# 2. TP for NCR TR 38.867

--------------------------------Start of change--------------------------------

## 8 Repeater management

#### 8.1 Solution on Repeater management

##### 8.1.1 Identification and Authorization

The gNB-DU needs to distinguish an NCR-MT and a normal UE, since the gNB-DU only transmits side control information to an NCR-MT but not a normal UE. The NCR-MT transmits a NCR indication to the gNB-CU in RRC setup procedure for identification. Specifically, an NCR indication IE is added in RRCSetupComplete message. After the NCR-MT setting up RRC connection, the gNB-CU informs the CN that the newly accessing UE is an NCR-MT via Initial UE Message procedure.

The NCR-MT reports its UE ID (e.g. SUCI, GUTI) to the AMF via NAS, and the authorization is performed by the core network. In case the NCR-MT is authorized to be served as an NCR, the AMF provides NCR authorized indication towards the gNB-CU.

Upon receiving the NCR authorized information from the AMF, the gNB-CU will further provide the NCR indication to the gNB-DU.

The NCR Identification and Authorization procedure is illustrated in Figure 8.1.1.x.

Figure 8.1.1.x NCR Identification and Authorization

#### 8.2 Specification impacts

The NCR identification and authorization causes spec impacts on the following specs:

- TS 38.331 (RRC): NCR indication IE is added in RRCSetupComplete message.

- TS 38.413 (NGAP): NCR indication IE is added in INITIAL UE MESSAGE message, and NCR Authorized IE is added in INITIAL CONTEXT SETUP REQUEST message.

- TS 38.473 (F1AP): NCR indication IE is added in UE CONTEXT SETUP REQUEST message.

--------------------------------End of change--------------------------------

# Reference

- RP-213700, SID on NR Network-controlled Repeaters, RAN#94-e.