3GPP TSG-RAN WG3 Meeting #117-e	R3-224699

E-meeting, 15 – 24 August 2022

Title: 	Consideration on intra-DU L1/L2 mobility procedure

Source: 	Huawei

Agenda item:	14.2

Document Type:	Discussion

1. Introduction

At RAN plenary meeting #96, a new WID on further NR mobility enhancement was approved. One of the objectives is to specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction in both inter-DU and intra-DU case which requires RAN3 works when considering the gNB CU-DU split architecture.

In this contribution, we are going to discuss the F1 impacts and procedures with L1/L2 based mobility in case of intra-DU HO.

2. Discussion

In legacy, we have defined the overall F1 procedure for CHO and normal L3 handover procedure under both inter-DU and intra-DU case which has some common points. We thus think those the defined F1 call flows can be used as a baseline when designing the L1/L2 based mobility for both inter-DU and intra DU case. Below we provide L1/L2 based mobility call flows for intra DU case with taking into account the legacy HO procedure. As usual, we separate the HO procedure into three phase: HO preparation phase, HO execution phase and HO complete phase. In each phase we discuss if there is any RAN3 impact.

Figure 1: intra DU L1/L2 based HO procedure

HO preparation phase

- The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received MeasurementReport message.

- The gNB-CU determines whether to configure L1/L2 mobility based on the MeasurementReport message.

- The gNB-CU sends an UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU for each candidate cell with L1/L2 mobility initiation indicator and the candidate cell ID for L1/L2 mobility.

- If the preparation request is accepted, the source gNB-DU responds with an UE CONTEXT MODIFICATION RESPONSE message including the generated RRC configuration for that candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message which may contain the RRC configurations for multiple candidate cells for L1/L2 mobility.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

HO execution phase

- FFS: The UE sends L1 measurement result to the source gNB-DU. To be discussed and decided by RAN1 and RAN2

- FFS: The source gNB-DU sends L1/L2 based HO command to the UE. To be discussed and decided by RAN1 and RAN2

HO complete phase

- The gNB DU detects the UE connected to the target cell by L1/L2 signalling then sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for releasing the resources of prepared candidate cells. The gNB-DU releases the candidate cell related UE context and responds the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

At following sections, we discuss each HO phase (as mentioned above) with RAN3 impacts in detail.

## 2.1 HO preparation

Normally gNB-CU has ANR functionality with the full knowledge on the neighbor cells and can initially select which neighbor cells can be the potential candidate cells for normal L3 handover, e.g. based on the existing L3 measurement result from UE and then ask the gNB-DU to confirm if the selected candidate cell is accepted via UE CONTEXT MODIFICATION REQUEST message with F1-U tunnel establishment information. Then the gNB-DUs may provide the configuration (i.e. RRC information) of candidate cell and F1-U tunnels of the UE to gNB-CU via UE CONTEXT MODIFICATION RESPONSE if accepted. We do not think we need to break the above principle for L1/L2 based mobility and thus propose.

For intra-DU L1/L2 handover, gNB CU decides to initiate the handover preparation of candidate cell to the gNB-DU based on L3 measurement result.

For intra-DU L1/L2 handover, the existing UE CONTEXT MODIFICATION procedure can be reused as baseline for handover preparation and update of candidate cell(s).

## 2.2 HO execution

Based on the WID, RAN1/RAN2 will work on the L1/L2 mobility based measurement report (see below).

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

For now we think that RAN3 can assume the gNB-DU is able to get L1/L2 measurement reports via L1/L2 signalling and then initiate the L1/L2 handover command and switch the serving cell dynamically based on the measurement result from UE. Under the above assumption RAN3 may discus and decide which node to initiate the dynamic switch of serving cell, e.g. HO execution. If gNB-CU makes the decision, there are some coordination signalling between gNB-DU and gNB-CU which brings more extra latencies. From handover latency point of view, it would be good to let the gNB-DU itself make the final decision. The content of HO command is up to RAN1 or RAN2.

For intra-DU L1/L2 handover, the gNB DU initiates the HO command to the UE e.g. based on L1 measurement result. The details of HO command is up to RAN1/RAN2.

## 2.3 HO complete

When gNB-DU decides to perform HO execution, the issue on whether it can immediately indicate to gNB-CU the Downlink Data Delivery Status (DDDS) for fast retransmitting the unsuccessfully transmitted packets should be discussed. In intra-DU case, it depends on whether the RLC is re-established or not. If RLC is re-established during L1/L2 handover meaning all buffered RLC data are discarded, the retransmission has to rely on the PDCP layer at the gNB-CU by data recovery procedure. In such case the DDDS from the gNB-DU to gNB-CU is needed. If RLC is not re-established during intra-DU L1/L2 handover, the DDS may be skipped.  We thus think that sending a LS to RAN2 for checking would be necessary.

To send a LS to RAN2 to confirm if RLC is re-established or not during intra-DU L1/L2 handover.

When the UE is switched to the target cell, the gNB-DU sends the ACCESS SUCCESS message to inform the gNB-CU the newest UE’s serving cell.

For intra-DU L1/L2 handover, the gNB DU sends the ACCESS SUCCESS message to the gNB CU after the UE accesses to the target cell successfully.

## 2.4 releasing of L1/L2 mobility

The gNB-CU should also be able to release the resources of the prepared candidate cell as mentioned in figure 1 if it determines that the prepared cell is not suitable for future L1/L2 handover anymore. For intra-DU case, we think that the existing UE CONTEXT MODIFICATION procedure (CU-initiated) can be reused.

For intra-DU L1/L2 handover, the gNB CU may initiate UE CONTEXT MODIFICATION procedure for releasing the prepared resources in the gNB DU.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- For intra-DU L1/L2 handover, gNB CU decides to initiate the handover preparation of candidate cell to the gNB-DU based on L3 measurement result.

For intra-DU L1/L2 handover, the existing UE CONTEXT MODIFICATION procedure can be reused as baseline for handover preparation and update of candidate cell(s).

For intra-DU L1/L2 handover, the gNB DU initiates the HO command to the UE e.g. based on L1 measurement result. The details of HO command is up to RAN1/RAN2.

To send a LS to RAN2 to confirm if RLC is re-established or not during intra-DU L1/L2 handover.

For intra-DU L1/L2 handover, the gNB DU sends the ACCESS SUCCESS message to the gNB CU after the UE accesses to the target cell successfully.

For intra-DU L1/L2 handover, the gNB CU may initiate UE CONTEXT MODIFICATION procedure for releasing the prepared resources in the gNB DU.

The tentative stage 2 CR to TS 38.401 is provided in [1] for reference.

4. Reference

[1] R3-224934 L1/L2 handover procedure