3GPP TSG-RAN WG3 Meeting #117-e	R3-224700

E-meeting, 15 – 24 August 2022

Title: 	Consideration on intra-CU inter-DU L1/L2 mobility procedure

Source: 	Huawei

Agenda item:	14.2

Document Type:	Discussion

1. Introduction

At RAN plenary meeting #96, a new WID on further NR mobility enhancement was approved. One of the objectives is to specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction in both intra-CU inter-DU and intra-DU case which requires RAN3 works when considering the gNB CU-DU split architecture.

In this contribution, we are going to discuss the F1 impacts and procedures with L1/L2 based mobility in case of inter-DU L1/L2Handover.

2. Discussion

In legacy, we have defined the overall F1 procedure for CHO and normal L3 handover procedure under inter-DU case which has some common points. We thus think those F1 call flows can be used as a baseline when designing the L1/L2 based mobility for inter-DU case. Below we provide L1/L2 based mobility call flows for inter-DU case with taking into account the legacy HO. As usual, we separate the HO procedure into three phases: HO preparation phase, HO execution phase and HO complete phase. In each phase we discuss if there is any RAN3 impact.

Figure 1: inter DU L1/L2 based HO

HO preparation phase

- The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received MeasurementReport message.

- The gNB-CU determines whether to configure L1/L2 mobility based on the MeasurementReport message.

- The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the target gNB-DU to create the UE context and prepare radio resources. .

- If the preparation request is accepted, the candidate gNB-DU responds to the gNB-CU with an UE CONTEXT SETUP RESPONSE message including the generated RRC configuration for the target cell ID that was requested from the gNB-CU.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

HO execution phase

- FFS: The UE sends L1 measurement result to the source gNB-DU. To be discussed and decided by RAN1 and RAN2

- FFS: The source gNB-DU sends L1/L2 based HO command to the UE. To be discussed and decided by RAN1 and RAN2. The source gNB-DU sends the Downlink Data Delivery Status to the gNB-CU for DL data retransmissionon.

- A Random Access procedure is performed at the target gNB-DU.

HO complete phase

- The target gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.\

- The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the UE context in the source gNB-DU.

- The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message if the UE CONTEXT RELEASE COMMAND message is received.

At following sections, we discuss each HO phase (as mentioned above) with RAN3 impacts in detail.

## 2.1 HO preparation

Normally, gNB-CU has ANR functionality with the full knowledge on the neighbor cells and can initially select which neighbor cells can be the potential candidate cells for normal L3 handover, e.g. based on the existing L3 measurement result from UE and then ask candidate gNB-DU(s) to confirm if the selected candidate cells are accepted via UE CONTEXT SETUP REQUEST (inter DU case) message with F1-U tunnel establishment information. Then the candidate gNB-DUs may provide the configuration (i.e. RRC information) of candidate cell and F1-U tunnels of the UE to gNB-CU via UE CONTEXT SETUP RESPONSE (inter DU case) if accepted. We do not think we need to break the above principle for L1/L2 based mobility and thus propose.

For intra-CU inter-DU L1/L2 handover, the gNB CU decides to initiate the handover preparation of candidate cell to the target DU, e.g. based on L3 measurement result.

For intra-CU inter-DU L1/L2 handover, the existing UE CONTEXT SETUP procedure can be reused as baseline for handover preparation.

As mentioned above, there is a configuration of candidate cell provided from target gNB-DUs to gNB-CU. We think that some of configurations (e.g. L1/L2 measurement configuration of candidate cell) shall be applied by the UE immediately at the reception of the RRCReconfiguration message for measuring the candidate cells. While the other part (e.g. RLC and MAC configuration) may not require to be applied immediately. It is out of RAN3 scope to decide the contents of DU to CU RRC configuration and when UE applies those RRC configurations. But it is expected to get some guidance from RAN2 so as to decide if new F1 parameters needs to be added inside the parameter of DU to CU RRC information sent from candidate gNB-DUs.

For now we have a parameter of CellGroupConfig referred in DU to CU RRC information which can include almost all RRC configuration of gNB-DU. If RAN2 agree that the CellGroupConfig does not take effect until HO is executed by gNB-DU. The L1/L2 measurement pre-configuration of candidate cell should be placed outside CellGroupConfig for enabling the measurement before HO execution. To figure out this issue we need to inform RAN2 for asking them to confirm when the L1/2 mobility related configuration takes effect.

To send a LS to RAN2 for confirmation on the time that the CellGroupConfig from target DU is applied by the UE

## 2.2 HO execution

Based on the WID, RAN1/RAN2 will work on the L1/L2 mobility based measurement report (see below).

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

For now we think that RAN3 can assume the source gNB-DU is able to get a L1/L2 measurement reports via L1/L2 signalling and then initiate the L1/L2 handover command and serving cell dynamic switch based on the measurement result from UE. Under the above assumption RAN3 may discuss and decide which node to initiate the dynamic serving cell switch, e.g. HO execution. If gNB-CU makes the decision, there are some coordination signalling between source gNB-DU and gNB-CU which brings more extra latencies. From handover latency point of view, it would be good to let the source gNB-DU itself make the final decision. The content of HO command can be up to RAN1 or RAN2.

For intra-CU inter-DU L1/L2 handover, the source gNB DU initiates the HO command to the UE e.g. based on L1 measurement result. The content of HO command is up to RAN1/RAN2.

## 2.3 HO complete

In inter-DU case when source gNB-DU decides to perform HO execution, it can immediately indicate to candidate gNB-CU the Downlink Data Delivery Status (DDDS) for fast retransmitting the unsuccessfully transmitted packets once gNB-CU has been indicated via ACCESS SUCCESS message about UE has successfully accessed to the candidate gNB-DU.

For inter-DU L1/L2 handover, the source gNB DU sends the Downlink data delivery status (DDDS) to the gNB CU after sending handover command to the UE.

The ACCESS SUCCESS message is used to determine when the gNB-CU switches the data path from the source gNB-DU to the target gNB-DU.

For intra CU inter-DU L1/L2 handover, the target gNB DU sends the ACCESS SUCCESS message to the gNB CU after the UE accesses to target cell successfully.

For intra-CU inter-DU L1/L2 handover, the gNB CU switches the data path to target gNB DU after receiving ACCESS SUCCESS.

For time sensitive service it would be beneficial on reducing the interruption if gNB-CU can early forward (e.g. before the HO execution phase) some data to all candidate gNB-DUs for back-up for which once the candidate gNB-DU is turned to target gNB-DU it can immediately send the those early forwarded DL data to the UE once the UE accesses the target cell.

RAN3 to study the support of early data forwarding to the candidate gNB DU in intra-CU inter-DU L1/L2 handover.

## 2.4 releasing of L1/L2 mobility

The gNB-CU should also be able to release the resources in source gNB-DU and in the prepared candidate cells if it determines that the prepared cell is not suitable for future L1/L2 handover anymore as mentioned in figure 1. We think that the existing UE CONTEXT RELEASE and UE CONTEXT MODIFICATION can be reused.

For intra-CU inter-DU handover, the gNB-CU may initiate UE CONTEXT RELEASE and UE CONTEXT MODIFICATION procedure for releasing the UE context in source gNB-DU and the prepared resources in candidate gNB DUs.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- For intra-CU inter-DU L1/L2 handover, the gNB CU decides to initiate the handover preparation of candidate cell to the target DU, e.g. based on L3 measurement result.

For intra-CU inter-DU L1/L2 handover, the existing UE CONTEXT SETUP procedure can be reused as baseline for handover preparation.

To send a LS to RAN2 for confirmation on the time that the CellGroupConfig from target DU is applied by the UE

For intra-CU inter-DU L1/L2 handover, the source gNB DU initiates the HO command to the UE e.g. based on L1 measurement result. The content of HO command is up to RAN1/RAN2.

For inter-DU L1/L2 handover, the source gNB DU sends the Downlink data delivery status (DDDS) to the gNB CU after sending handover command to the UE.

For intra CU inter-DU L1/L2 handover, the target gNB DU sends the ACCESS SUCCESS message to the gNB CU after the UE accesses to target cell successfully.

For intra-CU inter-DU L1/L2 handover, the gNB CU switches the data path to target gNB DU after receiving ACCESS SUCCESS.

RAN3 to study the support of early data forwarding to the candidate gNB DU in intra-CU inter-DU L1/L2 handover.

For intra-CU inter-DU handover, the gNB-CU may initiate UE CONTEXT RELEASE and UE CONTEXT MODIFICATION procedure for releasing the UE context in source gNB-DU and the prepared resources in candidate gNB DUs.

The tentative stage 2 CR to TS 38.401 is provided in [1] for reference.

4. Reference

[1] R3-224934 L1/L2 handover procedure