3GPP TSG-RAN WG3 Meeting #117-e	R3-224725

E-meeting, 15 - 24 Aug 2022

Title: 	Supporting network slice AS group

Source: 	Huawei, CMCC, ZTE

Agenda item:	9.2.8

Document Type:	Discussion

# 1. Introduction

Previous RAN3 #116-e meeting discussed the NSAG transfer over Xn interface, but there was no consensus as captured in the Chairman notes as follows (also see the SoD in [1]) .

RAN node just reports its own slice group information to AMF, and the slice group info of neighboring cells should not be transferred to AMF.

Support of NSAG in Xn signaling or by OAM configuration.

To be continued in corrections in R17...

This contribution aims to continue and conclude this remaining open issue.

# 2. Discussion

In TS 23.501, the AMF can provide the NSAG per TA to the UE.

- The AMF shall indicate in the NSAG Information in which TA a specific NSAG association to S-NSSAI(s) is valid if the AMF provides in the UE configuration a NSAG value which is used in different TAs with a different association with NSSAIs. The configuration the AMF provides includes at least the NSAGs for the UE for the TAs of the Registration Area.

It can be observed that the UE may be configured with the NSAG information, not only in its currently camped TA, but also in other TAs at least in the UE’s registration area. Also last RAN2 meeting agreed to include an optional TAC associated with supporting NSAG (s) in SIB16 considering the case that the same NASG ID with different NSAG configuration (i.e. the mapping of S-NSSAI (s)) may be used in different TAs.

Observation 1: The NG-RAN node is aware of the NSAG information of its neighbour nodes, and the TAC is optionally broadcasted in SIB16.

We think that the NSAG information should be signalled over Xn, due to the following reasons (some of them already covered in [1]).

- Follow the same principle for the slice information exchange between nodes. Starting from R15, the TAI Slice Support List is introduced to facilitate the connected mode mobility between neighbour NG-RAN nodes. Then for the NSAG information, it can be used for idle mode mobility (i.e. the cell reselection) as specified in TS 38.304.

- The self-configuration/self-optimization mechanism of the NSAG information could be enabled. Whenever the NSAG information is updated in a neighbour node, the serving RAN node can get automatically informed. Otherwise, each time a new S-NSSAI is enabled or an old one is released for a single NG-RAN node, the OAM may have to reconfigure the change towards many neighbour NG-RAN nodes, which is not only time-consuming, but also increases much burden for the OAM work.

- In SIB16 provided in the Annex, there is an allowed/excluded list in a cell granularity to indicate either the allow-list or the excluded-list cells for NSAG. It is much cumbersome and complicated work for the OAM to configure this cell list belonging to the neighbour NG-RAN nodes for the NG-RAN node. And considering the possible change of the configuration, it is even not possible work relying on the OAM.

In [1], there are some questions how the nsag-CellReselectionPriority/nsag-CellReselectionSubPriority are decided with the NSAG information over Xn interface as captured as follows.

If a RAN node receives NSAGs from its neighbour RAN nodes and uses them to broadcast them, how would that RAN node broadcast the required CellReselectionPriority for the neighbour´s Slice Group ID?

We understand this NSAG Cell Reselection Priority should be decided by the OAM, which has the same handling for the non-NSAG related cell reselection priority as specified as follows.

- Starting from R15, the NR-RAN node can decide the cell reselection priority for serving frequency in SIB2, and the one for inter-frequency in SIB4. When the NG-RAN node is aware of the neighbour frequencies by the Xn setup/configuration message, the OAM can decide the corresponding the frequency priority accordingly. And in this case, the OAM can coordinate the NG-RAN nodes to have aligned cell reselection priority to avoid any ping-pong.

- Then in terms of the cell reselection priority for NSAG, the same handling can be followed. The OAM can decide the frequency priority for each NSAG (if needed) after the NG-RAN node is aware of supported NSAGs from its neighbour NG-RAN nodes, and can avoid any mis-alignment. Then there is not any need for neighbour NG-RAN nodes to exchange NSAG cell reselection priority over Xn interface.

To sum up, in order to broadcast the NSAG IDs supported by neighbour cells, the serving gNB can acquire the NSAG configuration from its neighbour gNBs, and generate cell reselection priorities associated with the NSAGs according to OAM configuration.  The proposal is given as follows.

- Over Xn interface, add the TAI NSAG Support List IE in the Xn interface management messages.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: The NG-RAN node is aware of the NSAG information of its neighbour nodes, and the TAC is optionally broadcasted in SIB16.

- Over Xn interface, add the TAI NSAG Support List IE in the Xn interface management messages.

The XnAP CR is provided in [2].

# 4. Reference

- R3-223792 Summary of offline discussion on slicing grouping and priority (CMCC).

- R3-224275	Correction of Slice Group Configuration	ZTE, Huawei, CMCC.

# 5. Annex – FreqPriorityListSlicing  in SIB16

#### –	FreqPriorityListSlicing

The IE FreqPriorityListSlicing indicates cell reselection priorities for slicing in SIB16.

FreqPriorityListSlicing information element

-- ASN1START

-- TAG-FREQPRIORITYLISTSLICING-START

FreqPriorityListSlicing-r17 ::= SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17

FreqPrioritySlicing-r17 ::=     SEQUENCE {

dl-ImplicitCarrierFreq-r17            INTEGER (0..maxFreq),

sliceInfoList-r17                     SliceInfoList-r17                                               OPTIONAL  -- Need R

}

SliceInfoList-r17 ::=             SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17

SliceInfo-r17 ::=                 SEQUENCE {

nsag-IdentityInfo-r17             NSAG-IdentityInfo-r17,

nsag-CellReselectionPriority-r17  CellReselectionPriority                                             OPTIONAL,  -- Need R

nsag-CellReselectionSubPriority-r17 CellReselectionSubPriority                                        OPTIONAL,  -- Need R

sliceCellListNR-r17               CHOICE {

sliceAllowedCellListNR-r17        SliceCellListNR-r17,

sliceExcludedCellListNR-r17       SliceCellListNR-r17

}                                                                                                     OPTIONAL  -- Need R

}

SliceCellListNR-r17 ::=           SEQUENCE (SIZE (1..maxCellSlice-r17)) OF PCI-Range

-- TAG-FREQPRIORITYLISTSLICING-STOP

-- ASN1STOP

FreqPriorityListSlicing field descriptions

dl-ImplicitCarrierFreq Indicates the downlink carrier frequency to which sliceInfoList is associated with. The frequency is signalled implicitly, value 0 corresponds to the serving frequency, value 1 corresponds to the first frequency indicated by the InterFreqCarrierFreqList in SIB4, and value 2 coresponds to the second frequency indicated by the InterFreqCarrierFreqList in SIB4, and so on.

SliceInfo field descriptions

nsag-IdentityInfo This is the NSAG identifier of the NSAG.

sliceAllowedCellListNR List of allow-listed neighbouring cells for slicing. If present, cells not listed in this list do not support the corresponding nsag-frequency pair, according to 38.304 [20], clause 5.2.4.11.

sliceCellListNR Contains either the list of allow-listed or exclude-listed neighbour cells for slicing.

sliceExcludedCellListNR List of exclude-listed neighbouring cells for slicing. If present, cells not listed in this list support the corresponding slice nsag-frequency pair, according to 38.304 [20], clause 5.2.4.11.