3GPP TSG-RAN WG3 #117-e                                                                                       R3-224736

Online, 15-24, August 2022

Agenda Item:	9.2.8

Source:	CATT, Samsung, CMCC

Title:	Left issues about SL relay in Rel-17

Document for:	Discussion

- Introduction

There are two left issues about SL relay in RAN3#116-e as below:

No consensus on below two bullets:

Uu/PC5 RLC channel setup in UE Context Setup procedure for Relay UE?

During remote UE initial access procedure, how to configure the SRB0/1 mapping of remote UE at the gNB-DU side (e.g., in section 8.19.1, step 11 and step 14 needs such configuration between relay UE and gNB-DU)?

In this contribution, we discuss the possible solutions and propose the related CRs in [1] and [2].

- Discussion

Remote UE initial access procedure in section 8.19.1 in TS38.401is copied here for convenient.

Figure1: Remote UE Initial Access procedure

For step 11, Uu RLC configuration for remote UE’s SRB0 message could be (re)configured by NW. DU receives SRB0 via Uu RLC channel and further send it to CU via INITIAL UL RRC MESSAGE TRANSFER message. From DU perspective, it can transfer this RRCsetup message like a normal SRB0 from a normal UE hence there is no UL mapping related issue at DU.

For step 14, DU should know the DL mapping between SRB0 and Uu RLC channel to convey the RRCsetup message to remote UE via relay UE. However, we agreed that the UE associated F1AP of remote UE is used to configure bear mapping for remote UE’s SRB0/1. The bear mapping for remote UE’s SRB0/1is configured in step 20 according to the current specification.

the UE associated F1AP message(s) of remote UE are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU

The bearer mapping via UE associated F1AP of relay UE is not needed, e.g., remove RB mapping IE in stage3 TP.

Observation 1:  DU needs bear mapping configuration for remote UE’s SRB0/1 before step 14.

There are three F1 signaling send from CU to DU before step 14:

Step 5: (relay UE) UE CONTEXT MODIFICATION REQUEST

Step 7: (relay UE) DL RRC MESSAGE TRANSFER

Step 13: DL RRC MESSAGE TRANSFER

Only step 13 is the UE associated F1AP message of remote UE, it could be used to configure bear mapping between remote UE’s SRB0 and DL Uu relay RLC channel.

Proposal 1: DL RRC MESSAGE TRANSFER in step 13 can be used to convey bear mapping configuration for remote UE’s SRB0.

Based on the proposal 1, the purpose of step 5 and step 6 is to establish Uu/PC5 Relay RLC channel(s) only i.e., without bear mapping. If Uu/PC5 Relay RLC channel(s) are established in UE Context Setup procedure for Relay UE when relay UE enters the RRC_CONNECTED status, step 5 and step 6 can be omitted. Hence including Uu/PC5 RLC channel to Be Setup List IE in UE Context Setup procedure for Relay UE is beneficial.

Proposal 2: UE Context Setup procedure for Relay UE can be used to establish Uu/PC5 Relay RLC channel.

Section 8.19.1 in TS38.401 lacks the description of the establishment of PC5 Relay RLC channel in step 3, step 5 and step 8, it should align with step 15 which can establish both Uu Relay RLC channel and PC5 Relay RLC channel.

Proposal 3: Add the description of the establishment of PC5 Relay RLC channel in step 3, step 5 and step 8 in section 8.19.1 in TS38.401.

- Conclusion

Observation 1:  DU needs bear mapping configuration for remote UE’s SRB0/1 before step 14.

Proposal 1: DL RRC MESSAGE TRANSFER in step 13 can be used to convey bear mapping configuration for remote UE’s SRB0/1.

Proposal 2: UE Context Setup procedure for Relay UE can be used to establish Uu/PC5 Relay RLC channel.

Proposal 3: Add the description of the establishment of PC5 RLC Relay channel in step  3, step 5 and step 8 in section 8.19.1 in TS38.401

- Reference

[1] R3-224737, Correction on 38.473 for SL relay, CATT, Samsung, CMCC.

[2] R3-224738, Correction on 38.401 for SL relay, CATT, Samsung.