3GPP TSG-RAN WG3 #117-e                                                                                       R3-224739

Online, 15-24, August 2022

Agenda Item:	16.3

Source:	CATT

Title:	Support Service Continuity Enhancements for SL relay

Document for:	Discussion

- Introduction

The Rel-17 SL relay WID objectives [1] include:

Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

In this contribution, we discuss the service continuity and propos the related BL CRs in [2].

- Discussion

2.1 Intra-gNB indirect-to-indirect path switching

If the signal quality of PC5 between source relay UE and  remote UE or Uu between gNB and source relay UE is not good, gNB may decide switch the remote UE from source relay UE to target relay UE based on measurement report.

After switching, gNB-CU sends UE context release command to gNB-DU and also sends a RRC reconfiguration to source relay UE to release remote UE context (e.g., release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

In Rel-17, inter/intra-gNB-DU switch from direct to indirect path have been captured in TS38.401. The procedure of the intra-gNB indirect-to-indirect path switching in R18 would be similar as section 8.19.4.1 in TS38.401. The only different is that a RRC reconfiguration message for source relay UE to release remote UE context need to be captured if we want to introduce the flow chart of Intra-gNB indirect-to-indirect path switching in TS38.401.

Proposal 1: Compared with inter/intra-gNB-DU direct-to-indirect path switching, the procedure of intra-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context. No RAN3 technical impact.

2.2 Inter-gNB path switching

a) Inter-gNB indirect-to-direct path switching

Source gNB decides handover the remote UE from indirect path to direct path based on the measurement report. Source gNB sends handover request to target gNB conveyed with target CGI, target gNB generates a RRC reconfiguration message for remote UE if target gNB accept this UE. After that, another RRC reconfiguration message should be sent to relay UE to release remote UE context.

The RRC reconfiguration message for relay UE for release purpose is allowed to be sent any time after source gNB sends RRC reconfiguration message to remote UE via relay UE based on gNB implementation.

Proposal 2: Compared with normal UE inter-gNB handover, the procedure of inter-gNB indirect-to-direct path switching should add a RRC reconfiguration message for relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE. No RAN3 technical impact.

b) Inter-gNB direct-to-indirect path switching/Inter-gNB indirect-to-indirect path switching

Remote UE measures the signal quality of neighbor relay UEs and report measurement results to source gNB. Source gNB decides handover the remote UE to target relay UE based on the measurement report reported by remote UE. The Xn handover request message should include both target CGI and target relay UE L2ID. Otherwise, target gNB does not know which target relay UE is chosen. After remote UE receives RRC reconfiguration generated by target gNB, it will trigger the PC5 connection establishment with target relay UE.

Proposal 3: For inter-gNB direct-to-indirect path switching and inter-gNB indirect-to-indirect path switching, target relay UE L2ID should be introduced in Xn handover request message.

Another issue is the authorized information. Source gNB does not know whether the target relay UE has been authorized. If the source gNB decides handover the remote UE to a unauthorized target relay UE, the handover procedure will failure.

To avoid handover failure, source gNB should know the authorized information of target relay UE. There are two alternatives:

- Alt 1:Target relay UE reports the authorized information to remote UE via measurement procedure and remote UE further sends the authorized information to source gNB via measurement report.

- Alt 2: The authorized information can be informed to source gNB by CN or target gNB.

For Alt1, it will impact air interface and the authorized information may be tampered by false relay UE. Compared with Alt1, Alt2 is a safer solution but it will impact the NG/Xn interface. RAN3 should discuss how to inform the authorized information of target relay UE to source gNB.

Proposal 4: Source gNB should know the authorized information of target relay UE.

This procedure of inter-gNB indirect-to-indirect path switching is similar as the inter-gNB direct-to-indirect path switching. The difference is that source gNB should send RRC reconfiguration to source relay UE to release remote UE context after remote UE connects to target relay UE. This RRC reconfiguration message for relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE via relay UE based on gNB implementation.

Proposal 5: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE. No RAN3 technical impact.

- Conclusion

Proposal 1: Compared with inter/intra-gNB-DU direct-to-indirect path switching, the procedure of intra-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context. No RAN3 technical impact.

Proposal 2: Compared with normal UE inter-gNB handover, the procedure of inter-gNB indirect-to-direct path switching should add a RRC reconfiguration message for relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE. No RAN3 technical impact.

Proposal 3: For inter-gNB direct-to-indirect path switching and inter-gNB indirect-to-indirect path switching, target relay UE L2ID should be introduced in Xn handover request message.

Proposal 4: Source gNB should know the authorized information of target relay UE.

Proposal 5: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE. No RAN3 technical impact.

- Reference

[1] RP-221262, Revised WID on NR sidelink relay enhancements, LG Electronics.

[2] R3-224740, Correction to 38.423 for SL relay (R18), CATT