3GPP TSG-RAN WG3 Meeting #117-e	R3-224762

E-meeting, 15 – 24 Aug, 2022

Agenda item:	9.2.8

Source:	Xiaomi, Ericsson, CMCC

Title:	Misalignment with RAN2 in RedCap broadcast information

Document for:	Discussion and Decision

1	Introduction

In RedCap R17 WI, RedCap Broadcast Information IE is introduced in the Served Cell Information over F1AP and XnAP.

As defined in RAN3 specifications, the RedCap Broadcast Information IE is represented by a bitmap, and each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, so from RAN3 perspective, whether a RedCap UE is allowed to access the cell only depends on the setting of barring info in SIB1.

However, according to RAN2’s agreement and specifications, whether a RedCap UE is allowed to access the cell depends on both the cell barring info in MIB and the RedCap barring info in SIB1.

It seems that current RAN3 specification is not aligned with RAN2, in this paper, we’d like to discuss the misalignment issue and the way to solve the issue.

2	Discussion

2.1	Misalignment with RAN2

According to the following semantics description of RedCap Broadcast Information in TS 38.423 [1] and TS 38.473[2], only the setting of RedCap barring indicators in SIB1 is considered in RAN3 specifications.

| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [10]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [10]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Observation 1, in RAN3 specifications, whether the RedCap UE is allowed to access the cell depends on RedCap barring indicator in SIB1.

However, RAN2 has different understanding on this point according to previous agreement and RAN2’s specification.

RAN2 had the following agreement in RAN2#115-e meeting according to RP-211742 [3]:

- RedCap UE applies the existing cellBarred field in MIB

Observation 2, RAN2 agreed that RedCap UE applies the existing cellBarred field in MIB.

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the UE is a RedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselectionRedCap in SIB1', if available.

Also, according to the chapter “Cell status and cell reservations” in TS 38.304 [4], if the cell is set to barred and if the UE is a RedCap UE, the UE will read the intraFreqReselectionRedCap in SIB1 to decide whether to perform the cell reselection, which proves that the cellbarred in MIB is also considered for the RedCap UE.

Observation 3, RedCap UE should consider the cellbarred in MIB according to current RAN2 specifications.

Observation 4, in RAN2 specifications, whether the RedCap UE is allowed to access the cell depends on both cell barring indicator in MIB and RedCap barring indicator in SIB1.

Observation 5, according to observation 1 and 4, RAN3’s specification is not aligned with RAN2.

2.2	Possible issue caused by the misalignment

One possible issue caused by the misalignment would be that if the cellBrrred in MIB is set to barred, the intraFreqReselectionRedCap is present in SIB1, and cellBarredRedCap1Rx/cellBarredRedCap2Rx is set to notBarred, according to the current XnAP/F1AP specifications, the RedCap broadcast information will indicate that the RedCap1Rx/RedCap2Rx is not barred as only the info in SIB1 is considered, while the corresponding RedCap UE shouldn’t access the cell as the cell is barred. Normally, the cellbar is caused by the overload issue or cell reconfiguration, we think if the RedCap UE is handed over to a cell that is barred, it may result in bad UE experience or connection failure after the handover.

Observation 6: the misalignment may cause the RedCap UE access the cell that is barred, which is not aligned with RAN2.

Proposal 1, RAN3 acknowledge the issue and fix the misalignment with RAN2 in RedCap broadcast information.

2.3	Possible way to fix the misalignment

Considering that only the broadcast information for the RedCap UEs is exchanged over XnAP and F1AP, to make the change with less spec impact and more implementation flexibility, we propose to update the semantics description of RedCap Broadcast Information in TS 38.423 and TS 38.473 as follows,

| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [10]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of cell barring indicator in MIB and/or RedCap barring indicators in SIB1, see TS 38.331 [10]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

The corresponding CRs are in [5] and [6].

Proposal 2: RAN3 agrees the CR for TS 38.423 in R3-224763 and CR for TS 38.473 in R3-224764.

3	Conclusion

We have made the following proposals:

Observation 1, in RAN3 specifications, whether the RedCap UE is allowed to access the cell depends on RedCap barring indicator in SIB1.

Observation 2, RAN2 agreed that RedCap UE applies the existing cellBarred field in MIB.

Observation 3, RedCap UE should consider the cellbarred in MIB according to current RAN2 specifications.

Observation 4, in RAN2 specifications, whether the RedCap UE is allowed to access the cell depends on both cell barring indicator in MIB and RedCap barring indicator in SIB1.

Observation 5, according to observation 1 and 4, RAN3’s specification is not aligned with RAN2.

Observation 6: the misalignment may cause the RedCap UE access the cell that is barred, which is not aligned with RAN2.

Proposal 1, RAN3 acknowledge the issue and fix the misalignment with RAN2 in RedCap broadcast information

Proposal 2: RAN3 agrees the CR for TS 38.423 in R3-224763 and CR for TS 38.473 in R3-224764.

References

[1]	TS 38.423 V17.1.0

[2]	TS 38.473 V17.1.0

[3]	RP-211742 RedCap WI status report

[4]     	TS 38.304 V17.1.0

[5]	R3-224763 Correction on RedCap Broadcast Information for TS38.423

[6]	R3-224764 Correction on RedCap Broadcast Information for TS38.473