3GPP TSG-RAN WG3 Meeting #117-e	R3-224776

Electronic Meeting, August 15th – 24th, 2022

Agenda item:		12.2.2

Source:	Intel Corporation

Title:	(TP for NR_AIML_NGRAN BL CR for TS 38.423)

Document for:		Other

# Introduction

This contribution proposes text proposal for NR_AIML_NGRAN for TS 38.423 based on our proposals discussed in [1] and [2].

# Reference

[1] R3-224774, "Discussion on Common Aspects in Stage 3 of AI/ML based Use cases", Intel Corporation

[2] R3-224775, "Discussion on Stage 3 of AI/ML based network energy saving and mobility optimization", Intel Corporation

# Text Proposal for NR_AIML_NGRAN BLCR to TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Capability | AI/ML CAPABILITY REQUEST | AI/ML CAPABILITY RESPONSE |  |
| Predicted Information Reporting Initiation | PREDICTED INFORMATION REQUEST | PREDICTED INFORMATION RESPONSE | PREDICTED INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Predicted Information Reporting | PREDICTED INFORMATION UPDATE |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.X	AI/ML procedures

### 8.X.1	AI/ML Capability

#### 8.X.1.1	General

This procedure is used by an NG-RAN node to request AI/ML capability for prediction service(s) from another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: AI/ML Capability, successful operation

The NG-RAN node1 initiates the procedure by sending the AI/ML CAPABILITY REQUEST message to the NG-RAN node2.

If the Energy Efficiency Prediction Request IE is contained in the AI/ML CAPABILITY REQUEST message, the NG-RAN node2 shall evaluate its AI/ML capability for energy efficiency prediction and respond the Energy Efficiency Prediction Support IE in the AI/ML CAPABILITY RESPONSE message.

If the Resource Status Prediction Request IE is contained in the AI/ML CPABILITY REQUEST message, the NG-RAN node2 shall evaluate its AI/ML capability for resource status prediction and respond the Resource Status Prediction Support IE in the AI/ML CAPABILITY RESPONSE message.

Interactions with other procedures:

This procedure may be initiated by the NG-RAN node1 before initiating the Predicted Information Reporting Initiation procedure.

#### 8.X.1.3	Unsuccessful Operation

Void.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Predicted Information Reporting Initiation

#### 8.X.2.1	General

This procedure is used by an NG-RAN node to request the reporting of prediction information from another NG-RAN node based on the requested NG-RAN node AI/ML capability.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Predicted Information Reporting Initiation, successful operation

The NG-RAN node1 initiates the procedure by sending the PREDICTED INFORMATION REQUEST message to the NG-RAN node2 to start a predicted information generation, stop a predicted information generation or add cells to report for a predicted information generation. Upon receipt, the NG-RAN node2:

-	shall initiate the requested predicted information generation according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells prediction measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the prediction measurements initiated before for the given prediction measurement IDs, in case the Registration Request IE is set to "add". If the prediction measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the PREDICTED INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific prediction measurements, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "add" in the PREDICTED INFORMATION REQUEST message, the Cell To Report List IE shall be included.

If the NG-RAN node2 is capable to provide all requested predicted information, it shall initiate the prediction measurements as requested by the NG-RAN node1 and respond with the PREDICTED INFORMATION RESPONSE message.

If the Validity Time IE in the PREDICTED INFORMATION REQUEST is present, the NG-RAN node2 shall generate the predicted information for the corresponding validity time.

If the Reporting Periodicity IE in the PREDICTED INFORMATION REQUEST is present, this indicates the periodicity for the reporting of the predicted information since validity time. The NG-RAN node2 shall generates the predicted information at each corresponding report periodicity. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.2-2: Predicted Information Reporting Initiation, unsuccessful operation

If any of the requested prediction measurements cannot be initiated, the NG-RAN node2 shall send the PREDICTED INFORMATION FAILURE message with an appropriate cause value.

If the requested predicted information cannot be generated within the time configured by the Validity Time IE, the NG-RAN node2 shall send the PREDICTED INFORMATION FAILURE message with an appropriate cause value.

#### 8.X.2.4	Abnormal Conditions

Void.

### 8.X.3	Predicted Information Reporting

#### 8.X.3.1	General

This procedure is initiated by an NG-RAN node to report the result of the predicted information admitted by the NG-RAN node following a successful Predicted Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2-1: Predicted Information Reporting, unsuccessful operation

The NG-RAN node2 shall report the results of the admitted predicted information in the PREDICTED INFORMATION UPDATE message. The admitted predicted information are the information that were successfully initiated during the preceding Predicted Information Reporting Initiation procedure.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

# 9	Elements for XnAP Communication

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| irrelevant operations skipped |  |  |  |  |  |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Predicted Priorioty | O |  | INTEGER (1..100) | The priority predicted for the candidate target cell. Values between 1 and 100 are ordered in decreasing order of priority, i.e., 1 is the highest and 100 is the lowest. | YES | Ignore |
| >Predicted CHO Execution Timing | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the CHO execution timing predicted for the candidate target cell. | YES | ignore |
| >Confidence Level | O |  | INTEGER (1..100) | The confidence level of prediction in percentage. | YES | ignore |
| irrelevant operations skipped |  |  |  |  |  |  |
| Predicted Information | O |  |  |  | YES | ignore |
| >Predicted Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Predicted Priority | O |  | INTEGER (1..100) | The priority predicted for the HO target cell. Values between 1 and 100 are ordered in decreasing order of priority, i.e., 1 is the highest and 100 is the lowest. | – |  |
| >Predicted HO Execution Timing | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the HO execution timing predicted for the target cell. | – |  |
| >Predicted UE Traffic | O |  | ENUMERATED (high, medium, low) | The UE traffic level predicted for the target cell after HO. | – |  |
| >Confidence Level | O |  | INTEGER (1..100) | The confidence level of prediction in percentage. | – |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.3.18	RESOURCE STATUS REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit =Number of Active UEs, Fifth Bit =RRC connections, Sixth Bit = NR-U Channel List, Seventh Bit = UE Performance, Eighth Bit = System KPI, Ninth Bit = Energy Efficiency. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  | – |  |
| >>UE To Report List |  | 0..1 |  | UE list to which the request applies | – |  |
| >>>UE To Report Item |  | 1.. <maxnoofUEContexts> |  |  | – |  |
| >>>>NG-RAN node1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node1 | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofUEContexts | Maximum no. of UE Contexts. Value is 8192. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | – |  |
| >>UE Performance | O |  | 9.2.2.aa |  | – |  |
| >>System KPI | O |  | 9.2.2.bb |  | – |  |
| >>Energy Efficiency | O |  | 9.2.2.cc |  | – |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.3.aa	AI/ML CAPABILITY REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate AI/ML capability request.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Energy Efficiency Prediction Request | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Resource Status Prediction Request | O |  | ENUMERATED (true, …) |  | YES | ignore |

#### 9.1.3.bb	AI/ML CAPABILITY RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to transfer the requested AI/ML capability information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Energy Efficiency Prediction Support | O |  | ENUMERATED (supported, Not supported, …) |  | YES | ignore |
| Resource Status Prediction Support | O |  | ENUMERATED (supported, Not supported, …) |  | YES | ignore |

#### 9.1.3.cc		PREDICTED INFORMATION REQUSET

This message is sent by NG-RAN node1 to NG-RAN node2 to request the predicted information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the prediction information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates prediction measurement object the NG-RAN node2 is requested to report. First Bit = Energy Efficiency Prediction, Second Bit = Resource Status Prediction, Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1.. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1.. <maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1.. <maxnoofBPLMNs> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1.. <maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  | – |  |
| Validity Time | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the time that model inference output becomes valid in the system. | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of the requested predicted information. It also indicates the validity time of the corresponding prediction. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.dd	PREDICTED INFORMATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the measurements for all the requested predicted information are successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.ee	PREDICTED INFORMATION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.ff PREDICTED INFORMATION UPDATE

This message is sent by the NG-RAN node2 to the NG-RAN node1 to report the results of the requested predicted information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Prediction Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Predcition Measurement Result Item |  | 1.. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Energy Efficiency Prediction | O |  | Energy Efficiency 9.2.2.cc |  | – |  |
| >>Resource Status Prediction | O |  | Radio Resource Status 9.2.2.50 |  | – |  |
| Validity Time | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the time that model inference output becomes valid in the system. | YES | ignore |
| Confidence Level | O |  | INTEGER (1..100) | The confidence level of prediction in percentage. | YES | ignore |

## 9.2	Information Element definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	NG-RAN Node and Cell Configuration related IE definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.aa	UE Performance

The UE Performance IE indicates the packet delay, packet loss and throughput of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Performance List |  | 1 |  |  | YES | Ignore |
| >UE Performance Item |  | 1..<maxnoofUEContext> |  |  | – |  |
| >>Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| >>Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| >>Average Over-The-Air Interface Packet Delay | O |  | TBD | Per DRB UL average over-the-air interface packet delay as defined in TS 38.314 [42]. | – |  |
| >>Average RLC Packet Delay | O |  | TBD | Per DRB UL average RLC packet delay as defined in TS 38.314 [42]. | – |  |
| >>Average PDCP Packet Re-ordering Delay | O |  | TBD | Per DRB UL PDCP packet re-ordering delay as defined in TS 38.314 [42]. | – |  |
| >>Packet Uu Loss Rate | O |  | TBD | Per DRB DL packet Uu loss rate as defined in TS 38.314 [42]. | – |  |
| >>DL PDCP SDU Data Volume | O |  | TBD | Per DRB DL PDCP SDU Data volume as defined in TS 38.314 [42]. | – |  |
| >>UL PDCP SDU Data Volume | O |  | TBD | Per DRB UL PDCP SDU Data volume as defined in TS 38.314 [42]. | – |  |
| >>DL UE Throughput | O |  | TBD | DL UE throughput as defined in TS 38.314 [42]. | – |  |
| >>UL UE Throughput | O |  | TBD | UL UE throughput as defined in TS 38.314 [42]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEContexts | Maximum no. of UE Contexts. Value is 8192. |

#### 9.2.2.bb	System KPI

The System KPI IE indicates the throughput, delay, RLF event status of the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE System KPI Type | M |  |  |  |  |  |
| >Packet delay |  |  |  |  |  |  |
| >>Average Delay DL Air-Interface | O |  | TBD | This measurement provides the average (arithmetic mean) time that takes for packet transmission over air interface in the downlink direction. |  |  |
| >>Distribution of Delay DL Air Interface | O |  | TBD | This measurement provides the distribution of the time that takes for packet transmission over air-interface in the downlink direction. |  |  |
| >>Average Delay UL Over-The-Air-Interface | O |  | TBD | This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. |  |  |
| >>Average RLC Packet Delay in the UL | O |  | TBD | This measurement provides the average (arithmetic mean) RLC packet delay on the uplink, i.e. the delay within the gNB-DU. |  |  |
| >>Average PDCP Re-ordering Delay in the UL | O |  | TBD | This measurement provides the average (arithmetic mean) PDCP re-ordering delay on the uplink, i.e. the delay within the gNB-CU-UP. |  |  |
| >>Distribution of DL Delay between NG-RAN and UE | O |  | TBD | This measurement provides the distribution of DL packet delay between NG-RAN and UE, which is the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface. |  |  |
| >>Distribution of UL Delay between NG-RAN and UE | O |  | TBD | This measurement provides the distribution of UL packet delay between NG-RAN and UE, which is the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface. |  |  |
| >UE throughput |  |  |  |  |  |  |
| >>Average DL UE Throughput | O |  | TBD | This measurement provides the average UE throughput in downlink. |  |  |
| >>Distribution of DL UE Throughput | O |  | TBD | This measurement provides the distribution of the UE throughput in downlink. |  |  |
| >>Average UL UE Throughput | O |  | TBD | This measurement provides the average UE throughput in uplink. |  |  |
| >>Distribution of UL UE Throughput | O |  | TBD | This measurement provides the distribution of the UE throughput in uplink. |  |  |
| >>Percentage of Unrestricted DL UE Data Volume | O |  | TBD | This measurement provides the percentage of DL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. |  |  |
| >>Percentage of Unrestricted UL UE Data Volume | O |  | TBD | This measurement provides the percentage of UL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. |  |  |
| >Measurement related to MRO |  |  |  |  |  |  |
| >>Handover Failures related to MRO for Intra-System Mobility | O |  | TBD | This measurement provides the number of handover failure events related to MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Failures related to MRO for Inter-System Mobility | O |  | TBD | This measurement provides the number of handover failure events delated to MRO detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Unnecessary Handovers for Inter-System Mobility | O |  | TBD | This measurement provides the number of unnecessary handover events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Ping-Pong for Inter-System Mobility | O |  | TBD | This measurement provides the number of handover ping-pong events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Failures for Beam-Cell Pair related to MRO for Intra-System Mobility | O |  | TBD | This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the intra-system mobility within 5GS. |  |  |
| >PDCP Data Volume for non-split gNB |  |  |  |  |  |  |
| >>DL PDCP SDU Data Volume Measurements | O |  | TBD |  |  |  |
| >>UL PDCP SDU Data Volume Measurements | O |  | TBD |  |  |  |
| >Performance measurement for split gNB |  |  |  |  |  |  |
| >>UL PDCP SDU Loss Rate | O |  | TBD |  |  |  |
| >>UL F1-U Packet Loss Rate | O |  | TBD |  |  |  |
| >>DL F1-U Packet Loss Rate | O |  | TBD |  |  |  |
| >>DL PDCP SDU Drop Rate in gNB-CU-UP | O |  | TBD |  |  |  |
| >>DL Packet Drop Rate in gNB-DU | O |  | TBD |  |  |  |
| >>Average Delay DL in CU-UP | O |  | TBD |  |  |  |
| >>Average Delay DL on F1-U | O |  | TBD |  |  |  |
| >>Average Delay DL in gNB-DU | O |  | TBD |  |  |  |
| >>Distribution of Delay DL in CU-UP | O |  | TBD |  |  |  |
| >>Distribution of Delay DL on F1-U | O |  | TBD |  |  |  |
| >>Distribution of Delay DL in gNB-DU | O |  | TBD |  |  |  |

#### 9.2.2.cc	Energy Efficiency

The Energy Efficiency IE indicates the energy efficiency status of the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Energy Efficiency Type | M |  |  |  |  |  |
| >NG-RAN data energy efficiency |  |  |  |  |  |  |
| >>NG-RAN Data Energy Efficiency | O |  | TBD |  |  |  |
| >Network slice energy efficiency |  |  |  |  |  |  |
| >>Energy Efficiency of eMBB Network Slice | O |  | TBD |  |  |  |
| >>Energy Efficiency of uRLLC Network Slice | O |  | TBD |  |  |  |
| >>Energy Efficiency of mIoT Network Slice | O |  | TBD |  |  |  |
| >PNF power consumption |  |  |  |  |  |  |
| >>Average Power | O |  | TBD |  |  |  |
| >>Minimum Power | O |  | TBD |  |  |  |
| >>Maximum Power | O |  | TBD |  |  |  |
| >PNF energy consumption |  |  |  |  |  |  |
| >>PNF Energy Consumption | O |  | TBD |  |  |  |
| >energy state |  |  |  |  |  |  |
| >>Energy State | O |  | ENUMERATED (high, low, active, deactive) |  |  |  |