3GPP TSG-RAN WG3 Meeting #117-e	R3-224777

Electronic Meeting, August 15th – 24th, 2022

Agenda item:		13.2

Source:	Intel Corporation

Title:	Discussion on Full Migration of mobile IAB-node

Document for:		Discussion and Decision

# Introduction

In the WID of mobile IAB [1], the following objective has been identified to be studied:

Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2]

In this contribution, we first discuss the mobility scenario of a mobile IAB-node, and then discuss several potential options for inter-donor migration of a mobile IAB-node.

# Discussion

## Full Migration Mobility Scenario of mobile IAB-node

When the mobile IAB-node moves around, there are two possibilities for the mobile IAB-node to perform migration.

1) intra-donor CU migration

The source IAB-donor CU may select another IAB-node as the new parent for the mobile IAB-node, i.e. the IAB-donor CU is not changed. In this case, intra-donor CU migration defined in Rel-17 is performed. The IAB-donor CU only triggers handover for the mobile IAB-MT without impacting UEs served by mIAB-node, as the F1 termination of mobile IAB-DU is not changed.

2) inter-donor CU full migration

However, since the trajectory of the mobile IAB-node is not predictable, it is possible that the mobile IAB-node moves out of the source IAB-donor CU’s coverage. Hence, the source IAB-donor CU needs to handover the mobile IAB-MT to a new target cell under another IAB-donor CU.

Proposal 1: Mobile IAB-node needs to support both intra-donor CU migration and inter-donor CU migration.

In Rel-17, for stationary IAB-node, inter-donor CU partial migration was supported to handover the IAB-MT from one IAB-donor CU to another IAB-donor CU without changing F1 termination of IAB-DU. An IP connection is established between the target IAB-donor DU and the source IAB-donor CU to allow packet re-routing. Since IAB-nodes in Rel-17 are considered as stationary IAB-nodes, the location of each IAB-node and IAB-donor is not changed. It is reasonable to assume that the IP connection between the target IAB-donor DU and the source IAB-donor CU can be established, as the location of two IAB-donors are near to each other.

Different from Rel-17 stationary IAB-node, the mobile IAB-node on the vehicle can move far away from the source IAB-donor CU. It is possible that the IP connection between the new target IAB-donor DU and the source IAB-donor CU cannot be established, for example, the mobile IAB-node moves out of the coverage of source IAB-donor CU’s IP network. In this case, a full migration without IP connection establishment also needs to be supported. For this scenario, both mIAB-MT Uu termination and mIAB-DU F1 termination needs to change from source to the target IAB-donor CU.

Observation 1: IP connectivity between the target IAB-donor DU and the source IAB-donor CU cannot be guaranteed always when a mobile IAB-node moves across IP network boundaries.

Figure 1. Mobility Scenario of mobile IAB-node

As defined in RAN3 #112e meeting, in full migration, the boundary IAB-node and the descendant IAB-nodes(s) (if any) are migrated (both RRC and F1 connection) to the 2nd IAB-donor CU from 1st IAB-donor CU.

Agree on the following terminologies and definitions: - Boundary IAB node: IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU - Partial Migration: the boundary IAB-MT is migrated to the 2nd IAB-donor-CU, while the boundary IAB-DU and descendant IAB node(s) (if any) are terminated to the 1st IAB-donor-CU. - Full Migration: the boundary IAB node and the descendant IAB node(s) (if any) are migrated (both RRC and F1 connection) to the 2nd IAB-donor-CU from 1st IAB-donor-CU.

Therefore, based on above discussion, there are two options of supporting full migration for mobile IAB-node:

Option 1: partial migration based full migration (IP connection between 1st IAB-donor CU and 2nd IAB-donor DU)

In this option, the mobile IAB-node first performs partial migration towards the target IAB-donor CU, then change the F1 termination of the mobile IAB-DU towards the target IAB-donor CU.

Option 2: standalone full migration (No IP connection between 1st IAB-donor CU and 2nd IAB-donor DU)

In this option, the mobile IAB-node migrates both Uu interface and F1 termination towards the target IAB-donor CU, without establishing IP connection with source IAB-donor CU.

Proposal 2: Based on IP connectivity between the target IAB-donor DU and the source IAB-donor CU, RAN3 to further study two options in supporting full migration of an mobile IAB-node:

Option 1: Partial migration based full migration (IP connectivity between the target IAB-donor DU and the source IAB-donor CU is available)

Option 2: Standalone full migration (IP connectivity between the target IAB-donor DU and the source IAB-donor CU is not available)

## Partial Migration based Full Migration

During Rel-17, RAN3 studied the following cases for inter-donor CU migration:

a) IAB-MT is migrated between IAB-donors. b) IAB-MT is simultaneously connected to two IAB-donors c) IAB-DU is simultaneously connected to 2 donor-CUs (common understanding is that we won’t break F1 interface principles) d) IAB-MT performs RLF recovery at new IAB-donor

Whether or how to support IAB-DU simultaneously connecting to two IAB-donor CUs was not concluded in Rel-17. However, thanks to partial migration, establishing simultaneous F1 connection towards the source and target IAB-donor CUs without using dual connectivity of IAB-MT became possible. This is because, from IAB-MT point of view, partial migration is the same as full migration where its RRC termination point is fully handed over to the target IAB-donor CU, while, based on the IP connectivity between the target IAB-donor DU and the source IAB-donor CU, F1 traffic is routed back to the source IAB-donor CU to keep the F1 termination point unchanged. If the F1 termination point is also changed (which means "full migration"), F1 traffic can definitely be routed to the target IAB-donor CU at no problem (as usual) and no such special routing at the target IAB-donor DU would be necessary. From connectivity standpoint, once partial migration is performed, simultaneous connections toward two IAB-donor CUs are possible from IAB-DU without the help of IAB-MT's dual connectivity.

Observation 2: From connectivity standpoint, once partial migration is performed, simultaneous connections toward two IAB-donor CUs are possible from IAB-DU without the help of IAB-MT's dual connectivity.

Two Logical DUs with separate or same physical cell resources during Full Migration

Though a mobile IAB-DU is able to simultaneously connect to two IAB-donor CUs during inter-donor CU migration, the principle of NG-RAN architecture has been that one DU is connected to only one CU at a time [2]. However, keeping only one F1 connection and switching its termination point from the source IAB-donor CU to the target IAB donor-CU inherits many severe drawbacks (e.g. huge service interruption). To cope with the NG-RAN principle and also to avoid potential drawbacks of keeping only one F1 connection, the concept of two logical DUs was proposed during Rel-17, where one logical DU is connected to the source IAB-donor CU, and another logical DU, at the same time, can be connected to the target IAB-donor CU. This two-logical-DU concept may not be 100% necessary for the partial migration based full migration, but we think this is critical and a viable candidate for the partial migration based full migration.

Then the question comes to how two logical DUs should be managed/designed inside of the mobile IAB-node to make the transition to full migration work flawlessly.

Based on the discussion in RAN3 #114e meeting, it is still open whether two logical DUs use separate or same physical cell resources. RAN3 sent a LS to RAN1/2/4 asking about whether the following two alternatives can be supported [3]:

R3-212981 [3]

- Alt1: the two logical DUs use separate physical cell resources

- Alt2: the two logical DUs use the same physical cell resources

Based on RAN1 reply LS [4], separate physical cell resources of two logical DUs refers to either different carriers or orthogonal time and frequency resources of the same carrier. And from RAN2 point of view, as long as the UE can treat two logical DUs as different cells, it can be supported [5].

R1-2108529 [4] For Alt1, RAN1 understands that the separate physical cell resources used by the two logical DUs may refer to different carriers, or orthogonal time and frequency resources of the same carrier. RAN1 has not identified any technical issues for Alt1. R2-2109143 [5] With respect to the above description of Alt1 and the use of “separate” physical resources, RAN2 has reached the following understanding at RAN2#115-e: R2 assumes that the UE need to be able to treat the separate resources as different cells on L1.

Additionally, RAN4 also concluded that if separate physical cell resources are used by two logical DUs, there’s no RAN4 impact [6].

Based on above observations and reply LSes from RAN1/2/4, we think it is reasonable to consider two logical DUs with separate physical cell resources at the mobile IAB-node during its full migration, as there’s no additional impact to other WGs. Such two logical DUs with separate physical cell resources can be supported by implementation.

Proposal 3: For partial migration based full migration, two logical DUs with separate physical cell resources should be supported.

Handover Request handling for served UEs

In the companion contribution [7], in order to allow UEs to differentiate two different physical cells, cells in two logical DUs with separate physical cell resources are proposed to use different PCI values. In this case, after the mobile IAB-DU establishes F1 connection with the target IAB-donor CU, the served UEs would treat two cells of two logical DUs as two separate normal cells. Then, the served UEs just need to perform handover from one logical DU to another logical DU following the legacy handover procedure.

Observation 3: The served UEs can perform the legacy handover procedure from one logical DU to another logical DU after mobile IAB-DU establishes F1 connection with the target IAB-donor CU.

During the mobile IAB-node’s full migration, the served UEs that are inside the moving vehicle move together. The handover decision for those served UEs would depend on handover of the connected mobile IAB-node.

The IAB-donor CU already knows which RRC_CONNECTED UEs are served by a mobile IAB-DU, as well as whether the MT part and DU part of the mobile IAB-node are collocated or not. When the source IAB-donor CU sends handover request for a mobile IAB-MT, it can also include handover requests for its served RRC_CONNECTED UEs toward the same target. The group mobility of those served UEs can be made possible.

Additionally, by requesting handover for the mobile IAB-node and its served UEs together from the source IAB-donor CU, the target IAB-donor CU can also perform admission control to the group of served UEs and mobile IAB-MT together. This could also help avoid the served UE’s handover being rejected by the target IAB-donor CU and connecting to another gNB after the successful handover of the mobile IAB-MT.

Proposal 4: Support group mobility for UEs served by the mobile IAB-node: When the vehicle is moving and the source IAB-donor CU decides handover of a mobile IAB-MT, the source IAB-donor CU requests group mobility/handover for those served UEs towards the same target of the mobile IAB-MT.

When HO is requested, the target IAB-donor CU needs to establish the UE contexts of those HO requested UEs toward the target IAB-donor DU, which is the new logical mobile IAB-DU (i.e. one of two logical DUs). However, this new logical mobile IAB-DU is operational after the successful F1 setup with the target IAB-donor CU. This means that handover request acknowledge for those served UEs is possible only after the mobile IAB-DU's successful F1 setup with the target IAB-donor CU.

Observation 4: For partial migration based full migration, after successful F1 setup between mobile IAB-DU and target IAB-donor CU, the target IAB-donor CU establishes UE contexts of the served UEs in the new logical DU of the mobile IAB-DU and then sends Handover Request Acknowledge for the served UEs to the source IAB-donor CU.

Although the source NG-RAN node initiates handover for the served UEs together with the mobile IAB-MT, the source NG-RAN node doesn’t need to send all their RRCReconfiguration messages including HO CMD immediately to the served UEs at the same time, which may lead to congestion over the air interface. Since F1 connections towards both source and target IAB-donor CUs can be simultaneously maintained during the mobile IAB-node’s full migration, to avoid handover congestion, before releasing the source F1 connection, the source IAB-donor CU can distribute sending RRCReconfiguration messages to the served UEs over a certain period by implementation. With that, the served UEs can be gradually handed-over to the target IAB-donor CU. After all the served UEs are handed-over, the served UEs’ contexts at the source can be released, so can be the source F1 connection (i.e. between the mobile IAB-DU and the source IAB-donor CU).

Proposal 5: To avoid handover congestion of served UEs, the source IAB-donor CU sends RRCReconfiguration messages (i.e. including HO CMD) to the served UEs over a certain period after successful F1 setup between mobile IAB-DU and target IAB-donor CU. The source F1 connection (i.e. between the mobile IAB-DU and the source IAB-donor CU) is released after the completion of all served UEs’ handovers.

Release of IP connection between source IAB-donor CU and target IAB-donor DU

During partial migration, to allow traffic re-routing from the target IAB-donor DU to the source IAB-donor CU, the source IAB-donor CU requests to establish an IP connection via IAB Transport Migration Management procedure. After all the served UEs finish their handover procedure, i.e. the source IAB-donor CU receives the UE Context Release messages for those UEs, this temporary IP connection can then be released. For this, the same IAB Transport Migration Management procedure can be reused.

Proposal 6: For partial migration based full migration, RAN3 to re-use and extend the IAB Transport Migration Management procedure so that the source IAB-donor CU, once receiving UE context release from the target IAB-donor CU for all the served UEs, can request to release IP connection with the target IAB-donor DU.

An example of partial migration based full migration procedure is shown as below:

Figure 2. Example of partial migration based full migration procedure

## Standalone Full Migration

As discussed in Section 2.1, there may be no IP connectivity between the source IAB-donor CU and the target IAB-donor DU, e.g. due to coverage. In this case, standalone full migration procedure has to be used, where does not require DU to have simultaneous connections to two donor CUs.

UE Context Request Handling

As discussed in Section 2.2, upon receiving the group handover request for the served UEs, the target IAB-donor CU needs to establish their UE contexts in its the target DU. The mobile IAB-DU is expected to support the following behaviours:

Step 1) Over the target path: UE context request/response handling towards the target IAB-donor CU

Step 2) Over the source path: RRCReconfiguration message(s) transfer between the source IAB-donor CU and the served UEs

Step 3) Over the target path: RRCReconfigurationComplete message(s) transfer between the target IAB-donor CU and the served UEs.

It can be observed that handover RRC messages are transmitted over both target and source path alternately, which requires the target path to be established before releasing the source path, i.e. mobile IAB-DU needs to have simultaneous connection to both source and target IAB-donor CU. However, during standalone full migration, it is not possible to establish simultaneous F1 connections towards the source and target IAB-donor CUs. How to support the Step 1 before establishing F1 connection between the target IAB-donor CU and the mobile IAB-DU becomes an issue.

Considering the served UEs are still connecting to the same mobile IAB-DU before and after full migration, instead of requesting to establish their UE contexts in the mobile IAB-DU over new F1 connection with the target IAB-donor CU after the source path is released, the target IAB-donor CU may perform Step 1 via the source IAB-donor CU (i.e. before the source path is released). Then, this doesn’t require the mobile IAB-DU to establish new F1 connection with the target IAB-donor CU, so that the F1 connection between the mobile IAB-DU and source IAB-donor CU can still be maintained to perform Step 2.

The UE context can either be provided by the source IAB-donor CU directly (as both mobile IAB-DU and served UEs are not changed) or the mobile IAB-DU can be requested to provide such information via the channel cross the source IAB-donor CU.

Proposal 7: For standalone full migration, the target IAB-donor CU requests the served UEs’ UE context(s) from the source IAB-donor CU or from the mobile IAB-DU via the source IAB-donor CU.

DU configuration of mobile IAB-DU

According to TS 38.473 [8], it is known that DU configuration (including served cell information, gNB-DU system information, etc) are provided by the DU. Since the same mobile IAB-DU is used during full migration, the same configuration can be reused, including PCI, resource configuration, etc.

However, the target IAB-donor CU may provide an updated gNB-DU resource configuration to the mobile IAB-DU for IAB resources. It is possible that the gNB-DU resource configuration configured by the source IAB-donor CU and the target IAB-donor CU are different with each other, which may lead to reconfiguration at mobile IAB-DU.

Observation 5: For standalone full migration, the target IAB-donor CU may provide different gNB-DU resource configuration from the one configured by the source IAB-donor CU to the mobile IAB-DU.

The same mobile IAB-DU is responsible for sending RRCReconfiguration messages to its served UEs. If the configuration and/or PCI are changed at mobile IAB-DU during full migration, the served UEs cannot receive and/or decode RRCReconfiguration message (including HO CMD) due to the mismatch of the cell configuration. Therefore, the RRCReconfiguration messages to the served UEs should be sent by the mobile IAB-DU using the source configuration and PCI values, so that the served UEs can correctly receive and execute the handover command. How to avoid/reduce the impact of changing resource configuration and PCI at the mobile IAB-DU to the served UEs becomes important.

There are two possible ways of solving this issue:

Option 1: the source IAB-donor CU sends the current gNB-DU resource configurations to the target IAB-donor CU, so that the same configuration can be continued in the mobile IAB-DU.

Option 2: the target IAB-donor CU provides a new gNB-DU resource configuration to the mobile IAB-DU via the source IAB-donor CU, while new configuration will only be applied by the mobile IAB-DU after sending RRCReconfiguration messages to the served UEs.

Proposal 8: For standalone full migration, RAN3 to consider the following two options of gNB-DU resource configuration at the mobile IAB-DU:

Option 1) the target IAB-donor CU reuses the same gNB-DU resource configuration received from the source IAB-donor CU

Option 2) the target IAB-donor CU sends a new gNB-DU resource configuration to the mobile IAB-DU, which will be applied by the mobile IAB-DU after sending RRCReconfiguration messages to the served UEs.

An example of standalone full migration procedure is shown as below:

Figure 3. Example of standalone full migration procedure

# Conclusion

In this contribution, we discussed the mobility scenarios and several potential options for inter-donor migration of a mobile IAB-node.

We have following observations and proposals:

Observation 1: IP connectivity between the target IAB-donor DU and the source IAB-donor CU cannot be guaranteed always when a mobile IAB-node moves across IP network boundaries.

Observation 2: From connectivity standpoint, once partial migration is performed, simultaneous connections toward two IAB-donor CUs are possible from IAB-DU without the help of IAB-MT's dual connectivity.

Observation 3: The served UEs can perform the legacy handover procedure from one logical DU to another logical DU after mobile IAB-DU establishes F1 connection with the target IAB-donor CU.

Observation 4: For partial migration based full migration, after successful F1 setup between mobile IAB-DU and target IAB-donor CU, the target IAB-donor CU establishes UE contexts of the served UEs in the new logical DU of the mobile IAB-DU and then sends Handover Request Acknowledge for the served UEs to the source IAB-donor CU.

Observation 5: For standalone full migration, the target IAB-donor CU may provide different gNB-DU resource configuration from the one configured by the source IAB-donor CU to the mobile IAB-DU.

Proposal 1: Mobile IAB-node needs to support both intra-donor CU migration and inter-donor CU migration.

Proposal 2: Based on IP connectivity between the target IAB-donor DU and the source IAB-donor CU, RAN3 to further study two options in supporting full migration of an mobile IAB-node:

Option 1: Partial migration based full migration (IP connectivity between the target IAB-donor DU and the source IAB-donor CU is available)

Option 2: Standalone full migration (IP connectivity between the target IAB-donor DU and the source IAB-donor CU is not available)

Proposal 3: For partial migration based full migration, two logical DUs with separate physical cell resources should be supported.

Proposal 4: Support group mobility for UEs served by the mobile IAB-node: When the vehicle is moving and the source IAB-donor CU decides handover of a mobile IAB-MT, the source IAB-donor CU requests group mobility/handover for those served UEs towards the same target of the mobile IAB-MT.

Proposal 5: To avoid handover congestion of served UEs, the source IAB-donor CU sends RRCReconfiguration messages (i.e. including HO CMD) to the served UEs over a certain period after successful F1 setup between mobile IAB-DU and target IAB-donor CU. The source F1 connection (i.e. between the mobile IAB-DU and the source IAB-donor CU) is released after the completion of all served UEs’ handovers.

Proposal 6: For partial migration based full migration, RAN3 to re-use and extend the IAB Transport Migration Management procedure so that the source IAB-donor CU, once receiving UE context release from the target IAB-donor CU for all the served UEs, can request to release IP connection with the target IAB-donor DU.

Proposal 7: For standalone full migration, the target IAB-donor CU requests the served UEs’ UE context(s) from the source IAB-donor CU or from the mobile IAB-DU via the source IAB-donor CU.

Proposal 8: For standalone full migration, RAN3 to consider the following two options of gNB-DU resource configuration at the mobile IAB-DU:

Option 1) the target IAB-donor CU reuses the same gNB-DU resource configuration received from the source IAB-donor CU

Option 2) the target IAB-donor CU sends a new gNB-DU resource configuration to the mobile IAB-DU, which will be applied by the mobile IAB-DU after sending RRCReconfiguration messages to the served UEs.

# References

- RP-221815, Mobile IAB (Integrated Access and Backhaul) for NR

- TS 38.401, NG-RAN; Architectural description

- R3-212981, LS on Inter-donor migration

- R1-2108529, Reply LS on Inter-donor migration, RAN1

- R2-2109143, Reply LS on Inter-donor migration, RAN2

- R4-2115354, Reply LS on Inter-donor migration, RAN4

- R2-2207122, "Discussion on Migration and PCI handling of mobile IAB-node", Intel Corporation

- TS 38.473, F1 application protocol