3GPP TSG-RAN WG3 Meeting #117-e	R3-224786

Electronic Meeting, August 15th – 24th, 2022

Agenda Item:	13.1

Source:	Intel Corporation, Qualcomm, Huawei, Ericsson, Nokia, InterDigital

Title:	Discussion on multi-hop scenario for mobile IAB-node

Document for:	Discussion and Decision

# Introduction

In the WID of mobile IAB [1], the following note was captured during the RAN #96 plenary:

Note: At the beginning of the work period, RAN3, RAN2 should discuss the potential complexity of a scenario where a mobile IAB node connects to a stationary (intermediate) IAB node, with respect to the scenario where a mobile IAB node connects directly to an IAB-donor.

In this contribution, we conduct the discussion recommended by this note.

# Discussion

The Rel-18 WID on mobile IAB does not preclude the possibility of having intermediate IAB nodes between a mobile IAB-node and an IAB-donor

Observation 1: The Rel-18 WID does not preclude the possibility of having intermediate IAB nodes between a mobile IAB-node and an IAB-donor.

The Rel-16/17 specification can already support multi-hop backhauling for the mobile IAB-nodes. No additional specification is needed.

Observation 2: Rel-16/17 specification can already support multi-hop backhauling for the mobile IAB-node.

Whether the mobile IAB-node connects to the IAB-donor CU or an intermediate stationary IAB-node does not impact the migration procedures of mobile IAB-node, as the hop number between mobile IAB-node and IAB-donor CU is transparent to the mobile IAB-node itself. Therefore, when full migration is triggered at the mobile IAB-node, the same message flows can be applied for both scenarios.

Observation 3: The hop number is transparent to the mobile IAB-node, and thus the same full migration procedure can be used for a mobile IAB-node, no matter whether it is connected to the IAB-donor CU directly or connected to an intermediate IAB-node.

Rel-17 IAB extensively discussed enhancements to multi-hop latency of stationary IAB networks. Since the discussions did not converge, the topic was excluded from Rel-17.

One cause for latency is the retransmission of packets discarded at intermediate nodes and source IAB-donor-DUs during IAB-node migration. Rel-17 IAB discussed enhancements to mitigate such packet discard, and these enhancements were also deprioritized during Rel-17.

Observation 4: Enhancements to multi-hop latency and mitigate packet discard at intermediate nodes and source IAB-donor-DUs during IAB-node-migration were discussed in Rel-17 IAB, and they were deprioritized during Rel-17.

However, the Rel-18 WID on mobile IAB explicitly states:

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

We think that multi-hop latency itself is not specific to IAB-node mobility. Enhancements related to multi-hop latency should therefore also be precluded in Rel-18.

Observation 5: Multi-hop latency is not specific to IAB-node mobility. Discussions on enhancements for multi-hop latency for mIAB should therefore be precluded based on Rel-18 WID.

Rel-18 WI scope was agreed to primarily focus on solutions for the mobile IAB-node’s full migration, group mobility, etc. Considering these aspects, and also considering the limited TUs available for mobile IAB, we believe that enhancements and optimizations to multi-hop backhauling should be precluded from the Rel-18 WI. For RAN 2, this would conclude the discussion on multi-hop backhauling defined in the Rel-18 WID.

Proposal 1: Discussion on enhancements/optimization to multi-hop backhauling are precluded from the mobile IAB WI, but the scenario for mobile IAB connecting to a stationary (intermediate) IAB node is not precluded.

# Conclusion

This contribution discussed the complexity between a mobile IAB-node connecting to an intermediate node vs. directly to the donor recommended in the Rel-18 WID.

Observation 1: The Rel-18 WID does not preclude the possibility of having intermediate IAB nodes between a mobile IAB-node and an IAB-donor.

Observation 2: Rel-16/17 specification can already support multi-hop backhauling for the mobile IAB-node.

Observation 3: The hop number is transparent to the mobile IAB-node, and thus the same full migration procedure can be used for a mobile IAB-node, no matter whether it is connected to the IAB-donor CU directly or connected to an intermediate IAB-node.

Observation 4: Enhancements to multi-hop latency and mitigate packet discard at intermediate nodes and source IAB-donor-DUs during IAB-node-migration were discussed in Rel-17 IAB, and they were deprioritized during Rel-17.

Observation 5: Multi-hop latency is not specific to IAB-node mobility. Discussions on enhancements for multi-hop latency for mIAB should therefore be precluded based on Rel-18 WID.

Proposal 1: Discussion on enhancements/optimization to multi-hop backhauling are precluded from the mobile IAB WI, but the scenario for mobile IAB connecting to a stationary (intermediate) IAB node is not precluded.

# References

- RP-221815, WID on Mobile IAB for NR, 3GPP TSG RAN#96, Budapest, Hungary, June 2022