3GPP TSG-RAN WG3 #117-e		R3-224788

15 – 24 Aug 2022

E-Meeting

Agenda Item:	11.2

Source:	CATT

Title:	Discussion on NR QoE for new service type

Document for:	Discussion

- Introduction

In NR-QoE enhancement WID[1] RP-221803, the detailed objectives of NR QoE for new service type work item are as follows:

- Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2].

- Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains.

In this contribution, we provide some analysis.

- Discussion

2.1 Support for new service type

In R17, there are three kinds of service type supporting and including in the specification. These are streaming services, MTSI services, and VR services. In R18, at least the MBS service should be supported since we already include it in the WI. Regarding other new service supporting, we may align with SA4 work item and check whether any new service type they will specify during RAN R18 period. RAN3 doesn’t have the capability on the metrics define of the new services. Also the responsibility of introducing new service is not in RAN group.

Observation 1: At least MBS service type should be added in R18

Proposal 1: Wait for the output of SA4 on the new service type supporting

To support new service type, from RAN3’s specification view, it is simple to enhance the QMC Configuration Information IE in the related message to include the value code for new service type for legacy QoE. Similar work should be done in the RAN2 specification.

Proposal 2: Simply update the value code of service type for legacy QoE for new service type supporting from RAN specification view

For the RV-QoE metrics, we identified two metrics for RV-QoE in R17 specification. The metrics Buffer Level and Playout Delay for Media Start-up are specified as a RAN visible QoE metric for DASH and VR service types. But these two identified metrics are not adapted to MBS and not included in the MBS metrics. After we add the MBS as new service type in R18, we should study whether any new metrics for RV-QoE for MBS service.

Proposal 3: Study whether any new metrics for RV-QoE for MBS service

The following metrics in the below table shall be derived by the MBMS client implementing QoE captured in TS 26.346[2]

| QoE Metric | Streaming delivery method | Download delivery method | Metric type |
| --- | --- | --- | --- |
| Corruption duration metric | ✓ |  | Media |
| Rebuffering duration metric | ✓ |  | Session |
| Initial buffering duration metric | ✓ |  | Session |
| Successive loss of RTP packets | ✓ |  | Media |
| Frame rate deviation | ✓ |  | Media |
| Jitter duration | ✓ |  | Media |
| Content Access/Switch Time | ✓ |  | Session |
| Network Resource | ✓ | ✓ | Session |
| Average codec bitrate | ✓ |  | Media |
| Codec information | ✓ |  | Media |
| Loss of Objects1 |  | ✓ | Session |
| Distribution of Symbol Count Underrun for Failed Blocks1 |  | ✓ | Session |

From the definition of the above metrics, we may select Network Resource and Rebuffering duration as starting for RV-QoE metrics studying. The Network Resource identifies the cell which has been used during each measurement resolution duration. The Rebuffering duration is defined as any stall in playback time due to any involuntary event at the client side.

Proposal 4: Take the two metrics as starting for MBS RAN visible QoE: Network Resource and Rebuffering duration MBMS.

- Conclusion

According to the analysis in section 2, we have:

Observation 1: At least MBS service type should be added in R18

Proposal 1: Wait for the output of SA4 on the new service type supporting

Proposal 2: Simply update the value code of service type for legacy QoE for new service type supporting from RAN specification view

Proposal 3: Study whether any new metrics for RV-QoE for MBS service

Proposal 4: Take the two metrics as starting for MBS RAN visible QoE: Network Resource and Rebuffering duration MBMS.

- Reference

[1] RP-221803 Enhancement on NR QoE management and optimizations for diverse services, CU

[2] TS 26.346 Multimedia Broadcast/Multicast Service (MBMS);Protocols and codecs