3GPP TSG-RAN WG3 #117-e																			R3-224791

15th – 24th Aug 2022

Online

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for RAN visible QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In the WID[1] of Enhancement on NR QoE management and optimizations for diverse services, the QoE supporting in NR-DC related items are included as below:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

- Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

- Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

This contribution will further discuss this work item.

2		Discussion

2.1 Support for RAN-visible QoE configuration and reporting in NR-DC

In NR-DC case, there are two NG-RAN nodes included. i.e. MN and SN. RAN visible QoE is configured by RAN node and the report is used by the RAN node. So in NR-DC case, both MN and SN can configure the RAN visible QoE measurements. After the MN configures the legacy QoE, the MN may configure the RV-QoE for MN. The MN also need send the available metrics and RRC ID to SN. And then the SN can configure the RV-QoE with same RRC ID or different RRC ID for SN. How the SN sends the RV-QoE configuration to UE may have several options: 1. Via SRB3; 2. Send to MN, then MN sends to UE. If the option2 is used, the configuration from SN should carry the indication of SN node.  So the UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.

Proposal 1: MN and SN can configure the RV-QoE separately

Proposal 2: The MN need send the available metrics and RRC ID to SN

Proposal 3: SN sends RV-QoE configuration via SRB3 or via MN with SN indication

Proposal 4: UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.

If we support the MN and SN configures the RV-QoE separately, the RV-QoE report should be separately sent to the configuration initialled node. How to send the report to the configuration initialled node may be have the below several options.

1). UE AS layer directly sends the report to the configuration initialled node, to MN via SRB4, to SN via SRB3 or like SRB4 in SN

2). UE AS layer sends the RV-QoE reports of MN and SN to one node, either MN or SN. In this case, the configuration and report should carry the indication of the node. Otherwise, the report cannot be relay to the configuration initialled node

Proposal 5: UE AS layer can send RV-QoE report to the configuration initialled node or to other node with the configuration initialled node indication for relay.

For the RV-QoE report generation, the UE app layer may be informed the configuration initialled node because the RV-QoE configuration may be different. For example, the collection metrics may be different or the periodicity of the buffer level report is different.

Proposal 6: UE APP layer may be aware the configuration initialled node and carry it in the report

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: MN and SN can configure the RV-QoE separately

Proposal 2: The MN need send the available metrics and RRC ID to SN

Proposal 3: SN sends RV-QoE configuration via SRB3 or via MN with SN indication

Proposal 4: UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.

Proposal 5: UE AS layer can send RV-QoE report to the configuration initialled node or to other node with the configuration initialled node indication for relay.

Proposal 6: UE APP layer may be aware the configuration initialled node and carry it in the report

4		Reference

- RP-221803 Enhancement on NR QoE management and optimizations for diverse services, CU