3GPP TSG-RAN WG3 #117-e																			R3-224796

15th – 24th Aug 2022

Online

Agenda item:		9.2.8

Source:				CATT

Title:	Discussion on Supporting for target NSSAI during handover

Document for:		Discussion and Decision

1		Introduction

In RAN3#115 meeting, the supporting for target NSSAI during the mobility is discussed but any consensus was made. Capture the conclusion in [1] as below:

No consensus on Target NSSAI carried in the NG:HANDOVER REQUEST.

No consensus on Target NSSAI carried in the NG:PATH SWITCH REQUEST ACKNOWLEDGE.

No consensus on Target NSSAI carried in the XN:HANDOVER REQUEST

This contribution will further discuss the open issues and provide solutions.

2		Discussion

In 23.501[2], the target NSSAI is specified as below.

If a Network Slice, S-NSSAI, is configured to be available only in TAs covering specific dedicated frequency band(s), then there may be a need to redirect the UE to the dedicated frequency band(s) when such S-NSSAI is requested. If the Requested NSSAI contains S-NSSAI(s) that are not available in the UE's current TA, see clause 5.15.8, the AMF itself or by interacting with the NSSF as described in clause 5.15.5.2.1 may determine a Target NSSAI to be used by the NG-RAN, in addition to the information the AMF receives, such as the Allowed NSSAI and the RFSP for the Allowed NSSAI, to attempt to redirect the UE to a cell and TA in another frequency band and TA that supports the S-NSSAIs in the Target NSSAI. The Target NSSAI includes at least one S-NSSAI from the Requested NSSAI not available in the current TA, but available in another TA in different frequency band possibly overlapping with the current TA, and optionally additional S-NSSAIs from the Requested NSSAI that are configured to be available within the same TAs as the S-NSSAIs not available in the current TA. If the serving PLMN supports the subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12), and if the UE has NSSRG as part of the subscription information received from the HPLMN, the Target NSSAI includes only S-NSSAIs sharing at least one NSSRG.

The Target NSSAI may be excluding some of the Allowed NSSAIs and include some of the rejected S-NSSAIs due to lack of support in the TA where the UE is located based on network policies that are in line with customer and operator agreements. The Target NSSAI shall only include S-NSSAIs that can be provided in an Allowed NSSAI for the UE. The Target NSSAI may include e.g.:

-	all or a subset of the Rejected S-NSSAIs for RA when none of the S-NSSAIs in the Requested S-NSSAI were available in the TA where the UE is;

-	all the S-NSSAIs of the Allowed NSSAI and all or a subset of the Rejected S-NSSAIs for the RA;

-	a subset of the S-NSSAIs in the Allowed NSSAI and all or a subset of the Rejected S-NSSAIs for the RA if the operator policy is to prefer this Target S-NSSAI to the Allowed NSSAI.

As we know, the rejected NSSAI is valid in the registration area (RA). The rejected NSSAI will not change in the registration area. The allowed NSSAI and rejected NSSAI will be updated when the UE move out the registration area. From the above description about target NSSAI, we may find the target NSSAI include the rejected NSSAI. We may interpret the target NSSAI may also not change within RA. Also the AMF/NSSF determines a Target NSSAI based in the requested NSSAI. So the target NSSAI should be propagated when the UE mobility at least within TA. Also within RA, the target NSSAI may be valid.

Also in the liaison [3], SA2 has agreed that the Target NSSAI at least includes all or a subset of Rejected NSSAI for RA and conditionally includes all or a subset of Allowed NSSAI, as defined in clause 5.3.4.3.3 of TS 23.501. The Target NSSAI is a quite volatile information and has one-time use. The AMF does not maintain it. The Target NSSAI is only possibly affected by the Requested NSSAI, which may contain S-NSSAI(s) that are not available in the current TA of the UE. If the subscribed NSSAI is changed, re-registration providing new Requested NSSAI from UE may be triggered. Even though SA2 state the target NSSAI is one-time use, but the target NSSAI should valid when the request NSSAI and the RA are not changed. Once the redirection happened, the target NSSAI need to be updated. So when the UE is moving in the same RA, the target NSSAI will not change and can be used by all the RAN nodes in the RA.

Proposal 1: The Target NSSAI should be propagated in the RA when the UE mobility

The NG-RAN node receive target NSSAI and store the information for the redirection before the redirection condition is satisfied. So when UE handover another NG-RAN node without RAU, the target NSSAI should be transferred to the target NG-RAN node via XnAP HANDOVER REQUEST.

Proposal 2: Target NSSAI can be carried in XnAP message: HANDOVER REQUEST.

Proposal 2a: corresponding CR is provided for XnAP in R3-224797

When the NG handover performed, the target NSSAI also should be transferred to the target NG-RAN node. SA2 state the target NSSAI is one-time use, but the AMF should maintain it before the new target NSSAI derived. If SA2 does not agree to maintain this target NSSAI in AMF, the target NSSAI can be included in the Source to Target Transparent Container in handover required and handover request message. Also the target NSSAI can be carried in NGAP message PATH SWITCH REQUEST ACKNOWLEDGE and HANDOVER REQUEST for the AMF updating the target NSSAI..

Proposal 3: Target NSSAI can be carried in the Source to Target Transparent Container in HANDOVER REQUIRED/REQUEST during NG handover

Proposal 4: Target NSSAI can be updated during handover in message: PATH SWITCH REQUEST ACKNOWLEDGE and HANDOVER REQUEST.

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: The Target NSSAI should be propagated in the RA when the UE mobility

Proposal 2: Target NSSAI can be carried in XnAP message: HANDOVER REQUEST.

Proposal 2a: corresponding CR is provided for XnAP in R3-224797

Proposal 3: Target NSSAI can be carried in the Source to Target Transparent Container in HANDOVER REQUIRED/REQUEST during NG handover

Proposal 4: Target NSSAI can be updated during handover in message: PATH SWITCH REQUEST ACKNOWLEDGE and HANDOVER REQUEST.

4		Reference

- R3-222563 Summary of Offline Discussion on CB: # RANSlicing3_UESliceMBR, ZTE

- TS 23.501 System architecture for the 5G System (5GS); Stage 2

- S2-2200255 Reply LS on Clarification on the change of Target NSSAI