3GPP TSG-RAN WG3 #117-e	Error! Unknown document property name.

Online, 15th - 24th   August 2022

Agenda Item:	8.1

Source:	Ericsson

Title:		Proposition for RAN3 feedback to SA2 on Rel-18 RedCap enhancement study

Document for:	Discussion

# Introduction

SA2 is informing RAN3 in their LS on Rel-18 RedCap study "FS_REDCAP_Ph2" targeting support of UE in RRC_INACTIVE state with long eDRX>10.24s of the following selected solutions for normative phase [1]:

SA2 Rel-18 study FS_REDCAP_Ph2 is ongoing and approaching the conclusion phase. The study is focused on one key issue which enables long eDRX>10.24s cycle support for UE in RRC_INACTIVE state. See more details in latest TR 23.700-68 (link). SA2 discussed mainly two different types of solutions: A) NG-RAN provides UE unreachability information (e.g. the eDRX information) to CN when UE enters RRC_INACTIVE state with long eDRX and CN handles the MT data/signalling while the UE is unreachable. CN triggers MT data/signalling when the UE is considered reachable. For example, see solution#6 in TR 23.700-68 for reference. B) NG-RAN handles MT data/signalling while the UE is RRC_INACTIVE state. In case UE moves out of the RNA area during the unreachable time period and performs resume outside the RNA, the UE context retrieval between NG-RAN nodes and data forwarding are supported via CN when there is no Xn interface. For example, see solution#2 in TR 23.700-68 for reference.

Furthermore, SA2 asks RAN2 and RAN3 for feedback on the following points:

SA2 is starting to consider its conclusion and would like to ask RAN2 and RAN3 on their views on the following aspects: - NG-RAN providing UE unreachability information to CN for MT data/signalling handling when UE is not reachable in RRC_INACTIVE state. - NG-RAN can handle a new NG_AP message to trigger RAN paging when UE is in RRC-INACTIVE. - Including the UE context retrieval with data forwarding handling between NG-RAN nodes via CN. - NG-RAN buffering capabilities of MT data for the duration of the eDRX cycle. - NG-RAN’s ability to perform UE context release procedure towards the AMF and locally releases the UE to RRC-IDLE when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s. - Alternative to (5): NG-RANs ability to only provide an indication to AMF when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s. The UE remains in RRC_INACTIVE.

In the following, we analyze the impacts of the listed solutions.

# Discussion

The key issue defined by SA2 is how to support long eDRX values for RedCap UEs, longer than 10.24 seconds in RRC_INACTIVE state. Similar to what was done in Release-17, we can consider that these long eDRX values will be generalized for NR UEs with RRC_INACTIVE and not only limited to RedCap UEs.

Observation 1: RAN3 can consider that the long eDRX values >10.24 sec will be generalized for NR UEs with RRC_INACTIVE.

The two solutions listed by SA2 in the LS are:

- Solution A: represents a CN-centric approach, where CN will handle the MT Data and signalling;

- Solution B: is a RAN-centric where RAN handles the signalling.

## CN-centric solution

The flow-chart for the CN-centric approach is described as follows from Solution #6 described in the SA2 TR [2].

Figure1: UE entering CM-CONNECTED with RRC Inactive and eDRX>10.24s

From the description of the CN-centric solution in the SA2 TR solution #6, we can see that after the NG-RAN configures the UE to RRC_INACTIVE state in step 3 - e.g. when receiving the Core Network Assistance Information for RRC INACTIVE IE during the INITIAL UE CONTEXT SETUP REQUEST message, or any other NG-AP message carrying this IE - it feedbacks to AMF the configured eDRX cycle information for RRC INACTIVE. The NG-AP messages from AMF in step 2 can be enhanced by adding a request IE for the eDRX information, so that NG-RAN can differentiate the N2 message coming from legacy AMFs and from AMFs that support the CN-centric approach.

In the same way, the AMF awareness of RRC_INACTIVE eDRX can also be supported by enhancing the already existing procedure for RRC INACTIVE Transition Report defined in TS 38.413 by adding the eDRX cycle values for RRC_INACTIVE. Below an example of such impact.

#### 9.2.2.10 RRC INACTIVE TRANSITION REPORT This message is sent by the NG-RAN node to notify the 5GC the UE enters or leaves RRC_INACTIVE state. Direction: NG-RAN node AMF IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. RRC State; M; 9.3.1.92; YES; ignore. User Location Information; M; 9.3.1.16; YES; ignore. NR Paging eDRX Information for RRC INACTIVE; O; ENUMERATED (hfquarter, hfhalf, hf1, …); TeDRX, RAN defined in TS 38.304 [x]. Unit: [number of hyperframes].; YES; ignore.

Observation 2: The CN-centric approach requires AMF awareness of the RRC_INACTIVE eDRX cycle information for the configured UE

Proposal 1: The AMF awareness of the RRC_INACTIVE eDRX cycle information can be realized by NG-RAN signalling the configured information as response to messages in class1 procedures carrying the Core Network Assistance Information for RRC INACTIVE IE with a new eDRX Request IE and by enhancing the RRC INACTIVE TRANSITION REPORT message

Based on this information from NG-RAN, the AMF will deduce for how long the UE will be unreachable with long eDRX to enable data buffering at UPF. This is similar to CM-IDLE mode case, except that AMF has to ask NG-RAN to trigger the RAN paging, by sending a N2 notification in the step 2 shown in the Figure [3] for Network Triggered Connection Resume procedure:

Figure2: UE entering CM-CONNECTED with RRC Inactive and eDRX>10.24s

If UE does not respond, AMF will then escalate CN paging within RA.

Observation 3: AMF sends a new request to NG-RAN with new indication to trigger RAN paging. If UE does not respond, the AMF can escalate CN paging within RA.

The message in step 1b can be either a DL NAS message or a new message for notification DL MT data. In the case of MT data delivery, a new message would need then to be introduced by RAN3 for NG-RAN to trigger the RAN paging. An example of such new procedure can be as proposed below:

#### <<Procedure example>> ### 8.5.x Data Notification #### 8.5.x.1 General The purpose of the Data Notification procedure is for the AMF to notify the NG-RAN that the UE has pending downlink data or signaling at the core network. #### 8.5.x.2 Successful Operation Figure 8.5.x.1: Data Notification Procedure. Successful operation. The AMF initiates the procedure by sending the DATA NOTIFICATION message to the NG-RAN. The NG-RAN should use this information to trigger RAN Paging. #### <<Tabular example>> #### 9.2.4.x Data Notification The message is sent by the AMF to notify NG-RAN that the UE has pending Downlink Data or signaling in the network. Direction: AMF NG-RAN IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Data Notification; M; ENUMERATED (true, …); YES; reject.

Proposal 2: A new message DATA NOTIFICATION from AMF to NG-RAN needs to be introduced for NG-RAN to trigger the RAN paging in case of MT Data. RAN3 to take the proposed message as basis.

Another aspect of consideration is mobility outside of the RNA. RAN3 can consider that legacy behaviour will apply for the CN-centric approach, where the UE is released to RRC_IDLE and data is dropped if there is any.

Proposal 3: For mobility outside the RNA, RAN3 can consider that legacy behaviour will apply for the CN-centric approach, where the UE is released to IDLE and data is dropped if there is any.

## RAN-centric solution

The figure below from [4] illustrates the RAN-centric approach for UE is in RRC-Inactive with long eDRX.

Figure 4: MT signalling for UE is in RRC-Inactive with long eDRX

In the RAN-centric approach, the NG-RAN handles the signalling as in legacy in the sense that it pages the UE to deliver the MT NAS signalling (step 6) received from the DL NAS message (step 1). When NG-RAN cannot deliver any NAS message signalling or the buffering capability is reached (step 3), NG-RAN moves the UE to RRC_IDLE state and informs CN (step 3), via e.g., NAS NON DELIVERY INDICATION message.

Observation 4: If DL NAS is not delivered, there is a mechanism for DL NAS non delivery message in NG-AP, nothing else is needed.

Regarding the NG-RAN capability for buffering, it depends on various reasons, such as the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, etc. Thus, RAN3 can consider these aspects are mostly up to the deployment or implementation.

Observation 5: The NG-RAN buffering capability depends on the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, etc., which cannot be captured in specifications.

Alternatively to releasing the UE to RRC_IDLE state, if the NG-RAN buffering capacity is reached and to avoid data loss, the NG-RAN can request the CN to do buffering instead. Switching of buffering to CN while keeping the UE in RRC INACTIVE can help support the benefits of longer eDRX with RRC INACTIVE. This will require NG-RAN to signal to AMF the eDRX cycle configuration for RRC_INACTIVE, so that the unreachability information is known at CN. In this regard, both solutions RAN-centric and CN-centric approaches can be supported and complementing each other, where NG-RAN can first decide by implementation which buffering option to choose; once the NG-RAN buffering is selected, the NG-RAN can transfer to CN buffering solution smoothly and signal the RRC_INACTIVE eDRX information.

Proposal 4: If the NG-RAN buffering capacity is reached, NG-RAN can request the CN to do buffering instead to avoid data loss.

Proposal 4bis: NG-RAN can decide which buffering option to choose based on network implementation; once the NG-RAN buffering is selected, the NG-RAN can transfer to CN buffering solution smoothly and signal the RRC_INACTIVE eDRX information.

Proposal 5: both RAN-centric and CN-centric buffering approaches can be supported

One aspect however that constitutes the main impact of the RAN-centric approach, is the context fetch done via the CN and optional data forwarding over NG-AP in case the UE moves to an area where there is no Xn connectivity. This is described in the following section [5] form the TR:

Figure 5: Procedure for handling UE mobility outside the RNA

First, if there is no Xn connectivity, then the RNA will not include nodes that do not have Xn connection with the serving gNB; only one gNB in this case constitutes the RNA. Otherwise, the UE should not be kept in RRC_INACTIVE.

Observation 6: In case of absence of Xn connectivity, the RRC_ INACTIVE UE will not resume in node that do not have Xn-AP connection with the serving gNB

Furthermore, a new "NG-AP Retrieve UE Context" procedure to retrieve the UE context between RAN nodes over the CN cannot be specified without considering the full RAN3 specification impacts that it may induce, where the definition of the RNA will be impacted to include nodes connected via NG only. Such (non-exhaustive) list of impacts can be:

- There could be a need to enhance the NG-AP interface management procedures such as the NG Setup, RAN and AMF Configuration Update procedures to exchange the served cell information, so that the anchor gNB can set up the RNA for the gNBs that are only connected over NG.

- Considering Observation 1 above and that SDT is also a feature for RRC_INACTIVE UE, procedures for a new NG-AP Retrieve UE Context should also support the case of SDT with and without anchor relocation. Thus, more procedures need to be defined.

- To know which version of the ResumeMAC-I/shortResumeMAC-I to use, source and target gNBs need to exchange data from SIB to verify if the feature is supported at both nodes and in the UE

Besides, we see that some optimization can be considered in the case of the introduction of a UE context retrieval procedure over NG-AP and RAN buffering capacity is to be reached, where the NG-RAN can “upload” a copy of the RAN UE Context directly to AMF when e.g. UE goes to RRC_INACTIVE with the RAN buffered data, asking CN to proceed with data buffering.

Figure 6: Old NG-RAN can upload copy of UE context directly to CN when UE goes to RRC_INACTIVE or decides to switch to CN buffering, e.g. due to RAN buffering capacity reached. Network signalling is saved

Later, instead of fetching the UE context from the old NG-RAN node as shown in step 5 of Figure 5, the CN can send the stored UE context directly to the new NG-RAN node in step 4. This saves network signalling as steps 5 and 6 from Figure 5 can be skipped. AMF can inform the old NG-RAN to delete the old UE context. An example of the signalling is provided in Fig.6

Proposal 6: In case of absence of Xn connectivity and RAN buffering is reached, the NG-RAN can upload the RAN UE context to CN and switch to CN buffering. When UE resumes to a new NG-RAN node, the CN can send the RAN UE context directly to the new NG-RAN as response to NG-AP context fetch request.

All in all, while introducing a new procedure for the NG-AP context fetch to support the RAN-centric approach can be considered to cater to the absence of Xn-connectivity, the NG-AP specification impacts are quite important and need more careful assessment by the group. Before delving into the CR work, RAN3 can communicate to SA2 that RAN3 can further assess the specification work for NG-AP context fetch if SA2 decides to pursue this option in the normative phase.

Observation 7: while introducing a new procedure for the NG-AP context fetch to support the RAN-centric approach can cater to the absence of Xn-connectivity, the NG-AP specification impacts are quite significant and need more careful assessment by the group.

Proposal 7: Before delving into detailed CR work, RAN3 can communicate to SA2 that it will continue assessing the impacts of defining a new NG-AP Retrieve UE Context provided that SA2 decides to pursue this option in the normative phase.

# Summary

Based on the above analysis, we propose to provide the following answers to SA2’s LS. We note that the questions can be categorized based on the two solutions A et B. A LS capturing the below answer is provided in [6]:

CN-centric approach questions:

SA2 question 1: NG-RAN providing UE unreachability information to CN for MT data/signalling handling when UE is not reachable in RRC_INACTIVE state.

RAN3 answer: RAN3 considers it feasible to provide the configured eDRX cycle information for RRC_INACTIVE to AMF by re-using existing NGAP messages and adding the necessary IEs

SA2 question 2: NG-RAN can handle a new NG_AP message to trigger RAN paging when UE is in RRC-INACTIVE.

RAN3 answer: RAN3 considers it feasible for NG-RAN to trigger RAN Paging based on receiving a NG-AP message from AMF. This can be a DL NAS TRANSPORT message in case of signalling, or a new DATA NOTIFICATION message in case of MT Data.

RAN-centric approach questions:

SA2 question 3: Including the UE context retrieval with data forwarding handling between NG-RAN nodes via CN.

RAN3 answer: RAN3 considers that support of UE context retrieval with data forwarding handling between NG-RAN nodes via CN is feasible but has some significant NG-AP specification impacts and needs more discussion. One solution in case of absence of Xn connectivity and RAN buffering is reached, is that the NG-RAN uploads the RAN UE context to CN and asks to switch to CN data buffering. When UE resumes to a new NG-RAN node, the CN can send the RAN UE context directly to the new NG-RAN as response to NG-AP context fetch request. RAN3 can continue to assess this solution in case SA2 decides to pursue this option in the normative phase.

SA2 question 4: NG-RAN buffering capabilities of MT data for the duration of the eDRX cycle.

RAN3 answer: The NG-RAN buffering capability depends on the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, etc., which cannot be captured in specifications. If the NG-RAN buffering capacity is reached, NG-RAN can request the CN to do buffering instead and both buffering approaches can be supported.

SA2 question 5-6: NG-RAN’s ability to perform UE context release procedure towards the AMF and locally releases the UE to RRC-IDLE when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s.

Alternative to (5): NG-RANs ability to only provide an indication to AMF when receiving DL NAS message and the UE is not reachable for a time period longer than 10.28s. The UE remains in RRC_INACTIVE.

RAN3 answer: RAN3 considers the above to be legacy behaviour. If DL NAS is not delivered, there is a mechanism for DL NAS non delivery message in NG-AP, nothing else is needed.

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: RAN3 can consider that the long eDRX values >10.24 sec will be generalized for NR UEs with RRC_INACTIVE.

Observation 2: The CN-centric approach requires AMF awareness of the RRC_INACTIVE eDRX cycle information for the configured UE

Proposal 1: The AMF awareness of the RRC_INACTIVE eDRX cycle information can be realized by NG-RAN signalling the configured information as response to messages in class1 procedures carrying the Core Network Assistance Information for RRC INACTIVE IE with a new eDRX Request IE and by enhancing the RRC INACTIVE TRANSITION REPORT message

Observation 3: AMF sends a new request to NG-RAN with new indication to trigger RAN paging. If UE does not respond, the AMF can escalate CN paging within RA.

Proposal 2: A new message DATA NOTIFICATION from AMF to NG-RAN needs to be introduced for NG-RAN to trigger the RAN paging in case of MT Data. RAN3 to take the proposed message as basis.

Proposal 3: For mobility outside the RNA, RAN3 can consider that legacy behaviour will apply for the CN-centric approach, where the UE is released to IDLE and data is dropped if there is any.

Observation 4: If DL NAS is not delivered, there is a mechanism for DL NAS non delivery message in NG-AP, nothing else is needed.

Observation 5: The NG-RAN buffering capability depends on the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, etc., which cannot be captured in specifications.

Proposal 4: If the NG-RAN buffering capacity is reached, NG-RAN can request the CN to do buffering instead to avoid data loss.

Proposal 4bis: NG-RAN can decide which buffering option to choose based on network implementation; once the NG-RAN buffering is selected, the NG-RAN can transfer to CN buffering solution smoothly and signal the RRC_INACTIVE eDRX information.

Proposal 5: both RAN-centric and CN-centric buffering approaches can be supported

Observation 6: In case of absence of Xn connectivity, the RRC_ INACTIVE UE will not resume in node that do not have Xn-AP connection with the serving gNB

Observation 7: while introducing a new procedure for the NG-AP context fetch to support the RAN-centric approach can cater to the absence of Xn-connectivity, the NG-AP specification impacts are quite significant and need more careful assessment by the group.

Proposal 6: In case of absence of Xn connectivity and RAN buffering is reached, the NG-RAN can upload the RAN UE context to CN and switch to CN buffering. When UE resumes to a new NG-RAN node, the CN can send the RAN UE context directly to the new NG-RAN as response to NG-AP context fetch request

Proposal 7: Before delving into detailed CR work, RAN3 can communicate to SA2 that it will continue assessing the impacts of defining a new NG-AP Retrieve UE Context provided that SA2 decides to pursue this option in the normative phase.

# References

S2-2204989, LS on FS_REDCAP_Ph2 option feasibility, SA2

TR 23.700-68, Figure 6.6.3.1-1: UE entering CM-CONNECTED with RRC Inactive and eDRX>10.24s

Ibid, Figure 6.6.3.3-1: Network Triggered Connection Resume procedure

Ibid, Figure 6.2.3.1-1: MT signalling for UE is in RRC-Inactive with long eDRX

Ibid, Figure 6.2.3.3-1: Procedure for handling UE mobility outside the RNA

R3-224801, [Draft] Reply LS On FS_REDCAP_Ph2 TR option feasibility, Ericsson, RAN3#117-e