3GPP TSG-RAN WG3 #117-e	 R3-224811

Online, 15th - 24th   August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0084 | rev |  | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the PRS Measurement configuration procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) When PPW is preconfigured, but UE instead sends a request for Measurement gap, the gNB can release the PPW resources. The gNB can then inform the LMF that neither of the requested PPW/MG can be pre-configured and that another Measurement gap is in use by the UE. 2) After Xn the handover confirmation by the target, the source gNB indicates in the MEASUREMENT PRECONFIGURATION CONFIRM message the serving cell ID) to the LMF, so that LMF can send the MEASUREMENT ACTIVATION message to the target node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of a Result Cause IE, conditonally present if the Preconfiguration Result IE bits are all set to the value "0". - Addition of the NR CGI IE in the MEASUREMENT PRECONFIGURATION CONFIRM message - Addition of appropriate procedure text and ASN.1 Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only introduces a new IE in the MEASUREMENT PRECONFIGURATION CONFIRM message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current structure of the MEASUREMENT PRECONFIGURATION CONFIRM message is not useful when informing LMF that none of the PPW/MG are preconfigured. The LMF cannot know when the PPW resources are released by gNB due to UE unable of using gapless PRS measurements and instead goes with a MG. The LMF cannot know to which node it should send the MEASUREMENT ACTIVATION message when UE has performed Xn mobility |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2.2, 9.1.1.25, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0884 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

#### 8.2.12.1	General

The Measurement Preconfiguration procedure allows the LMF to provide necessary information to the serving gNB and request the gNB to preconfigure measurement gap and/or PRS processing window for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.12.2	Successful Operation

Figure 8.2.12.2-1: Measurement Preconfiguration procedure, successful operation

The LMF initiates the procedure by sending a MEASUREMENT PRECONFIGURATION REQUIRED message.

If the NG-RAN node is able to configure measurement gap or PRS processing window, it shall reply with the MEASUREMENT PRECONFIGURATION CONFIRM message.

If the Result Cause IE is included in the MEASUREMENT PRECONFIGURATION CONFIRM message, the LMF shall consider that the gNB has released the PRS processing window resources.

If the Cell ID IE is included in the MEASUREMENT PRECONFIGURATION CONFIRM message, the LMF shall, if supported, initiate the MEASUREMENT ACTIVATION message towards the NG-RAN node hosting that cell ID.

<Next change>

#### 9.1.1.25	MEASUREMENT PRECONFIGURATION CONFIRM

This message is sent by the NG-RAN node to the LMF to confirm successful configuration of measurement gap or PRS processing window of the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Preconfiguration Result | M |  | BIT STRING (SIZE(8)) | Indicates what has been preconfigured in the UE. first bit: measurement gaps second bit: PRS processing windows. Other bits reserved for future use. Value '1' indicates 'has been preconfigured', Value '0' indicates 'not preconfigured'. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Result cause | C-ifResultZero |  | Enumerated(other MG in use,…) |  | YES | ignore |
| Cell ID | O |  | NR CGI 9.2.9 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResultZero | This IE shall be present if the Preconfiguration Result IE bits are all set to the value "0". |

<Next change>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

<unchaged text omitted>

MeasurementAmount,

PreconfigurationResult,

RequestType,

ResultCause

<unchaged text omitted>

id-MeasurementAmount,

id-PreconfigurationResult,

id-RequestType,

id-ResultCause

-- **************************************************************

--

-- MEASUREMENT PRECONFIGURATION CONFIRM

--

-- **************************************************************

MeasurementPreconfigurationConfirm::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationConfirm-IEs}},

...

}

MeasurementPreconfigurationConfirm-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PreconfigurationResult		CRITICALITY	ignore	TYPE PreconfigurationResult		PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-ResultCause					CRITICALITY ignore	TYPE ResultCause				PRESENCE conditional }|-- The IE shall be present if Preconfiguration Result IE bits are all set to the value "0"

{ ID id-CGI-NR						CRITICALITY ignore	TYPE CGI-NR						PRESENCE 	optional},

...

}

<Next change>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

<unchaged text omitted>

#### -- R

<unchaged text omitted>

ResultCause ::= ENUMERATED {other MG in use,...)

<Next change>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

<unchaged text omitted>

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementAmount											ProtocolIE-ID ::= 95

id-pathPower													ProtocolIE-ID ::= 96

id-PreconfigurationResult										ProtocolIE-ID ::= 97

id-RequestType													ProtocolIE-ID ::= 98

id- ResultCause													ProtocolIE-ID ::= xx