3GPP TSG-RAN WG3 Meeting #117-e	R3-224834

Online, 15th– 24th August, 2022

Agenda item:	14.3

Source:	Samsung

Title:	(TP to TS37.340 on Mobility Enhancements) Considerations on

CHO+CPA/CPC procedure

Document for:	Discussion & Decision

Introduction

The WID for Rel-18 Further NR mobility enhancements was approved [1], and the objectives4 about CHO+CPA/CPC. The objective4 is given below.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

In this contribution we will give our considerations on CHO+CPA/CPC procedure. And the text proposal to TS 37.340 is proposed in Annex.

Discussion

CHO+CPA/CPC, as the enhancement of CHO, the procedure of which will still be 3 phases like CHO:

- Phase1: CHO+CPA/CPC preparation

- Phase2: Evaluate the CHO execution conditions and the CPA/CPC execution conditions

- Phase3: CHO+CPA/CPC execution

However, with multiple candidate MNs and candidate SNs involvement simultaneously, the CHO+CPA/CPC should be designed based on Rel-16 CHO and Rel-17 CPA/CPC.

Proposal1: CHO+CPA/CPC procedure should be designed based on Rel-16 CHO and Rel-17 CPA/CPC procedure.

CHO+CPA/CPC preparation

    - CHO+CPA/CPC configuration

During CHO+CPA/CPC preparation, multiple candidate target SNs might be involved in each CHO preparation procedure to prepare radio resource for UE. Therefore multiple SN addition procedures are triggered by HANDOVER REQUEST from S-gNB/S-MN. During the CPA/CPC preparation, it might also occur that multiple candidate PCells under the same T-MN may perform multiple CPA/CPC procedures towards the same T-SN. And we suggest each CPA/CPC procedure to the same candidate SN is performed through SN addition procedure.

Proposal 2：SN addition procedure might be triggered by each HANDOVER REQUEST from S-gNB/S-MN. If multiple candidate PCells under the same T-MN perform multiple CPA/CPC procedures towards the same T-SN, each CPA/CPC procedure is performed through SN addition procedure.

    - CHO+CPA/CPC configuration modification

Before the execution of CHO/CPA/CPC, CHO+CPA/CPC configuration might update/be modified. The possible scenarios of CHO+CPA/CPC configuration updating/modified before UE executing CHO/CPA/CPC are shown below:

T-MN initiated:

- Scenario1-1:   CPA/CPC configuration modification/update for PSCells corresponding to one candidate PCell

- Scenario1-2：Cancel all PSCells corresponding to one candidate PCell

- Scenario1-3：Cancel all candidate PSCells on one candidate Target SN

T-SN initiated:

- Scenario 2-1：CPA/CPC configuration modification/update for PSCells corresponding to one candidate PCell

- Scenario 2-2：Cancel all PSCells corresponding to one candidate PCell

- Scenario 2-3：Cancel some PSCells corresponding to one candidate PCell

- Scenario 2-4：Add prepared PSCells corresponding to one candiate PCell within the limit

- Scenario 2-5：Cancel all prepared Pscells of T-SN

S-MN initiated:

- Scenario 3-1: The candidate PSCell becomes worse than absolute threshold or neighbour cell becomes better than absolute threshold.

Observation 1: CHO+CPA/CPC configuration modification might be triggered by T-MN or T-SN, and new message might to be introduced for handover preparation modification triggered by candidate target nodes.

Proposal 3: The scenarios for CHO+CPA/CPC configuration modification and the new message for handover preparation modification triggered by candidate target nodes should be discussed.

CHO+CPA/CPC evaluation and execution

CHO condition and CPA/CPC condition should be both configured to UE and should be both evaluated by UE.

    - Evaluation of CHO/CAPC execution condition

In terms of UE behavior, the condition evaluation can be categorized into 2 modes:

1) Mode-1 (evaluation in sequence): UE first evaluates CHO condition and access target PCell, then evaluates CPA/CPC condition for configured CPA/CPC candidate PSCells associated with the target PCell or for all configured CPA/CPC candidate PSCells, which may depends on how the association between PCell(s) and PSCell(s) candidates is configured.

2) Mode-2 (evaluation simultaneously): UE evaluates CHO condition and CPA/CPC condition at the same time for the associated pairs, need to specify the definition of "CHO condition and CPA/CPC condition satisfy simultaneously" (e.g. satisfy one after another within a duration).

Observation 2: UE’s behavior is different in different condition evaluation modes.

Proposal 4: The two modes of CHO+CPA/CPC execution condition evaluation are both feasible and should be discussed to specify UE’s behavior.

    - Potential scenarios for the satisfaction of CHO/CAPC execution condition

CHO execution condition and CPA/CPC execution condition might not be satisfied simultaneously, shown in table1.

Table1: Scenarios for satisfaction of CHO/CAPC execution condition

| scenarios | Satisfaction of CHO execution condition meet | Satisfaction of CPA/CPC execution condition | PCell associated with PSCell |
| --- | --- | --- | --- |
| 0 | √ | √ | √ |
| 1 | √ | √ | x |
| 2 | √ | x | - |
| 3 | x | √ | - |

Observation 3: There are different scenarios for satisfaction of CHO/CAPC execution condition.

For different scenarios, when one of the execution condition is satisfied, UE’s behavior should be specified, such as continuing to evaluate the other execution condition or performing other operations.

Proposal 5: UE’s behavior should be specified for the different scenarios for satisfaction of CHO/CAPC execution condition. And we think scenario0 and 2 might be the staring points.

    - Combination of “Evaluation” and “Satisfaction”

“Evaluation” and “Satisfaction” are two aspects w.r.t UE behavior on CHO and CPA/CPC execution condition, and “satisfaction” part is more related to the consequence of “evaluation”. Thus, it is better to combine them together for discussion, i.e.

- Evaluation Mode-1 with satisfaction scenario 0~2

- Evaluation Mode-2 with satisfaction scenario 0~3

We suggest mode-1 with satisfaction scenario0/2, and mode-2 with satisfaction scenario0/2 as the starting point.

Proposal 6: UE’s behavior in evaluation of CHO/CAPC execution condition and in scenarios for the satisfaction of CHO/CAPC execution condition should be studied. And UE’s behavior study in mode-1 with satisfaction scenario0/2, and mode-2 with satisfaction scenario0/2 can be  the starting point.

Others

On Data forwarding

In CHO+CPA/CPC, multiple T-SNs may be prepared. To support early data forwarding, S-MN/S-SN may need forward the same data to different T-SNs. This will result in redundant data forwarding. Thus, the indirect data forwarding may be a better choice in terms of saving data forwarding resource, such as the data is forward to T-MN side first, and then T-MN determines the further forwarding towards target T-SN after UE selects the target PSCell.

Proposal 7: The method for early data forwarding should be discussed to reduce the redundant data transmission.

Conclusion

Based on the discussion, we have the following proposals

Proposal 1: CHO+CPA/CPC procedure should be designed based on Rel-16 CHO and Rel-17 CPA/CPC procedure.

Proposal 2：SN addition procedure might be triggered by each HANDOVER REQUEST from S-gNB/S-MN. If multiple candidate PCells under the same T-MN perform multiple CPA/CPC procedures towards the same T-SN, each CPA/CPC procedure is performed through SN addition procedure.

Observation 1: CHO+CPA/CPC configuration modification might be triggered by T-MN or T-SN, and new message might to be introduced for handover preparation modification triggered by candidate target nodes.

Proposal 3: The scenarios for CHO+CPA/CPC configuration modification and the new message for handover preparation modification triggered by candidate target nodes should be discussed.

Observation 2: UE’s behavior is different in different condition evaluation modes.

Proposal 4: The two modes of CHO+CPA/CPC execution condition evaluation are both feasible and should be discussed to specify UE’s behavior.

Observation 3: There are different scenarios for satisfaction of CHO/CAPC execution condition.

Proposal 5: UE’s behavior should be specified for the different scenarios for satisfaction of CHO/CAPC execution condition. And we think scenario0 and 2 might be the staring points.

Proposal 6: UE’s behavior in evaluation of CHO/CAPC execution condition and in scenarios for the satisfaction of CHO/CAPC execution condition should be studied. And UE’s behavior study in mode-1 with satisfaction scenario0/2, and mode-2 with satisfaction scenario0/2 can be the starting point.

Proposal 7: The method for early data forwarding should be discussed to reduce the redundant data transmission.

Based on proposal 1,2 and 3, we provide the text proposal to TS 37.340 in Annex.

Reference

[1] RP-213565 New WID on Further NR mobility enhancements

- 5 Annex – TP to TS 37.340

-------------------------------------------Change 1-------------------------------------------

- 10.7	Inter-Master Node handover with/without Secondary Node change

<unrelated part is omitted>

10.7.2	MR-DC with 5GC

Inter-MN handover with/without MN initiated SN change is used to transfer UE context data from a source MN to a target MN while the UE context at the SN is kept or moved to another SN. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN (or release the SN, as described in clause 10.8). Only intra-RAT Inter-Master node handover with/without SN change is supported (e.g. no transition from NGEN-DC to NR-DC).

<unrelated part is omitted>

Conditional Handover with Conditional Secondary Node Addition

Figure 10.7.x-x shows the Conditional Handover with Conditional SN Addition procedure.

Figure 10.7.X-X: Conditional Secondary Node Addition procedure

1. The source gNB requests CHO for one or more candidate cells belonging to one or more candidate MN. A CHO request message is sent for each candidate cell, including the indication of CPA/CPC preparation and the upper limit for the number of PSCells that can be prepared by the candidate SN.

2. The candidate MN might perform to configure CPA for the UE when it receives the CHO request message. The candidate MN requests the candidate SN(s) to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info), indicating that the request is for CPA and providing the upper limit for the number of PSCells that can be prepared by the candidate SN. In addition, for bearers requiring SCG radio resources, the MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the candidate MN also provides the candidate cells recommended by candidate MN via the latest measurement results for the candidate SN to choose and configure the SCG cell(s). The candidate MN may request the candidate SN to allocate radio resources for split SRB operation. In NR-DC, the candidate MN always provides all the needed security information to the candidate SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision.

For candidate MN terminated bearer options that require Xn-U resources between the candidate MN and the candidate SN, the candidate MN provides Xn-U UL TNL address information. For SN terminated bearers, the candidate MN provides a list of available DRB IDs. The candidate SN shall store this information and use it when establishing SN terminated bearers. The candidate SN may reject the addition request.

For SN terminated bearer options that require Xn-U resources between the candidate MN and the candidate SN, the candidate MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the candidate SN decides how to map QoS flows to DRB.

NOTE: T-MN might receive multiple CHO request message for different candidate PCells of T-MN. If T-MN triggers CPA/CPC procedure, T-MN might perform multiple CPA/CPC procedures to the same candidate SNs for different candidate PCells, and each CPA/CPC procedure is performed through SN addition procedure.

3. If the RRM entity in the candidate SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources, and provides the prepared PSCell ID(s) to the candidate MN. For bearers requiring SCG radio resources the candidate SN configures Random Access so that synchronisation of the SN radio resource configuration can be performed at the CPA execution. Within the list of cells as indicated within the measurement results indicated by the candidate MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the candidate MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the candidate MN in an NR RRCReconfiguration** message, contained in the SN Addition Request Acknowledge message. The candidate SN can either accept or reject each of the candidate cells listed within the measurement results indicated by the candidate MN, i.e. it cannot configure any alternative candidates. In case of bearer options that require Xn-U resources between the candidate MN and the candidate SN, the candidate SN provides Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the candidate SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

4. The candidate gNB(s) sends CHO response (HO REQUEST ACKNOWLEDGE) including configuration of CHO candidate cell(s) to the source gNB. The CHO response message is sent for each candidate cell, including multiple MCG and SCG configurations for candidate PCell and candidate PSCells.

5. The MN sends to the UE an RRCReconfiguration message including the CHO+CPA/CPC configuration, i.e. multiple MCG and SCG configurations and associated CHO execution conditions and CPA/CPC execution conditions.

6. The UE applies the RRCReconfiguration message received in step 5, stores the CHO+CPA/CPC configuration and replies to the source gNB/MN with an RRCReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

6a. If early data forwarding is applied, the source gNB/MN sends the EARLY STATUS TRANSFER message.

If UE evaluates CHO condition and CPA/CPC condition at the same time, step7a-7b will be performed. Else if UE first evaluates CHO condition and access target PCell, then evaluates CPA/CPC condition for configured CPA/CPC candidate PSCells, step8a-9d will be performed.

7a-7b. The UE synchronizes to the target MN and replies with MN RRC reconfiguration complete message including the SN RRC reconfiguration complete message.

7c-7e. The target MN informs the target SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message. The target MN sends the SN Release Request message(s) to cancel CPA in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

8a-8b, The UE synchronizes to the target MN and replies with MN RRC reconfiguration complete message without the SN RRC reconfiguration complete message.

9a. The UE replies to the target MN with MN RRC reconfiguration complete message, including an SN RRC response message for SN.

9b-9c. The target MN informs the target SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message. The target MN sends the SN Release Request message(s) to cancel CPA in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

10. The UE performs synchronisation towards the PSCell indicated in the RRCReconfiguration* message applied in step 9a. The order the UE sends the MN RRCReconfigurationComplete* message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

11a-11b. The target MN sends the HANDOVER SUCCESS message to the source gNB/MN to inform that the UE has successfully accessed the target cell. In return, the source gNB/MN sends the SN STATUS TRANSFER message.

12. If applicable, data forwarding takes place from the source side.

13a. The source gNB/MN sends the HANDOVER CANCEL message toward the other signalling connections or other candidate target gNBs, if any, to cancel CHO for the UE.

13b-13c. The candidate MN sends the SN Release Request message(s) to cancel CPA in the other candidate SN(s), if configured. The candidate SN(s) acknowledges the release request.

14-17.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

18. The target MN initiates the UE Context Release procedure towards the source gNB/MN.