3GPP TSG-RAN3 Meeting #117-e	R3-224837

Electronic meeting, 15 Aug – 24 Aug 2022

Agenda item:	8.1

Source: 	Samsung

Title: 	Discussion on RAN visible QoE

Document for:	Discussion & decision

# Introduction

Based on the LS in R3-224215, RAN2 has discussed RAN visible QoE and made the following agreement with regards to buffer level reporting:

- Application layer shall report the latest values of the buffer level to the AS layer.

However, on other aspects RAN2 could not reach consensus and asked RAN the below questions:

Based on the discussions above, RAN2 has the following questions to RAN3 and SA4: Question 1: Is a periodicity specific for buffer level measurement necessary for RVQoE? If yes, what is the motivation and what should be the configurable values? If not, what are the assumptions on how often the application layer performs the measurements of buffer level and how the buffer level list is filled? Question 2: Should the PDU session ID(s) be provided for each RAN visible QoE report and should it be mandatory or optional in the signaling? Question 3: What is the motivation for specifying that RAN visible QoE reports should be sent together with the legacy QoE reports? Is the requirement that RAN visible QoE reports should be sent together with the legacy QoE reports intended for the application layer or AS layer? If for AS layer, could the reporting periodicity for RAN visible QoE reports be considered mandatory because AS layer is not aware of when the legacy QoE reports will be triggered? RAN2 respectfully asks RAN3 to provide feedback to the questions 1-3 above.

In this paper, we would like to discuss on the above questions raised by RAN2 and provide proposals accordingly.

# Discussion

## 2.1 Buffer level measurements

RAN2’s discussion about buffer level measurement is as following.

RAN2 has discussed the request from SA4 in the feedback received in the reply LS S4-220239. - Assumption 1a: RAN2 specifies the maximum number of buffer level entries (ASN.1 value) for each buffer level metric report in one reporting message. [Feedback]: SA4 agrees with the above expectation on Application layer reporting behavior and AS layer can limit the maximum number in one reporting message. However, SA4 notes that RAN2 also need to specify how often buffer level measurements shall be done. SA4 asks RAN2 to specify how often buffer level measurements shall be done. RAN2 has specified a list of buffer level entries and reporting periodicity for RAN visible QoE, but not a periodicity specific for buffer level measurements.

As we discussed and the below agreement has been reflected in Release 17 TS38.300 21.4 RAN Visible QoE measurements [1].

RAN visible QoE measurements are configured by the gNB, where a subset of QoE metrics is reported from the UE as an explicit IE readable by the gNB. The set of available RAN visible QoE metrics is a subset of the metrics which are already configured as part of QoE measurement configuration encapsulated in the transparent container.

That’s to say, RV QoE is just the subset of the legacy QoE. It's no additional QoE measurement requirements from RAN to application layer to perform for RAN visible QoE.

As for the buffer level values, the list of buffer level values and the periodic value are provided to the UE application layer to fill in the buffer list items.

In AppLayerMeasConfig message, the information element about RV QoE is as following:

…

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity               ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries           INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportInitialPlayOutDelay            BOOLEAN,

...

}

In MeasurementReportApplayer message, the inforamtion element about RV QoE is as following:

RAN-VisibleMeasurements-r17 ::=       SEQUENCE {

applicationLayerBufferLevelList-r17    SEQUENCE (SIZE (1..8)) OF ApplicationLayerBufferLevel                   OPTIONAL,

initialPlayoutDelay-r17                INTEGER (0..30000)                                                      OPTIONAL,

pdu-SessionIdList-r17                  SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionID           OPTIONAL,

...

}

field descriptions

ran-VisiblePeriodicity The field indicates the periodicity of RAN visible reporting. Value ms120 indicates 120 ms, value ms240 indicates 240 ms and so on. If no value is indicated and the UE is configured with RAN visible reporting, the same periodicity as indicated in the measConfigAppLayerContainer is used.

numberOfBufferLevelEntries The field contains the maximum number of buffer level entries that can be reported for RAN visible application layer measurements.

applicationLayerBufferLevel Indicates the application layer buffer level in ms. Value 1 corresponds to 10ms, value 2 corresponds to 20 ms and so on. If the buffer level is larger than the maximum value of 30000 (5 minutes), the UE reports 30000.

Observation 1: RAN configures the periodicity and the number of buffer level list entries to the UE application layer. The UE application layer is requested to fill the buffer level list with the buffer level values which have been measured and saved in the legacy QoE report within a RV QoE reporting period.

Let’s check how the buffer level is measured in legacy QoE report.

As specified in TS 26.247 [2], the reporting interval of DASH services for legacy QoE is configured on a second basis, which can be treated as a reference for reporting interval for legacy QoE,

|  | @reportinginterval | O | Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, @reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. For each report sent, only the newly collected information since the previous report shall be reported. |
| --- | --- | --- | --- |

10.2.6 Buffer Level

Annex D.4.5 in ISO/IEC 23009-1 [3] defines the metrics for buffer level status events.

D.4.5 Buffer level
Table D.4 defines the metric for buffer level status events. The key in Table D.4 shall be used to refer to the metric as defined in Table D.4.

The buffer level value is recoded in the Buffer level entry every n ms, where n is a positive integer and is assumed defined by OAM.

The buffer level value, which is in milliseconds, indicates the playout duration for which media data of all active media components is available starting from the current playout time. The current playout time should be the t in the entry, which is the time of the measurement of the buffer level in our understanding.

The more media data is buffered, the more the buffer level value will be. And the media play should be smooth, if other conditions are normal.

Observation 2: In the legacy QoE report, the buffer level value is recoded in the Buffer Level entry list every n ms, where n should be the parameter defined by OAM.

In the current R17 spec, RAN side has not any helpful information to determine which value of RV-QoE periodicity should be set. From the above description, it will be beneficial and reasonable when the RV-QoE periodicity value and the n value of buffer level value recoded in legacy QoE report are harmonized.

For example, if the RV-QoE periodicity value is set to 120ms, while the n value of buffer level value recoded in legacy QoE report is 500ms, there will be no meaningful buffer level values which could be filled in the buffer level list for RV-QoE.

Observation 3: It is beneficial for RAN to get the n value, which indicates how often in milliseconds the buffer level value is recorded in legacy QoE report.

Proposal 1, RAN3 takes into account the buffer level recorded interval time in legacy QoE report, which is helpful to determine the value of RV-QoE periodicity value.

Proposal 2, OAM is required to explicitly signal the integer n value, which indicates how often in milliseconds the buffer level value is recorded in legacy QoE report to NG-RAN, if the proposal 1 is agreed.

RAN2 Quetion1: Is a periodicity specific for buffer level measurement necessary for RVQoE? If yes, what is the motivation and what should be the configurable values? If not, what are the assumptions on how often the application layer performs the measurements of buffer level and how the buffer level list is filled?

RAN3 Answer: 
The buffer level values for RVQoE are extracted from the legacy QoE values which had been measured and recorded by the UE application layer. RAN side does not request the new measurement for RVQoE. The buffer level list for RVQoE should be filled in by the latest values in the buffer level list of the legacy QoE within the RV QoE reporting period.

It’s necessary for RVQoE to configure the periodicity because it is used for real time optimization if needed.

The integer n value, which indicates how often in milliseconds the buffer level value is recorded in legacy QoE report, should be known by RAN to determine or optimize the RV-QoE periodicity.

Proposal 3, RAN3 takes the above discussion into account and feedback RAN2 about question1.

## 2.2 Reporting of PDU session ID(s)

RAN2’s discussion about reporting PDU session ID is as following.

RAN2 specified that PDU session ID(s) corresponding to the service that is subject to QoE measurements can be reported by the UE along with the RAN visible QoE measurement results. According to current signaling the PDU session ID(s) are optional, but RAN2 was not certain whether from RAN3 point of view, PDU session ID(s) should be mandatory or optional in the RAN visible QoE report.

RAN2 Question2: Should the PDU session ID(s) be provided for each RAN visible QoE report and should it be mandatory or optional in the signaling?

In our understanding and discussed in [5], the identification ID of RAN visible QoE report can be optimized in Release 18. The PDU session ID is not the perfect choice for RAN-Visible QoE and real time optimization.

It’s better to keep the current status without any change about the PDU session ID status in R17 spec.

Proposal 4, The PDU session ID(s) may be provided for each RAN visible QoE report and it can keep the current status, which is optional in the signalling

## 2.3 Reporting of RAN visible QoE measurements

RAN2’s discussion about reporting RV-QoE measurement is as following.

Furthermore, based on the RAN3 stage 2 input to QoE the below highlighted requirement is specified in TS 38.300, sub-clause 21.4: RAN visible QoE measurements can be reported with a reporting periodicity different from the one of regular QoE measurements. If there is no reporting periodicity defined in the RAN visible QoE configuration, RAN visible QoE reports should be sent together with the legacy QoE reports. RAN2 noted that the RAN3 agreement was captured as: If the reporting periodicity of RVQoE is not explicitly indicated in the RVQoE configuration, RVQoE reports can be sent together with the legacy QoE reports.

RAN2 Quetion3: What is the motivation for specifying that RAN visible QoE reports should be sent together with the legacy QoE reports? Is the requirement that RAN visible QoE reports should be sent together with the legacy QoE reports intended for the application layer or AS layer? If for AS layer, could the reporting periodicity for RAN visible QoE reports be considered mandatory because AS layer is not aware of when the legacy QoE reports will be triggered?

In our understanding, the current spec has described how to use the reporting periodicity clearly. And it’s the agreement achieved by us in the previous meetings. We do not think it’s necessary to modify the current status.

Proposal 5, The reporting periodicity for RAN visible QoE reports could keep the current status, which is optional in the signalling.

# Conclusion

Base on the above discussion, the following are proposed

Observation 1: RAN configures the periodicity and the number of buffer level list entries to the UE application layer. The UE application layer is requested to fill the buffer level list with the buffer level values which have been measured and saved in the legacy QoE report within a RV QoE reporting period.

Observation 2: In the legacy QoE report, the buffer level value is recoded in the Buffer Level entry list every n ms, where n should be the parameter defined by OAM.

Observation 3: It is beneficial for RAN to get the n value, which indicates how often in milliseconds the buffer level value is recorded in legacy QoE report.

Proposal 1, RAN3 takes into account the buffer level recorded interval time in legacy QoE report, which is helpful to determine the value of RV-QoE periodicity value.

Proposal 2, OAM is required to explicitly signal the integer n value, which indicates how often in milliseconds the buffer level value is recorded in legacy QoE report to NG-RAN, if the proposal 1 is agreed.

Proposal 3, RAN3 takes the above discussion into account and feedback RAN2 about question1.

Proposal 4, The PDU session ID(s) may be provided for each RAN visible QoE report and it can keep the current status, which is optional in the signalling

Proposal 5, The reporting periodicity for RAN visible QoE reports can keep the current status, which is optional in the signalling.

The draft reply LS is provided in [4].

# References

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

- 3GPP TS 26.247: “Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)”.

- ISO/IEC 23009-1:2020/Amd. 1" Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats".

- R3-22XXXX R3-22XXXX [Draft]Reply LS on RVQoE, Samsung

- R3-22XXXX NR QoE Discussion on Left-over from R17, Samsung