3GPP TSG-RAN3 Meeting #117-e	R3-224840

Electronic meeting, 15 Aug – 24 Aug 2022

Agenda item:	11.2

Source: 	Samsung

Title: 	NR QoE Discussion on support for new service type and RRC inactive-idle states

Document for:	Discussion & Agreement

# Introduction

In RAN #96 meeting, the updated WID [1] for enhancement on NR QoE was agreed. The objectives for support for new service type and RRC_INACTIVE/RRC_IDLE states are listed as following.

The detailed objectives of the work item are as follows: - Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2]. - Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. - Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting. …

In this paper, we would like to discuss some topics about them.

# Discussion

## 2.1 New Service Type

In R18, QoE framework will support the new service type, such as AR, MR, MBS and other new service types defined or to be supported by SA4. While waiting for the progress on SA4, the current signalling design could be checked whether it can support the new service type by RAN3. If no additional requirement for some specific new service type is identified, we can keep the current status.

Observation1, the legacy QoE configuration and report may be supported in the current signalling design, by extending the service type value simply, if no additional requirement is identified.

Proposal1, the Legacy QoE enhancement for new service type is pending on SA4 progress.

As for the RAN Visible QoE, the current spec only supports the DASH streaming and VR service types, and only two items are defined in the RAN visible QoE metrics. One is the application layer buffer level list, the other one is the playout delay for media startup. In our understanding, the playout delay for media startup only be recorded once when the media start to play at the first time. The buffer level list could be filled in by the UE application layer and reported to RAN side periodically. The RAN side could know the QoE tendency from the changing buffer level values after real-time optimization for the UE. But RAN has the only one parameter to check the optimization result. Maybe it’s not enough for the new service types which will be supported in R18.

Proposal2, the new RAN visible parameters should be analysed and introduced for the additional service types or the existing service if needed.

## 2.2 High mobility scenarios

From our point of view, the high mobility scenario is the one of important scenarios for operator and the related QoE report could be utilized for network optimization. The current NR spec has supported the high speed up to 500km per hour in FR1 and 350km per hour in FR2. The typical scenario is on the high speed trains. And the high mobility scenario should be supported for all the service types supported by QoE.

We think the network side could get the QoE report for high mobility scenarios though the below solutions:

Option1: The MCE/TCE collects the QoE/radio-related measurement results respectively. And then the post-processing for the “big data” could be done to find any problem for high mobility scenarios by any potential AI algorithm. As we know, the radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE.

Option2: OAM/CN requests RAN side to select the UE which is in the high mobility scenarios and report the QoE measurement results explicitly. The radio-related measurement may be collected for the same UE and reported to TCE.

For the legacy QoE, which option is preferred is out of RAN3 work scope. Maybe we should ask for the related working group’s opinion?

If option2 is used, the signal based QoE procedure should be enhanced. An explicit IE about the high speed mobility should be added in the UE Application Layer Measurement Configuration Information IE on NGAP and XnAP. The option2 is our preferred.

Proposal3, OAM/CN requests RAN side to configure the legacy QoE report for high speed mobility scenarios explicitly.

As for the RAN visible QoE, the below two solutions from RAN side could be used to collect QoE report for high mobility scenarios. Actually they could be used for collecting the legacy QoE also.

Option1: Define the dedicated QMC for high mobility scenario in the specific area scope.

For management based QoE solution, OAM only sends the QoE measurement configuration to the NG-RAN nodes that provide the high mobility coverage.

For signalling based QoE solution, Use the IE CHOICE Area Scope of QMC to specify the area scope.

In the current CHOICE Area Scope of QMC IE, the cell based, TA based and TAI based area scope are defined. The maximum number of cell IDs comprising the QMC scope is 32. The maximum number of TA comprising the QMC scope is 8.

Considering the high speed trains travel long distances, it maybe not enough for the current scope to cover such a long distance. Even the maximum number of cell IDs or TA could be enlarged to cover more area, it’s still not a good choice to provide such a long list in the signalling interface. During the mobility procedure, the target RAN node has to check the area scope list. If it’s too long, the target RAN node will take more time to check. That’s one of the reasons why it’s not a good way we think.

Option2: Configure the QoE report only when UE’s speed reaches the specific threshold.

Collecting QoE measurement only when the UE is in the high speed state will be very helpful for operator to optimize the special scenarios. It’s a better solution for QoE collection in high mobility scenarios, while more efforts and more spec impacts are needed.

In our views, event-based triggering helps operators to collect QoE information form UE when UE is in some special scenarios, such as high-speed scenarios, bad coverage scenarios and high interference scenarios. And those special scenarios are experienced by UE itself, so the condition check should be performed by UE according to the triggering conditions provided in QoE measurement configuration from the network.

For high-speed scenarios, the trigger condition could be the cell change number and the evaluation time.

And another observation is here: in TS38.331, the CommonLoationInfo IE could be reported by UE to NW.

- CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

It seems that some special UEs have the ability to know the velocity. Maybe it could be used to trigger the QoE report for high speed scenarios.

Proposal4, the triggering conditions could be included in the RVQoE configuration for high mobility scenarios.

## 2.3 QoE measurement in RRC_INACTIVE and RRC_IDLE state

In Release 18, the QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS should be supported. In the first meeting of R18, we just provide some thoughts about the main procedure instead of too detail information.

For MBS service, when UE in RRC_CONNECTED mode, the QoE configuration and collection could be done also. If OAM wants to collect the QoE report in RRC_INACTIVE or RRC_IDLE states for MBS, the requirements should be indicated in the QMC

Observation2, The UE states, which includes RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE, should be included in the QMC configuration.

Proposal5, An explicit IE to indicate the UE states should be added in the IE UE Application Layer Measurement Configuration Information on XnAP and NGAP interface.

The gNB can select the UE which in RRC_INACTIVE or RRC_IDLE state to perform the QoE collection.

For UE in RRC_INACTIVE state, there two kinds of alternatives to configure the RAN-based Notification Area (RNA).

-	List of cells:

-	A UE is provided an explicit list of cells (one or more) that constitute the RNA.

-	List of RAN areas:

-	A UE is provided (at least one) RAN area ID, where a RAN area is a subset of a CN Tracking Area or equal to a CN Tracking Area. A RAN area is specified by one RAN area ID, which consists of a TAC and optionally a RAN area Code;

In order to indicate the RNA for RRC_INACTIVE status, the RAN Area ID list could be added in the QMC Area scope as following.

| CHOICE Area Scope of QMC | M |  |  |  |
| --- | --- | --- | --- | --- |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1..<maxnoofCellIDforQMC> |  |  |
| >>>NG-RAN CGI | M |  | 9.3.1.73 | This IE can only indicate the NR CGI. |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1..<maxnoofTAforQMC> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >PLMN area based |  |  |  |  |
| >>PLMN List for QMC |  | 1..<maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |
| >RAN Area Based |  |  |  |  |
| >>RAN Area ID List for QMC |  | 1 .. <maxnoofRanAreasinRNA> |  |  |
| >>>RAN Area ID | M |  | 9.2.3.39 |  |

Proposal6, the RAN area ID list for QMC should be added in the IE Choice Area Scope of QMC on XnAP and NGAP interface.

Just like logged MDT, the similar “logged QoE report” should be used for QoE report collection when UE is in RRC_INACTIVE and RRC_IDLE states. And the MCE id should be introduced in the “logged QoE report” to indicate which MCE should receive the configured QoE report.

Proposal7, the MCE id should be introduced into QoE configuration and report messages when UE is in RRC_INACTIVE or RRC_IDLE state.

# Conclusion

In this paper, we discussed the NR QoE about new service types, high mobility scenarios, RRC_Inactive and RRC_IDLE states. The following are observations and proposals:

Observation1, the legacy QoE configuration and report may be supported in the current signalling design, by extending the service type value simply, if no additional requirement is identified.

Proposal1, the Legacy QoE enhancement for new service type is pending on SA4 progress.

Proposal2, the new RAN visible parameters should be analysed and introduced for the additional service types or the existing service if needed.

Proposal3, OAM/CN requests RAN side to configure the legacy QoE report for high speed mobility scenarios explicitly.

Proposal4, the triggering conditions could be included in the RVQoE configuration for high mobility scenarios.

Observation2, The UE states, which includes RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE, should be included in the QMC configuration.

Proposal5, An explicit IE to indicate the UE states should be added in the IE UE Application Layer Measurement Configuration Information on XnAP and NGAP interface.

Proposal6, the RAN area ID list for QMC should be added in the IE Choice Area Scope of QMC on XnAP and NGAP interface.

Proposal7, the MCE id should be introduced into QoE configuration and report messages when UE is in RRC_INACTIVE or RRC_IDLE state.

# References

- RP-221803 WID update for Enhancement on NR QoE

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".