3GPP TSG RAN3 meeting #117-e			R3-224864

15th Aug – 24 Aug 2022 Online

Agenda Item:	11.2

Source:	China Unicom

Title:	Discussion on QoE measurement in RRC_INACTIVE and RRC_IDLE states

Document for:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

This paper discusses the QoE measurement configuration and reporting in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service.

Discussion

QoE configuration for MBS service

QoE configuration for MBS service is important for the optimization of MBS service. MBS is generic name of the Multicast/Broadcast service, but the configuration for MBS service is divided by TMGI like bellow, and the radio  configuration is configured by different TMGI. TMGI is an indication of a MBS session. For QoE reporting analysis and network optimization for MBS service, whether need to distinguish the QoE per-session need to be discussed, and need to coordinate with SA4.

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (0..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17                        OPTIONAL,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCell-MBS-r17))      OPTIONAL, -- Need R

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Need R

}

Proposal 1: For MBS service, RAN3 should discuss whether QoE reporting should be reported per-session, and send LS to SA4 to confirm whether application layer can collect and report QoE per MBS session.

QoE configuration in RRC_INACTIVE and RRC_IDLE states

M-based QoE configuration in RRC_INACTIVE and RRC_IDLE states

In Rel-17, QoE configuration is send only when UE in RRC_CONNECTED state, but for some service type, such as MBS, it will be proceeded even when UE in RRC_INACTIVE and RRC_IDLE state. For m-based QoE configuration, it is for all the UEs in certain area scope, it is no need to wait for all the UEs in the area scope turn to CONNECTED state, and then send the QoE configuration. It is also no need to setup the connection only for sending QoE configuration, it will cause signaling overhead.

Since MBS has its own SIB message MCCH message, management-based QoE configuration for MBS can be included in the SIB or MCCH configuration for MBS, and it can reduce the signaling overhead, and it is easy to send the QoE configuration for all UEs in RRC_IDLE and RRC_INACTIVE state.

Proposal 2: RAN3 should discuss whether SIB message or MCCH message can be used to configure m-based QoE configuration for MBS service.

S-based QoE configuration for MBS service in RRC_CONNECTED state

MBS service can be received in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE state, for UE in RRC_CONNECTED state, the QoE configuration can use the same way as other service type. In RRC_CONNECTED state, UE will use MBSInterestIndication message to inform the network that the UE in RRC_CONNECTED state is receiving or is interested to receive MBS broadcast service(s), and the network can send the QoE configuration for MBS service if the UE is receiving the QoE configuration.

Proposal 3: UE use the QoE measurement configuration message defined in R17 for S-based QoE configuration for MBS service.

If the UE is turn to RRC_INACTIVE or RRC_IDLE state from RRC_CONNECTED state, the MBS service will continue, and the QoE measurement will also need to continue. But in current specification, the QoE configuration will be released when the UE release, so how to deal with the QoE configuration during RRC release procedure need to be further studied.

Option1: All the QoE measurements for MBS service need to be keep when UE is turn to RRC_INACTIVE or RRC_IDLE state from RRC_CONNECTED state;

Option2: Network should specify which QoE measurements should be keep when UE is turn to RRC_INACTIVE or RRC_IDLE state from RRC_CONNECTED state;

Proposal 4: When UE turn to RRC INACTIVE or RRC_IDLE state from RRC_CONNECTED state, QoE measurements for MBS service need to keep, how to keep the QoE measurements should be discussed.

S-based QoE configuration for MBS service in RRC_INACTIVE and IDLE state

In RRC_CONNECTED state, if the gNB receives MBSInterestIndication message and configure the QoE configuration, the QoE measurement can start, and can continue to measure even when UE turn to RRC_INACTIVE and RRC_IDLE state. But if gNB not receive MBSInterestIndication message in RRC_CONNECTED state, gNB will not know whether it need to configure the QoE configuration to UE if the QoE configuration is received from CN. Since the UE may stay in RRC_INACTIVE and RRC_IDLE states and start to receive the MBS service, how to configure the QoE configuration for MBS when the UE turn to RRC_INACTIVE and RRC_IDLE state need to be discussed.

Another case is when the gNB receive the QoE configuration from OAM/CN when the UE connection is about to release, it need to discuss whether QoE configuration can be send during RRC release procedure.

Proposal 5: How to configure the s-based QoE configuration for MBS service when UE didn’t receive MBS service in RRC_CONNECTED state, and how to configure s-based QoE when UE is about to release the RRC connection should be discussed.

QoE reporting in RRC_INACTIVE and RRC_IDLE states

In Rel-17, QoE reporting can only send in RRC_CONNECTED state. If the UE is start or continue the QoE measurements in RRC_INACTIVE and RRC_IDLE state, the QoE report need to send to the network when UE turn to RRC_CONNECTED state.

Proposal 6: QoE measurement reporting in RRC_INACTIVE and RRC_IDLE states need to be send to gNB when UE turn to RRC_CONNECTED state.

When UE turn to RRC_CONECTED state, whether QoE report will be send to gNB directly or upon the request of gNB need to be discussed. Since the QoE report in RRC_INACTIVE and RRC_IDLE may be very large, it is better for gNB to decide when to acquire the QoE report.

Proposal 7: It is better for gNB to acquire the QoE measurement report in RRC_INACTIVE and RRC_IDLE states when UE turn to RRC_CONNECTED state.

QoE continuity during cell re-selection

When UE in RRC_INACTIVE or RRC_IDLE states, it will re-select to other cells if cell re-selection condition fulfilled. If QoE measurements is ongoing when UE in RRC_INACTIVE and RRC_IDLE states, the measurement scope of the QoE should be checked by the target cell, but gNB is not aware of the cell re-selection of UE in RRC_INACTIVE and RRC_IDLE states, so the area scope should be checked by UE itself.

Proposal 8: For MBS service, QoE area scope information for QoE configuration should be send as part of QoE configuration from gNB to UE.

Conclusion

Proposal 1: For MBS service, RAN3 should discuss whether QoE reporting should be reported per-session, and send LS to SA4 to confirm whether application layer can collect and report QoE per MBS session.

Proposal 2: RAN3 should discuss whether SIB message or MCCH message can be used to configure m-based QoE configuration for MBS service.

Proposal 3: UE continues to use the QoE measurement configuration message defined in R17 for S-based QoE configuration for MBS service.

Proposal 4: When UE turn to RRC INACTIVE or RRC_IDLE state from RRC_CONNECTED state, QoE measurements for MBS service need to keep, how to keep the QoE measurements should be discussed.

Proposal 5: How to configure the s-based QoE configuration for MBS service when UE didn’t receive MBS service in RRC_CONNECTED state, and how to configure s-based QoE when UE is about to release the RRC connection should be discussed.

Proposal 6: QoE measurement reporting in RRC_INACTIVE and RRC_IDLE states need to be send to gNB when UE turn to RRC_CONNECTED state.

Proposal 7: It is better for gNB to acquire the QoE measurement report in RRC_INACTIVE and RRC_IDLE states when UE turn to RRC_CONNECTED state.

Proposal 8: For MBS service, QoE area scope information for QoE configuration should be send as part of QoE configuration from gNB to UE.