3GPP TSG-RAN WG3 Meeting #117-e	R3-224888

E-meeting, 15 – 24 August 2022

Title: 	Initial discussions on the support of QoE measurement in RRC_INACTIVE and RRC_IDLE states for MBS service

Source: 	Huawei

Agenda item:	11.2

Document Type:	discussion

1. Introduction

One new WID RP-221803 [1] on R18 QoE has been agreed. This new WID includes the following objective.

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

In this paper we provide our views on this objective.

2. Discussion

In R17, RAN3 and RAN2 specified the support of MBS broadcast service. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

As discussed in our contribution [2], the MBS QoE measurement depends on the progress of SA4. But from RAN3 and RAN2 perspectives, the specification of SA4 only has impacts on the containers of QoE configuration and reporting. Therefore we can first assume that the SA4 supports the MBS QoE measurement.

Observation 1: SA4 specification impacts are mainly on the containers of QoE configuration and reporting.

For the MBS QoE measurement that is configured and started in RRC_CONNECTED, we can reuse the R17 QoE procedure. RAN3 and RAN2 only need to define new service type. In the rest of the paper, we focus the discussions on the QoE measurement for MBS service under RRC_IDLE/RRC_INACTIVE state.

2.1 MBS QoE procedure for RRC_IDLE/RRC_INACTIVE UE

In R17 QoE, RAN3 and RAN2 only specified the QoE measurement for streaming, MTSI and VR services for RRC_CONNECTED UE. In R17 QoE, for the signaling based QoE measurement, the CN sends the QoE measurement configuration information to the gNB via the UE-associated NG signaling for a given RRC_CONNECTED UE. For both the signaling based and management based QoE measurement, the gNB sends the QoE measurement configuration to the RRC_CONNECTED UE via the RRC reconfiguration message, the RRC_CONNECTED UE then reports the QoE measurement results to the gNB via the MeasurementReportAppLayer message.

For the RRC_IDLE/INACTIVE UE, the RRC connection between UE and gNB is released. Therefore RAN3 and RAN2 need to design one new procedure for the MBS QoE in RRC_IDLE/RRC_INACTIVE.

In our understanding, the motivations of signalling based and management based QoE are different. The signalling based QoE is triggered for a specific UE, the purpose could be, for example to collect information and status of an ongoing service for a subscriber who complained, in order to find out the reason for the complaint. The management based QoE is not triggered for a specific UE but for UEs in a specified area, the purpose could be, for example, testing a new feature. Therefore, we think both of both the signalling based and management based QoE measurement should be supported.

Support both the signalling based and management based QoE measurement for MBS service in RRC_IDLE/RRC_INACTIVE state.

In R17 MBS, the MBS session ID is used to identify the MBS service. The gNB receives the MBS session ID from the CN and broadcast the MBS session ID in Uu. In R17 QoE, RAN3 and RAN2 supports the multiple QoE measurement for the same service type. In our understanding, some operators may only want to know the QoE results for the specific broadcast services. Therefore we think the MBS session based QoE measurement is needed.

In R17 MBS, the CN knows the MBS session that supported by the gNB. Also we think the OAM knows the MBS session that supported by the gNB. Therefore we think the gNB does not need to know the target MBS session of the QoE configuration. The MBS session ID only need to be included in the QoE measurement configuration container and the QoE reporting container.

Include the MBS session ID for the broadcast in the QoE measurement configuration container and QoE measurement reporting container. SA4 should be informed if consensus to include is reached.

After receiving the QoE measurement configuration for the broadcast service, the gNB sends the QoE measurement configuration to the UE. For the signalling based broadcast QoE measurement, the gNB only needs to check the area scope. If the UE is in the area scope, the gNB directly sends the QoE measurement configuration to the UE. For the management based broadcast QoE measurement, the gNB needs to check the UE capability and area scope. Then the gNB sends the QoE measurement configuration to the UE. In our understanding, we think the gNB does not need to check which UEs is receiving or is interested in receiving a broadcast service, since UE knows everything, UE will decide the next step according to its MBS service reception status.

gNB doesn’t need to check if the UE is receiving or is interested in receiving a broadcast service upon the reception of QoE request from OAM/CN.

In our understanding, it is better for the gNB sends the QoE measurement configuration to the UE when the UE is in RRC_CONNECTED via the RRCReconfiguration message as in R17. Otherwise, RAN2 needs to define the system information for the QoE measurement. It will increase the specification works.

Reuse the R17 mechanism to configure  the QoE measurement for MBS service.

Upon reception of QoE measurement configuration for MBS service, the UE starts the QoE measurement as long as there is MBS service ongoing. Then, the next issue is about the procedure of QoE reporting.

In R17 QoE, the gNB configures the SRB4 for the QoE reporting in RRC_CONNECTED. For the UE in RRC_IDLE/RRC_INACTIVE state, there is no SRB between the UE and the gNB, which is actually the same as the issue of logged MDT in RRC_IDLE/RRC_INACTIVE state. In the logged MDT, the RRC_IDLE/RRC_INACTIVE UE stores the logged MDT results, and when the UE enters the RRC_CONNECTED state with SRB2 being setup, the UE reports the stored logged MDT results to the gNB. In addition, the UE will not trigger the RRC connection setup request just due to the logged MDT reporting, in order to reduce the radio load. The UE only reports the logged MDT to the gNB only when the UE has entered to the RRC_CONNECTED due to other reasons, e.g. the normal service request. In our understanding, the reporting of broadcast QoE results can use the same principle.

Take the logged MDT reporting mechanism as base line, i.e.:

- RRC_IDLE/RRC_INACTIVE UE stores the QoE measurement results for MBS service

- UEs reports the stored QoE measurement results for MBS service after entering the RRC_CONNECTED state

- The UE only reports the QoE measurement results for MBS service to the gNB only when the UE has entered to the RRC_CONNECTED due to other reasons, i.e. the UE will not trigger the RRC connection step request procedure just due to the availability of QoE measurement results for MBS service

In our understanding, it is RAN2 to decide how the UE stores the QoE measurement results and how the UE informs the gNB of the availability of QoE measurement results.

In R17 QoE, the QoE measurement configuration includes the QoE reference ID and MCE IP address. Different QoE measurement may have different MCE IP address. In order to reduce the signalling overhead in Uu, the gNB allocates and stores the measConfigAppLayerId for each configured QoE measurement and sends this measConfigAppLayerId together with the QoE measurement configuration to the UE. If the UE handover to another gNB, the source gNB sends the mapping relation between the QoE reference ID and the measConfigAppLayerId to the target gNB. When the UE reports the QoE results together with the measConfigAppLayerId to the target gNB, the target gNB looks up the QoE reference ID and MCE IP address based on the mapping relation between the QoE reference ID and the measConfigAppLayerId, and then forwards the QoE results, QoE reference ID to the MCE.

Another issue is, since the serving gNB will release the UE context after the UE enters he RRC_IDLE state,  the gNB which receives the QoE results from the UE will not be able to identify received QoE measurement results, due to the release of the mapping relation between the QoE reference and the measConfigAppLayerId. RAN3 needs to discuss how the gNB identifies the QoE measurement results and forwards to correct MCE.

This issue also exists in the logged MDT.

- In order to avoid the leaking of TCE IP address, the logged MDT configuration sent to the UE includes one TCE ID instead of the TCE IP address. The OAM allocates the TCE ID for each logged MDT measurement and sends the mapping relation between TCE IP address and TCE ID to all the gNBs. When one gNB receives the logged MDT results, the gNB can know the target TCE IP address based on the TCE ID.

- In order to know the measurement job associated to the logged MDT results, the logged MDT configuration sent to the UE includes the trace reference. And, the logged MDT results from the UE also include the trace reference. The gNB forwards the logged MDT results to the TCE. Therefore the TCE can know the measurement job based on the trace reference included in the logged MDT results.

In our understanding, the simplest solution here is to reuse the same principle as logged MDT. It means the OAM allocates one MCE ID for each MCE IP address and sends the mapping relation to all the gNBs. For the QoE measurement configuration, the gNB sends the QoE reference ID and MCE ID together with the QoE measurement configuration container to the UE. For the QoE measurement results reporting, the UE reports the QoE reference ID and MCE ID together with QoE measurement reporting container to the new gNB.  This new gNB looks up the MCE IP address based on the MCE ID from the UE and the mapping relation between MCE ID and MCE IP address.

For QoE measurement for MBS service, the gNB includes the QoE reference ID and MCE ID in the configuration message to the UE, and the UE includes the QoE reference ID and MCE ID in the reporting message, as explicit IEs.

Actually this methods also applies to RRC_INACTIVE state, since target gNB can identify and forward the received report, the contexts retrieval procedure is not needed. In summary, we propose the following procedures.

Figure 1. General propocedure of QoE measurement for MBS service

2.2 Alignment of logged MDT measurement and QoE reporting

In R17 QoE, RAN3 discussed and specified the alignment of immediate MDT and QoE results.

## 21.5 Alignment of MDT and QoE Measurements Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The following is supported: - Alignment between a signalling-based QoE measurement and a signalling-based MDT measurement. In this case, the signalling-based QoE configuration sent to the NG-RAN node includes the NG-RAN Trace ID of the signalling-based MDT measurement. - Alignment between a management-based QoE measurement and a management-based MDT measurement. The UE configured for QoE measurements can send to the NG-RAN node a Session Start Indication or a Session End Indication to inform the NG-RAN node about the start or the end of a session of configured QoE measurements. The NG-RAN node can activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE upon/after receiving the Session Start Indication from the UE. The NG-RAN node may activate the MDT measurements upon/after receiving the MDT activation message from OAM. The NG-RAN node can deactivate the aligned MDT measurements according to OAM command which may, e.g., be triggered by the Session End Indication. The NG-RAN node includes time stamp information to the QoE reports to enable the correlation of corresponding measurement results of MDT and QoE at the MCE/TCE. In addition, the NG-RAN node includes the MDT session identifiers (Trace Reference and Trace Recording Session Reference) to the corresponding QoE report.

According to the objective of R18 QoE WID, RAN3 also needs to discuss the alignment of QoE measurement and radio measurement for MBS service.

In R16, NR has specified the logged MDT to collect the radio measurement result for the RRC_IDLE/RRC_INACTIVE UE. The logged MDT also includes the signaling based logged MDT and management based logged MDT. In R17 QoE, the QoE measurement request from the CN/OAM includes the alignment requirement including the MDT trace ID. In our understanding, the same principles can be used.

Support the alignment between a signalling-based QoE measurement for MBS service and a signalling-based logged MDT measurement, and the alignment between a management-based QoE measurement for MBS service and a management-based logged MDT measurement.

The request of QoE measurement for MBS service from CN/OAM includes the alignment requirement.

In R17 QoE, the MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. In our understanding, the QoE measurement for MBS service can also use the same principle. It is the MCE/TCE performs the correlation.

The MCE/TCE performs the correlation of the logged MDT results and the QoE measurement results for MBS service collected at the same UE.

In R17 QoE, the gNB includes time stamp information to the QoE reports and includes time stamp information to the immediate MDT results to enable the correlation of corresponding measurement results of MDT and QoE at the MCE/TCE.

In R16 logged MDT, the UE includes the time stamp information to the logged MDT results. Also according to above discussion of broadcast QoE reporting, the UE reports the QoE results only after entering the RRC_CONNECTED. Therefore it is not suitable for the gNB to include the time stamp information to the QoE reports. We suggest the UE include the time stamp information to the QoE reports.

UE includes the time stamp information in the QoE measurement reports for MBS service.

In R17 QoE, the gNB includes the MDT session identifiers (Trace Reference and Trace Recording Session Reference) to the corresponding QoE report. For the QoE measurement for MBS service, the new gNB does not have the context of QoE measurement. The issue is how the new gNB knows whether the alignment is needed or not. Therefore we suggest the QoE measurement configuration sent to the UE includes the alignment indication and the UE reports the alignment indication to the new gNB. Then the new gNB knows the alignment is needed and includes MDT session identifier to the corresponding QoE reporting.

The QoE measurement configuration for MBS service sent to the UE includes the alignment indication. After entering the RRC_CONNECTED, the UE sends the alignment indication to the new gNB.

- In summary, we propose the following procedures.

Figure 2 General propocedure for the alignment of QoE measurement for MBS service and logged MDT

3. Proposal

In this contribution, we provide the considerations on QoE measurement in RRC_IDLE/RRC_INACTIVE, and get the following proposal:

Observation 1: SA4 specification impacts are mainly on the containers of QoE configuration and reporting.

- Support both the signalling based and management based QoE measurement for MBS service in RRC_IDLE/RRC_INACTIVE state.

- Include the MBS session ID for the broadcast in the QoE measurement configuration container and QoE measurement reporting container. SA4 should be informed if consensus to include is reached.

- gNB doesn’t need to check if the UE is receiving or is interested in receiving a broadcast service upon the reception of QoE request from OAM/CN.

Reuse the R17 mechanism to configure the QoE measurement for MBS service.

Take the logged MDT reporting mechanism as base line, i.e.:

- RRC_IDLE/RRC_INACTIVE UE stores the QoE measurement results for MBS service

- UEs reports the stored QoE measurement results for MBS service after entering the RRC_CONNECTED state

- The UE only reports the QoE measurement results for MBS service to the gNB only when the UE has entered to the RRC_CONNECTED due to other reasons, i.e. the UE will not trigger the RRC connection step request procedure just due to the availability of QoE measurement results for MBS service

For QoE measurement for MBS service, the gNB includes the QoE reference ID and MCE ID in the configuration message to the UE, and the UE includes the QoE reference ID and MCE ID in the reporting message, as explicit IEs.

Support the alignment between a signalling-based QoE measurement for MBS service and a signalling-based logged MDT measurement, and the alignment between a management-based QoE measurement for MBS service and a management-based logged MDT measurement.

The request of QoE measurement for MBS service from CN/OAM includes the alignment requirement.

The MCE/TCE performs the correlation of the logged MDT results and the QoE measurement results for MBS service collected at the same UE.

UE includes the time stamp information in the QoE measurement reports for MBS service.

The QoE measurement configuration for MBS service sent to the UE includes the alignment indication. After entering the RRC_CONNECTED, the UE sends the alignment indication to the new gNB.

4. References

- [1] RP-221803, WI on Enhancement on NR QoE management and optimizations for diverse services

- [2] R3-224886, General considerations on R18 QoE, Huawei