3GPP TSG-RAN WG3 Meeting #117-e	R3-224889

E-meeting, 15 – 24 August 2022

Title: 	Discussion on the support for QoE in NR-DC

Source: 	Huawei

Agenda item:	11.3

Document Type:	discussion

1. Introduction

One new WID RP-221803 [1] on R18 QoE has been agreed. This new WID includes the following objective.

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2]. - Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity. Note 1: The QoE measurements are not performed separately for each leg. - Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios. - Specify the QoE measurement continuity in mobility scenarios in NR-DC. - Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

In this paper we provide our views on this objective.

2. Discussion

2.1 QoE measurement over SN

The motivation of QoE measurement is to collect the application layer measurement results of the services when these services are transmitted over the network. The operators can know the user experience and optimize the transmission based on these QoE results. In R17 QoE, only the MN can configure the QoE measurement and receive the QoE results.

In NR-DC, there are two simultaneous connections, one to MN and the other with SN. The services could be scheduled over the SN.

Observation 1: In NR-DC, the services could be scheduled over the SN.

R17 QoE supports the signaling based QoE and management based QoE. The signalling based QoE is triggered towards a specific UE from CN. The management based QoE is not triggered for a specific UE but for UEs in a specified area from OAM.

In NR-DC, only the MN is connected to the control plane of the CN. The CN can only send the signalling based QoE measurement configuration to the MN. The signalling based QoE measurement is used to collect the QoE results in the application layer for a specific UE. Upon the reception of a signalling based QoE measurement request, the MN just directly sends the signalling based QoE measurement configuration to the UE. Also, we think the QoE measurement configuration will not increase the downlink overload. It seems the MN does not need to send the signalling based QoE measurement configuration to the SN to request the SN to send the signalling based QoE measurement to the UE.

For NR-DC case, the MN sends the signalling based QoE measurement configuration to the UE.

For the management based QoE, we propose to support the QoE measurement configured by the SN based on the following reasons:

- In general, the coverage of the SN is smaller than the coverage of the MN. The user experience of services are different when they are transmitted in the MN or in the SN. The operators may want to know the user experience for the services that transmitted over the SN. Therefore we think the OAM also can send the management based QoE measurement to the SN.

- In additional, the SN also want to optimize the scheduling based on the RAN visible QoE results. And the RAN visible QoE depends on the QoE configuration container. In the same cases, the MN does not configure the QoE measurement. Then the SN needs to configure the management based QoE measurement for the RAN visible QoE.

RAN3 to discuss and agree to support the SN to configure the management based QoE measurement

We think there are two options for the SN to configure the QoE measurement.

Option 1: SN generates the SN RRC message including the QoE measurement configuration and sends the SN RRC message to the UE via SRB3 or via the MN SRB1

Option 2: SN sends the QoE measurement configuration to the MN via the XnAP to request the MN to configure the QoE measurement

In option 1, the SN sends the SN RRC message including the QoE measurement configuration directly to the UE as sending the legacy SN RRC message. There is no impact on RAN3’s specification. RAN2 needs to modify the specification to allow the SN RRC message to include the QoE measurement.

In option 2, the SN sends the QoE measurement configuration to the MN via the XnAP. Then MN generates the MN RRC message to include the QoE measurement configuration and sends the MN RRC message to the UE as in R17 QoE measurement. There are impacts on RAN3’s specification. RAN2 can reuse the R17 signalling to configure and receive.

RAN3 discusses the following options for the SN to configure the management based QoE measurement.

- Option 1: SN generates the SN RRC message including the QoE measurement configuration and sends the SN RRC message to the UE via SRB3 or via the MN SRB1

- Option 2: SN sends the request including the QoE measurement configuration to the MN, the MN configures the QoE measurement based on the request

Here it is a normal case that both MN and SN should receive the same m-based QoE measurement request from OAM, even there might be rare cases where MN and SN receive different m-based QoE measurement request at a similar time, or m-based QoE measurement request received by SN only. However, whatever the case would be, necessary action has to be discussed is, how MN and SN should be coordinated before SN sends the configuration, so that no duplicated QoE configuration is received by the UE and one QoE measurement configured by one node will not be overwritten by another node. The possible mechanisms are also straight forward, e.g. SN asks for permission from MN, or MN indicates SN to do it.  Nevertheless, the basic rule to follow should be that we allow only one node to be the charging entity which makes the decision for the above mentioned possibilities. In our understanding, MN should be this entity.

Proposal 3bis:	RAN3 to discuss the coordination mechanism between MN and SN, e.g. MN to decide if SN is allowed to configure QoE measurement.

In R17 QoE, RAN can request the pause of QoE reporting due to the overload of Uu. During the pause period, the UE continues the QoE measurement and stores the QoE results in the AS layer. The UE may discard reports when the memory reserved for storing application layer measurement reports becomes full.

In NR-DC, there are two legs between the UE and the network. If one leg is overloaded and the other one is not, the UE can report the QoE results to the network via the non-overloaded leg. Then the network does not need to pause the reporting and store the QoE report, hence avoiding discarding the report of QoE results. In extreme cases where both of the legs are overloaded, the network can still request UE to pause the QoE reporting as we did in R17.

There are two options for the network to change the leg of QoE reporting.

Option 1: define the split SRB4

Option 2: change the SRB of reporting between the UE and the network

In NR-DC, there are two legs for the split SRB. The split SRBs are setup only for the RRC messages between the MN and the UE. Therefore option 1 can only be used for the QoE configuration from the MN. Also for uplink of split SRB, the UE is configured whether to use MCG path or duplicate the transmission on both MCG and SCG. For the overload case, the duplication solution cannot reduce the overload. Therefore only the non-duplication solution can be used. But the UE can only transmit the uplink data via the MCG leg in the non-duplication case of the split SRB according to the legacy design in NR-DC. Therefore option 1 need to change the rule of split SRB.

In the option 2, the network needs to set up two SRBs for the UE. One SRB is used between the MN and the UE, another SRB is used between the SN and the UE. The network can inform the UE to switch the QoE reporting between these two SRBs. We think this option is simpler and can be used for both the overload cases of MN and SN.

If the node that configures the QoE measurement is overloaded, the network can configure the UE to report via another node.

For the overload case, switch the SRB between MN leg and SN leg for the reporting based on the overload situation.

2.2 RAN visible QoE

In R17 QoE, RAN3 discussed and specified the RAN visible QoE for network optimization by the gNB. In NR-DC, some services can be transmitted and scheduled by the SN. Therefore, the SN can also optimize the network based on RAN visible QoE.

The RAN visible QoE results should be available at SN side if SN participated in the transmission of the ongoing service type.

- Here we assume that anyway both MN and SN could learn which service is served by which entity, i.e., MN, SN or both, from the RAN visible QoE report, and either MN could send the received report to SN, or UE could send the report together with QoE measurement report to SN.

Either MN could send the received RAN visible QoE report to SN, or UE could send the RAN visible QoE report together with QoE measurement report to SN.

2.3 QoE measurement continuity in mobility scenarios

In R17 QoE, RAN3 and RAN2 specified the QoE measurement continuity in mobility scenarios. We think the R17 solution should be the baseline of the continuity in NR-DC.

In NR-DC, there are two nodes for the same UE. Therefore we need to discuss the mobility cases for both nodes. For the PCell change or PSCell change without node change, i.e. the intra-MN PCell change or intra-SN PSCell change, we can directly use the R17 solutions. In the following, we will discuss the cases with nodes change.

| Cases for inter-node mobility in NR-DC | Analysis |
| --- | --- |
| Case 1: inter-SN change without MN change | For the QoE measurement configured by the MN, it is the intra-MN PCell change. We can use the same solution of R17. For the QoE measurement configured by the SN, it is the inter-SN PSCell change. We can use the same solution of R17 inter MN PCell change. |
| Case 2: inter MN change without SN change | For the QoE measurement configured by the MN, it is the inter-MN PCell change. We can use the same solution of R17. For the QoE measurement configured by the SN, there is no change for the QoE measurement. |
| Case 3: inter MN change with SN change | For the QoE measurement configured by the MN, it is the inter-MN PCell change. We can use the same solution of R17. For the QoE measurement configured by the SN, it is the inter-SN PSCell change. We can use the same solution of R17 inter MN PCell change. |
| Case 3: inter-SN change without MN change | For the QoE measurement configured by the MN, there is no issue for the continuity. For the QoE measurement configured by the SN, we can use the same solution of R17. |
| Case 4: SN node Release | Sees the following detail discussion. |

For the QoE measurement configured by the SN in the SN change case, use the same solution in R17 inter-node change.

In R17 QoE, the UE releases the QoE measurement when the UE enters to the RRC_IDLE. In NR-DC, the UE will release all the SCG configuration when the SCG is released. But in order to keep the continuity of services, the MN can transmit the services which are served by the SN. For example, the MN can reconfigure or add DRBs for these services. For these cases, we need to keep the QoE measurement continuity because the services are still on-going, the UE can also report the QoE results to the MN.

For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case.

2.4 Alignment of QoE measurements and radio related measurement

In R17 QoE, RAN3 specified the alignment of signalling based QoE measurement and signalling immediate MDT measurement and the alignment of management based QoE measurement and management immediate MDT measurement. As discussed in the above, the SN can only configure the management based QoE measurement and the SN also can configure immediate MDT. Therefore it is straightforward to support the alignment of management based QoE measurement and management immediate MDT measurement using the R17 solution.

Support the alignment of management based QoE measurement from SN and management based MDT from SN using the same solution as in R17.

In NR-DC, the network can transmit the same service via both the MN and the SN. For example, the network can configure the MN terminated split bearer or SN terminated split bearer. In these cases, the QoE results has relation with the radios of both nodes. Therefore it is useful to align with the immediate MDT of both nodes for the purpose of QoE analysis.

For the services that use the split bearers, the QoE results need to align with immediate MDT of both nodes.

3. Proposal

In this contribution, we provide the views on the QoE in NR-DC, and get the following proposal:

Observation 1: In NR-DC, the services could be scheduled over the SN.

- For NR-DC case, the MN sends the signalling based QoE measurement configuration to the UE.

- RAN3 to discuss and agree to support the SN to configure the management based QoE measurement

RAN3 discusses the following options for the SN to configure the management based QoE measurement:

- Option 1: SN generates the SN RRC message including the QoE measurement configuration and sends the SN RRC message to the UE via SRB3 or via the MN SRB1

- Option 2: SN sends the request including the QoE measurement configuration to the MN, the MN configures the QoE measurement based on the request

Proposal 3bis:	RAN3 to discuss the coordination mechanism between MN and SN, e.g. MN to decide if SN is allowed to configure QoE measurement.

If the node that configures the QoE measurement is overloaded, the network can configure the UE to report via another node.

For the overload case, switch the SRB between MN leg and SN leg for the reporting based on the overload situation.

The RAN visible QoE results should be available at SN side if SN participated in the transmission of the ongoing service type.

Either MN could send the received RAN visible QoE report to SN, or UE could send the RAN visible QoE report together with QoE measurement report to SN.

For the QoE measurement configured by the SN in the SN change case, use the same solution in R17 inter-node change.

For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case.

Support the alignment of management based QoE measurement from SN and management based MDT from SN using the same solution as in R17.

For the services that use the split bearers, the QoE results need to align with immediate MDT of both nodes.

4. References

- [1] RP-221803, New WI: Enhancement on NR QoE management and optimizations for diverse services