3GPP TSG-RAN WG3 Meeting #117-e	R3-224893

# E-meeting, 15 – 24 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1026 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RAN AI&ML |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of the input, output and feedback for AL/ML based mobility enhancement, load balancing and energy saving. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include Predicted PRB, Predicted TNL Capacity Ind, Predicted Composite Available Capacity, Predicted HW LoadInd, Predicted Number of Active UEs and the associated intended time for prediction in RESOURCE STATUS REQUEST. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AL/ML based mobility enhancement, load balancing and energy saving is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.10.2, 9.2.1.20, 9.2.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

8.2.10	Resource Status Reporting Initiation

8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE if the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell shall include the MIMO PRB usasge Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

- 	the Predicted Radio Resource Status IE, if the seventh bit, "Predicted PRB" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one SSB, the Predicted Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE if the cell for which Predicted Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell shall include the MIMO PRB usasge Information IE.

- 	the Predicted TNL Capacity Indicator IE, if the eighth bit, "Predicted TNL Capacity Ind" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

- 	the Predicted Composite Available Capacity Group IE, if the ninth bit, "Predicted Composite Available Capacity" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". If the Cell Capacity Class Value IE is included within the Predicted Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If NG-RAN node2 is a gNB and if the cell for which Predicted Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Predicted Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value. If the SSB To Report List IE is included for a cell, the Predicted Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE.

- 	the Predicted Hardware Load Indicator IE, if the Tenth bit, "Predicted HW LoadInd" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

-	the Predicted Number of Active UEs IE, if the Eleventh bit, " Predicted Number of Active UEs" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

<Unchanged Text Omitted>

9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs, Sixth Bit = NR-U Channel List, Seventh Bit = Predicted PRB, Eighth Bit = Predicted TNL Capacity Ind, Ninth Bit = Predicted Composite Available Capacity, Tenth Bit = Predicted HW LoadInd, Eleventh Bit = Predicted Number of Active UEs. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  |  |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |  |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. | YES | ignore |
| Intended Time for Prediction | O |  | FFS | Requested time interval to which the Predicted PRB, Predicted TNL Capacity Ind, Predicted Composite Available Capacity, Predicted HW LoadInd, Predicted Number of Active UEs apply. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<Unchanged Text Omitted>

9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | - |  |
| >>Predicted Radio Resource Status | O |  | 9.3.1.129 |  |  |  |
| >>Predicted TNL Capacity Indicator | O |  | 9.3.1.128 |  |  |  |
| >>Predicted Composite Available Capacity Group | O |  | 9.3.1.130 |  |  |  |
| >>Predicted HW LoadInd | O |  | 9.3.1.136 |  |  |  |
| >>Predicted Number of Active UEs | O |  | 9.3.1.135 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

Change Ends