3GPP TSG-RAN WG3 Meeting #117 electronic                                                       R3-224908

Online, August 15-26, 2022

Agenda Item:	12.2.2

Source:	CMCC

Title:	Open Issues for AI/ML for NG-RAN

Document for:	Discussion and Decision

1	Introduction

The Work Item “Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved for Rel-18 at the 3GPP TSG RAN #94E meeting and updated in [1].

Based on the conclusions of the Rel-17 SI, the objective for Rel-18 WI is to Specify data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

Still, there are some open issues that did not achieve agreements during the study phase. In this contribution, we provide our views on open issues that left to be discussed during R18 normative work.

2	Discussion

During the Rel-17 study phase of AI/ML for NG-RAN, three high prioritized use cases, i.e., the AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization, were studied and solutions were captured in TR37.817 [2]. Following are some open issues needs more discussion in Rel-18:

- Model output validity time

- Whether the UE trajectory prediction is an external output to the node hosting the Model Inference function

- The details of performance information from target NG-RAN for feedback of AI/ML-based Mobility Optimization

- Current/predicted UE traffic

- Define the framework and procedure to let one gNB in a neighborhood to provide Model Training function to all the other gNBs in this neighborhood.

- Differentiate the input and output for Model Training function and Model Inference function

- Model output validity time

The model output validity time is to indicate the applicative time for the results obtained from AI/ML model, it might be a time period or time point for predicted decision. The validity time was discussed during the SI, but no consensus achieved.

The proponents deemed that without such information, the results may not benefit to the RAN if applying it to a misplaced time. Only when the decision would be done in the determined time, the function is valuable. However, opponents thought it is unnecessary since the received output information is valid till a new version of such information is received. Furthermore, if the output information is prediction type, the output information would be reported periodically, which implicitly indicate the validity time.

In our understanding, the information of the model output validity time is not necessary for aperiodic model output, since when new version of such information is received, the old one becomes invalid. As for periodical output, the valid time could be the periodicity of the model output.

Proposal 1: The model output validity time could be introduced for periodical output, and the valid time could be the periodicity of the output.

- Whether UE trajectory prediction is an external output

The predicted UE trajectory from source node to target node could be used for the subsequent mobility optimization. For example, UE trajectory prediction could predict the UE’s locations for a future period of time. The target RAN node may use the predicted UE trajectory prediction after UE moves to the target RAN node. However, opponents think the UE trajectory information is a very sensitive information subject to user consent. For that, signalling it over open interfaces should be discouraged.

We agree that UE trajectory information may need to subject to user consent due to it is sensitive, but the information is beneficial for target RAN node to further optimize UE’s mobility.  So, we propose that:

Proposal 2: UE trajectory prediction could be an external output for UE with user consent.

- Details of performance information from target NG-RAN for feedback of AI/ML-based Mobility Optimization

During SI, performance information from target NG-RAN used as feedback of AI/ML-based Mobility Optimization was discussed, but details on the performance information was diverse:

- UE performance affected by the model inference action (e.g. handed-over UEs), including bitrate, packet loss, latency; system KPIs (e.g. throughput, delay, RLF of current and neighboring NG-RAN node).

- Handover performance observed at a target, UE Configuration received by a UE or a group of UEs at a target, etc.

- Merge with the first feedback item agreed in the TR as “Performance information from target NG-RAN, including QoS parameters such as throughput, packet delay of the handed-over UE”

- This item can be removed as the 2 existing feedback items agreed (“QoS parameters such as throughput, packet delay of the handed-over UE, etc.” and “Resource status information updates from target NG-RAN”) already cover sufficient performance feedback.

In our understanding, the performance information consists of two parts: the performance of handed-over UEs and NG-RAN, and the existing feedback items in the TR QoS parameters of the handed-over UE and Resource status information updates from target NG-RAN have covered the performance information.

Proposal 3: Remove the item “performance information from target NG-RAN” from the feedback of AI/ML-based Mobility Optimization.

- Current/predicted UE traffic

Current and predicted UE traffic was discussed during the SI phase but not captured in the TR. In our understanding, the UE traffic information is beneficial for target node to prepare necessary resource for the UE, e.g., configure CA or DC for the UE with high traffic. Furthermore, the existing UE Aggregate Maximum Bit Rate or UE Slice Maximum Bit Rate List IE in HANDOVER REQUEST message could be reused to indicate the UE Aggregate Maximum Bit Rate or UE Slice Maximum Bit Rate in both the downlink direction and the uplink direction.

Proposal 4: Reuse existing UE Aggregate Maximum Bit Rate or UE Slice Maximum Bit Rate List IE in HANDOVER REQUES to obtain the UE’s traffic information from neighbouring node.

- Model Training function and Model Inference function exist in different gNBs

Current high-level principle provides two options for Model Training function and Model Inference function:

• AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

• AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

However for option2, if some gNB can’t provide Model Training function, the another gNB in the neighborhood which can provide the Model Training function need to be selected. This framework can also help improve the resource utility. The detailed framework and procedure need to be further discussed in R18. The model and feedback can be transmitted between two gNBs through Xn interface.

Proposal 5: How to use one gNB in a neighbourhood to provide Model Training function to all the other gNBs in the neighbourhood need to be discussed and defined. The Xn interface should be used to exchange the trained/updated models and feedbacks.

- Differentiate the input for Model Training function and Model Inference function

In current three use cases: Energy Saving, Load Balance and Mobility Optimization, the Input description are combined for Model Training function and Model Inference function. The input for Model Inference function should be a subset of the input for Model Training function. In the architecture where these two functions exist in different locations, e.g. Model Training function in OAM and Model Inference function in gNB, or in different gNBs, the input should be differentiated. What is the input for Model Inference function should be figured out, to avoid the duplicated data transfer. This may be discussed case by case in each step.

Proposal 6:  When the Model Training function and Model Inference function exist in different locations, the input for Model Training function and Model Inference function should be differentiated. This should be studied case by case.

3	Summary

In this contribution, we provide our views on open issues that left to be discussed during R18 normative work. Following proposals are made:

Proposal 1: The model output validity time could be introduced for periodical output, and the valid time could be the periodicity of the output.

Proposal 2: UE trajectory prediction could be an external output for UE with user consent.

Proposal 3: Remove the item “performance information from target NG-RAN” from the feedback of AI/ML-based Mobility Optimization.

Proposal 4: Reuse existing UE Aggregate Maximum Bit Rate or UE Slice Maximum Bit Rate List IE in HANDOVER REQUES to obtain the UE’s traffic information from neighbouring node.

Proposal 5: How to use one gNB in a neighbourhood to provide Model Training function to all the other gNBs in the neighbourhood need to be discussed and defined. The Xn interface should be used to exchange the trained/updated models and feedbacks.

Proposal 6:  When the Model Training function and Model Inference function exist in different locations, the input for Model Training function and Model Inference function should be differentiated. This should be studied case by case.

4	Reference

[1] 	3GPP RP-220635, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, CMCC, Ericsson

[2]      3GPP RP-220617 TR 37.817-200, CMCC