3GPP TSG-RAN WG3 Meeting #117-e	R3-224915

Online, 15-24 August 2022

Agenda item:	15.3

Source:	CMCC

Title:	Multicast Reception in RRC_INACTIVE state

Document for:	Discussion

Introduction

The Release 18 work item on “Enhancements of NR Multicast and Broadcast Services” was approved in RAN#94, which can be found in RP-213568 [1].

The objectives to support for supporting of multicast reception by UEs in RRC_INACTIVE state are highlighted as below,

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

In this contribution, we analyse the possible scenarios of UEs in RRC-INACTIVE state.

Discussion

2.1	Scenario Analysis

From the description in SA2’s SR [2], a key issue waiting to be solved is to enable service for a large number of UEs in a cell. Under the public safety scenario, public safety UEs engaged in group calls have the capability to perform multicast transmission which shares only one bearer for downlink delivery. In the extreme condition, numerous connected public safety UEs may exceed the maximum number of connections in a cell. A potential solution for enabling UEs to receive the service under this circumstance is provided in the solution part of the same TR. The solution assumes that UE continues to receive the service based on using UEs in IDLE state without attempting to modify the admission control procedures of the RAN. Although, UE receiving multicast in idle state finally does not reach an agreement in RAN plenary. An alternative plan is transferring UE to inactive state and enable UE multicast reception in inactive state. Additionally, multicast reception in inactive state improves the resource efficiency and the cell capacity.

Considering that supporting the multicast reception in RRC_INACTIVE state, two potential scenarios could be divided into the following two cases:

- An RRC_CONNECTED UE which is receiving multicast transitions to inactive state and continues the multicast reception

- An RRC_INACTIVE UE which has joined the multicast session starts the multicast reception in inactive state once receiving the session activation

From our view, both two scenarios should be considered and discussed in the following RAN3 meetings.

Proposal 1: RAN3 should consider both scenarios in the following meetings.

- An RRC_CONNECTED UE which is receiving multicast transitions to inactive state and continues the multicast reception.

- An RRC_INACTIVE UE which has joined the multicast session starts the multicast reception in inactive state once receiving the session activation.

For scenario 1), the transition could be decided by UE or RAN.

If the transition is decided by UE, UE could send the request indicating its preference state to network via RRC signalling. UE itself decides stays in connected state or moves to inactive state combining its own situation, i.e., RSRP value, UE location, velocity. Before UE initiates the request, gNB can also send the 5QI values for low QoS requirement service indicating the multicast service with low QoS requirement mapping to those 5QI values are received in inactive state. After obtaining the preference state, gNB moves UE into inactive state.

If the transition is decided by RAN, gNB is a judge who determines whether to keep UE in connected state or move it to inactive state. gNB could make the decision based on the QoS requirements for the multicast service or UE periodically report information. UE in connected state can report its UE-specific information, i.e., RSRP value, UE location, velocity, to gNB periodically in connected state as assistance information.

Besides, the indication from CN may influence the decision made by RAN. gNB may receive the MBS session deactivation procedure triggered by CN or assistance information indicating the MBS session should be deactivated. From the description in SA2 specification [3], MBS session deactivation procedure is triggered by MB-SMF, when it receives the notification from MB-UPF in the case that there is no downlink data to be transmitted for some duration, or when it receives the request directly from AF or via NEF. After receiving the MBS session deactivation procedure, gNB moves the UE from RRC_CONNECTED state to RRC_INACTIVE state.

From above analysis, whatever the decision is made by UE or RAN, signalling exchange between gNB and UE is mostly in RAN2 scope. Thus, RAN3 should focus on the signalling exchange between CN and RAN in the case of transitioning UE from connected state to inactive state.

Proposal 2: RAN3 should focus on the signalling exchange between CN and RAN in the case of transitioning UE from connected state to inactive state.

For scenario 2), we have already reached the agreement in Rel-17 that RAN nodes perform group paging after the AMF sends NGAP activation request message. Aiming to support inactive state UEs receiving multicast, gNB should keep UEs in inactive state instead of bringing back them to connected state.

When UE is in inactive state, the schemes for PTM configuration provision and are in RAN2’s scope. Basically, there are two options carrying the PTM configuration. One is using RRC dedicated signalling, the other is acquiring content in MCCH after obtaining SIB content. From our perspective, the discussion about the way of transmitting PTM configuration should not be involved in RAN3’s scope.

Proposal 3: The discussion about the way of transmitting PTM configuration should not be involved in RAN3’s scope.

2.2	Mobility in RRC_INACTIVE

Based on Rel-17 discussion about MBS mobility with service continuity for UEs in connected state, we would continue to investigate MBS mobility with service continuity for UEs in inactive state. Considering that the capability for inactive multicast service at source and target side, we classify the cell reselection scenario into four cases.

- Source cell supports inactive multicast service. Target cell supports inactive multicast service with same PTM configuration;

- Source cell supports inactive multicast service. Target cell supports inactive multicast service with different PTM configuration;

- Source cell supports inactive multicast service. Target cell does not support inactive multicast service;

- Source cell does not support inactive multicast service. Target cell supports inactive multicast service;

In all scenarios, UE should first determine whether its neighbour cells support inactive multicast service. The capability of supporting inactive multicast service in a cell could be exchanged over Xn or broadcast in MCCH with an explicit indication.

If source and target cell both support inactive multicast service with same or different PTM configuration, UE acquires the new PTM configuration of target cell before or after the cell reselection or reuses the legacy PTM configuration of source cell. The source and target gNB may exchange the information of PTM configuration of inactive multicast service to help multicast reception for UE after reselection.

Observation 1: The source and target gNB should exchange the information of PTM configuration of inactive multicast service if source and target cell both support inactive multicast service.

If source cell supports inactive multicast service and target cell does not support inactive multicast service, UE should initiate RRC resume request in target cell or receiving the paging message from source cell. Furthermore, source cell could acquire the PTM configuration of target cell for connected multicast service and other MBS context information in advance over Xn or NG.

If source cell does not support inactive multicast service and target cell supports inactive multicast service, UE should perform the legacy handover procedure first and get access to the target cell. The target cell transitions UE state from connected to inactive. The procedure may use the legacy handover without introducing new signalling exchange over Xn or NG.

Proposal 4: RAN3 could focus on the scenario where source and target cell both support inactive multicast service and the scenario where source cell supports inactive multicast service and target cell does not support inactive multicast service.

Conclusions

In this paper, we have discussed the multicast reception in RRC_INACTIVE state. The observations and proposal are listed below:

Observation 1: The source and target gNB should exchange the information of PTM configuration of inactive multicast service if source and target cell both support inactive multicast service.

Proposal 1: RAN3 should consider both scenarios in the following meetings.

- An RRC_CONNECTED UE which is receiving multicast transitions to inactive state and continues the multicast reception.

- An RRC_INACTIVE UE which has joined the multicast session starts the multicast reception in inactive state once receiving the session activation.

Proposal 2: RAN3 should focus on the signalling exchange between CN and RAN in the case of transitioning UE from connected state to inactive state.

Proposal 3: The discussion about the way of transmitting PTM configuration should not be involved in RAN3’s scope.

Proposal 4: RAN3 could focus on the scenario where source and target cell both support inactive multicast service and the scenario where source cell supports inactive multicast service and target cell does not support inactive multicast service.

Reference

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, CATT;

- 3GPP TR 23.774 V17.0.0 Study on mission critical services over 5G multicast broadcast system (Release 17);

- 3GPP TS 23.247 V17.3.0 Architectural enhancements for 5G multicast-broadcast services;