3GPP TSG-RAN WG3 Meeting #117-e	R3-224930

# Online, 15th August – 24th August, 2022

Agenda Item:	10.4

Source:	Ericsson

Title:	Corrections related to Excess Packet Delay

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we continue the discussion started in RAN3#116-e aimed at syncing the current Excess Packet Delay specifications in RAN3 with the current RAN2 agreements on the subject.

# 2	Discussion

## 2.1 Discussion on the presence field of the M6 Delay Threshold

During the previous RAN3#115-e meeting it has been agreed to change the current M6 configuration IE by introducing the M6 Delay Threshold IE to enable the calculation of the PDCP Excess Packet Delay in the UL as defined in TS 38.314. In particular, the following changes on the M6 configuration have been agreed in RAN3 [1]:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  |  |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |  |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity,…) | Number of reports | Yes | Ignore |
| M6 Delay Threshold | C-ifUL |  | ENUMERATED (ms0.25,ms0.5,ms1,ms2,ms4,ms10, ms20, ms50, ms100, ms500, …) |  | Yes | Ignore |

| Condition | Explanation |
| --- | --- |
| ifUL | This IE shall be present if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink”. |

Nevertheless, the following agreements have been made in RAN2#116 on Excess Packet Delay:

Agreements

1 The new delay measurement can be called excess packet delay for NR.

2 FFS: the definition of the measurement of excess packet delay for NR is:

- it represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant)

3 The network can collect the measurement excess packet delay for NR from the UE.

4 LTE excess packet delay reporting can be used as a baseline, and details can be further discussed.

RAN2 went on to specify the details of the UL PDCP Excess Packet Delay measurement in both TS38.314 and TS38.331. Excerpts from these specifications is shown below:

Start of Excerpt from TS38.314

#### 4.3.1.2	UL PDCP Excess Packet Delay per DRB

The objective of this measurement performed by UE is to measure Excess Packet Delay in Layer PDCP for QoS verification of MDT.

Protocol Layer: PDCP

Table 4.3.1.2-1: Definition for UL PDCP Excess Packet Delay per DRB

| Definition | PDCP Excess Packet Delay in the UL per DRB. It represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=nExcess(T,drbid)nTotal(T,drbid) tULdelay(i,drbid)=tDeliv(i,drbid)-tArrival(i,drbid),where explanations can be found in the table 4.3.1.2-1 below. |
| --- | --- |

Table 4.3.1.2-2

| M(T,drbid) | Ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs for which the UL MAC PDU including the first part of UL PDCP SDU is scheduled for transmission during the time period T. |
| --- | --- |
| nExcess(T,drbid) | Number of PDCP SDUs of a data radio bearer with DRB Identity = drbid,for which ULdelay tULdelay(i,drbid) exceeded the configured delayThreshold as defined in TS 38.331 [3] during the time period T. |
| nTotal(T,drbid) | Number of PDCP SDUs of a data radio bearer with DRB Identity = drbid, for which the UL MAC PDU including the first part of UL PDCP SDU is scheduled for transmission during the time period T. |
| tULdelay(i,drbid) | Queuing delay observed at the UE PDCP layer from the UL PDCP SDU i belonging to a data radio bearer with DRB Identity = drbid arrival at PDCP upper SAP until the UL MAC PDU k including the first part of UL PDCP SDU i is scheduled for transmission during the time period T. |
| tDeliv(i,drbid) | The point in time when the UL MAC PDU k including the first part of UL PDCP SDU i is scheduled for transmission. |
| tArrival(i,drbid) | The point in time when the UL PDCP SDU i arrivals at PDCP upper SAP. |
| i | Index of PDCP SDU that arrives at the PDCP upper SAP during time period T. |
| T | Time period during which the measurement is performed. |

End of Excerpt from TS38.314

Start of Excerpt from TS38.331

#### –	UL-ExcessDelayConfig

The IE UL-ExcessDelayConfig IE specifies the configuration of the UL PDCP Excess Packet Delay per DRB measurement specified in TS 38.314 [53].

UL-ExcessDelayConfig information element

-- ASN1START

-- TAG-ULEXCESSDELAYCONFIG-START

UL-ExcessDelayConfig-r17 ::=  SEQUENCE {

excessDelay-DRBlist-r17       SEQUENCE (SIZE(1..maxDRB)) OF ExcessDelay-DRB-IdentityInfo-r17

}

ExcessDelay-DRB-IdentityInfo-r17 ::=  SEQUENCE {

drb-IdentityList                      SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity,

delayThreshold                        ENUMERATED {ms0dot25, ms0dot5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70,

ms80, ms90, ms100, ms150, ms300, ms500}

}

-- TAG-ULEXCESSDELAYCONFIG-STOP

-- ASN1STOP

UL-ExcessDelayConfig field descriptions

drb-IdentityList Indicates the DRB IDs used by UE to provide results of UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53].

delayThreshold Indicates the delay threshold for the DRB IDs indicated in DRB-IdentityList. Value ms0dot25 corresponds to 0.25ms, ms0dot5 corresponds to 0.5ms, ms1 corresponds to 1ms and so on.

End of Excerpt from TS38.331

Therefore, in RAN2, excess packet delay is a new measurement, and it is not directly coupled to M6. Namely, RAN2’s conclusions are that the M6 delay measurement should be provided as per legacy specifications, but additionally excess packet delay measurements may be added to the M6 measurement report.

The current IE name gives the incorrect impression that the legacy M6 measurement is subject to a threshold, which is not correct. This was also the view shared by most companies in the RAN3#116-e meeting, captured in the SoD [2]:

Therefore, based on majorities view, the following can be agreed.

Add Excess Packet Delay Threshold IE in M6 Configuration.

Observation 1:	Excess Packet delay in RAN2 is a L2 measurement not necessarily coupled to the M5 measurement.

As it can be seen in the excerpt from TS38.314 above, RAN2 did not prohibit using the M6 configuration IE to communicate the Excess Packet Delay Threshold IE but on the other hand the current RAN3 specification captured a mandatory M6 Delay Threshold IE when the condition C-ifUL is satisfied. For clarity, the condition that mandates inclusion of the M6 Delay Threshold is defined as follows (see TS38.413):

| Condition | Explanation |
| --- | --- |
| ifUL | This IE shall be present if the M6 Links to Log IE is set to “uplink” or to “both-uplink-and-downlink”. |

Correspondingly, the current RAN3 spec is not correct because it implies that Excess Packet Delay measurements need to be collected whenever the M6 measurements are collected in UL, which is totally against RAN2´s agreements.

As a first correction to align with RAN2, the M6 Delay Threshold IE should be renamed Excess Packet Delay Threshold IE, to be in line with RAN2’s agreements, and the presence condition should be updated to Optional to match the RAN2 behaviour (namely to make the Excess Packet Delay measurements independent from the M6 measurements).

Correct the presence field of the M6 Delay Threshold IE to Optional and rename the M6 Delay Threshold IE to Excess Packet Delay Threshold IE to align with RAN2.

## 2.2 Multiple Excess Packet Delay values

During the RAN2#117 meeting the following agreements related to Excess Packet Delay have been made:

Agreements:

1: For UL PDCP Excess Packet Delay (related to section 4.3.1.e in TS 38.314 CR), network should be able to configure different delay threshold for different DRBs.

2: For excess delay configuration in NR-DC, Node owning the PDCP terminating point configures the UE

- Similar to the solution for D1 configuration, for all the bearers, the CU-CP of the node owning the PDCP terminating point configures the UE with excess delay measurement configuration. To enable this solution UE is allowed to be configured with at most one excess delay measurement per PDCP, which follows the D1 measurement for NR DC.

3: D1 delay ratio measurement results should include DRB id and excessDelay info, and they can be included in the IE MeasResults.

4: For D1 delay threshold values, at least the following values can be included:

250us, 0.5ms, 1ms, 2ms, 4ms, 10ms, 20ms, 50ms, 100ms, 500ms (10 values)

It is quite evident from the above RAN2 agreement that the RAN is enabled to configure different delay thresholds for different DRBs. These different delay threshold values need to be provided by the OAM while the RAN would be responsible for mapping the provided values to the relevant DRBs, as these are under the RAN’s responsibility.

Currently only a single delay threshold value is communicated to the RAN through the M6 Configuration, denying the possibility at RAN to configure different Excess Packet Delay thresholds per DRB.

Observation 2:	The present M6 Configuration in Rel-17 does not allow to associate different thresholds on a per DRB basis, which contradicts RAN2 agreements.

This problem was acknowledged in the last RAN3#116-e meeting by most companies, but no final agreement on how to solve this issue was achieved. A proposed solution to this problem is to enable multiple threshold parameters in the renamed Excess Packet Delay Threshold IE as shown below:

| - IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Excess Packet Delay Threshold |  | 1..<maxnoofThresholds> |  |  |
| >Excess Packet Delay Threshold Value | M |  | ENUMERATED (ms0.25, ms0.5, ms1, ms2, ms4, ms10, ms20, ms50, ms100, ms500, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of Thresholds allowed within one PDU session. Value is 64. |

One criticism to the above solution is that it does not achieve full potential and a better one could be proposed, perhaps with more specific instructions on how to associate each threshold value to a DRB.

We agree that the above solution is a compromise which allows to configure multiple thresholds and leaves up to implementation how to associate different thresholds on a per DRB basis. However, in order to minimise the impact on Rel17 and to achieve alignment with RAN2 specifications, we believe that Rel-17 specs should be fixed at least to a minimum level, so as not to contradict RAN2 agreements. It should be considered that the current RAN3 specifications are “broken” and if not fixed they define a measurement, to be collected by a UE, that does not exist in RAN2 specs.

In the coming meetings, RAN3 can discuss on Rel-18 enhancements to this solution and possibly converge more detailed ways on how to assign each threshold value to a DRB.

Enable multiple Excess Packet Delay Threshold configurations as proposed above for Rel-17.

RAN3 to agree on discussing enhancements of the proposed solution during Rel-18.

It is proposed to take the draft CR to TS 38.413 in Appendix A as a baseline and to agree to it.

Given that the M6 configuration is propagated over the Xn, similar changes need to be applied to the 38.423 specification.

It is proposed to take the draft CR to TS 38.423 in Appendix B as a baseline and to agree to it.

# 3	Conclusion

In this contribution, the following observations and proposals were derived:

Observation 1:	Excess Packet delay in RAN2 is a L2 measurement not necessarily coupled to the M5 measurement.

- Correct the presence field of the M6 Delay Threshold IE to Optional and rename the M6 Delay Threshold IE to Excess Packet Delay Threshold IE to align with RAN2.

Observation 2:	The present M6 Configuration in Rel-17 does not allow to associate different thresholds on a per DRB basis, which contradicts RAN2 agreements.

Enable multiple Excess Packet Delay Threshold configurations as proposed above for Rel-17.

RAN3 to agree on discussing enhancements of the proposed solution during Rel-18.

It is proposed to take the draft CR to TS 38.413 in Appendix A as a baseline and to agree to it.

It is proposed to take the draft CR to TS 38.423 in Appendix B as a baseline and to agree to it.

# 4. References

- R3-222883, (TP for MDT BL CR for TS38.413) Introduce PDCP Excess Packet Delay, ZTE, Ericsson, CATT

- R3-223844, Summary of Offline Discussion on CB: # SONMDT2_MDT, ZTE

# Appendix A - CR for TS 38.413

3GPP TSG-RAN WG3 Meeting #117-e	R3-22xxxx

# Online, 15th August – 24th August, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | - | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP Corrections related to Excess Packet Delay based on RAN2 status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current Excess Packet Delay specifications in RAN3 are not in sync with the RAN2 agreements on the subject. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During the previous RAN3#115-e meeting it has been agreed to change the current M6 configuration IE by introducing the M6 Delay Threshold IE to enable the calculation of the PDCP Excess Packet Delay in the UL. Nevertheless, in RAN2, excess packet delay is viewed as a new measurement and is not directly coupled to M6. Namely, RAN2’s conclusions are that the M6 delay measurement should be provided as per legacy specifications, but additionally excess packet delay measurements may be added to the M6 measurement report. In 38.314, section 4.3.1.2, the PDCP Excess Packet Delay is defined as follows: PDCP Excess Packet Delay in the UL per DRB. It represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. The RAN2 agreement does not prohibit using the M6 configuration IE to communicate the Excess Packet Delay Threshold IE but on the other hand the current RAN3 specification captured a mandatory M6 Threshold IE when the condition C-ifUL is satisfied. Correspondingly, the current RAN3 spec is not correct. The M6 Threshold IE should be renamed Excess Packet Delay Threshold IE and the presence condition should be updated to Optional to match the RAN2 behavior. Furthermore, RAN2 has agreed that for UL PDCP Excess Packet Delay (related to section 4.3.1.2 in TS 38.314) different delay threshold values for different DRBs are to be used. These different delay threshold values need to be provided by the CN/OAM while the RAN would be responsible for mapping the provided values to the relevant DRBs as these are under the RAN’s responsibility. Currently only a single delay threshold value is communicated to the RAN through the M6 Configuration. A solution to this problem it to enable multiple threshold parameters in the renamed Excess Packet Delay Threshold IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact with the previous version of the specification. There is a non-backwards compatible change in the ASN.1 related to the name change on the Excess Packet Delay Threshold IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN2 and RAN3 specifications are not in sync while RAN3 specification will fail to provide functionality already assumed by agreed RAN2 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.174, 9.3.1.x, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS38.423 CRxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.174	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  | - |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  | - |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity, …) | Number of reports | YES | ignore |
| Excess Packet Delay Threshold | O |  | 9.3.1.x | Indicates that excess packet delay measurements need to be reported, as specified in TS 38.314 [48], additionally to M6 measurements | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Excess Packet Delay Threshold

This IE defines the parameters for Excess Packet Delay Threshold configuration in order to enable the calculation of the PDCP Excess Packet Delay in the UL per DRB as specified in TS 38.314 [48].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Excess Packet Delay Threshold |  | 1..<maxnoofThresholds> |  |  |
| >Excess Packet Delay Threshold Value | M |  | ENUMERATED (ms0.25, ms0.5, ms1, ms2, ms4, ms10, ms20, ms50, ms100, ms500, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of Thresholds allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< End of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- TEXT OMITTED –

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT		PRESENCE optional		}|

{ ID id-ExcessPacketDelayThreshold	CRITICALITY ignore	EXTENSION ExcessPacketDelayThreshold		PRESENCE optional		},

...

}

ExcessPacketDelayThreshold ::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF ExcessPacketDelay-thresholdItem

ExcessPacketDelay-thresholdItem ::= SEQUENCE {

excessPacketDelay-thresholdValue			ExcessPacketDelay-thresholdValue,

iE-Extensions		ProtocolExtensionContainer { { ExcessPacketDelay-thresholdItem-ExtIEs} }	OPTIONAL,

...

}

ExcessPacketDelay-thresholdItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExcessPacketDelay-thresholdValue ::= ENUMERATED {

ms0dot25, ms0dot5, ms1, ms2, ms4, ms10, ms20, ms50, ms100, ms500,

...

}

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< End of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- TEXT OMITTED –

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedCAGsperPLMN			INTEGER ::= 256

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCAGSperCell					INTEGER ::= 64

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsforMBS					INTEGER ::= 8192

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinNGRANNode				INTEGER ::= 16384

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

maxnoofCellsUEMovingTrajectory		INTEGER ::= 16

maxnoofDRBs							INTEGER ::= 32

maxnoofEmergencyAreaID				INTEGER ::= 65535

maxnoofEAIforRestart				INTEGER ::= 256

maxnoofEPLMNs						INTEGER ::= 15

maxnoofEPLMNsPlusOne				INTEGER ::= 16

maxnoofE-RABs						INTEGER ::= 256

maxnoofErrors						INTEGER ::= 256

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofForbTACs						INTEGER ::= 4096

maxnoofFreqforMDT					INTEGER ::= 8

maxnoofMBSAreaSessionIDs			INTEGER ::= 256

maxnoofMBSFSAs						INTEGER ::= 64

maxnoofMBSQoSFlows					INTEGER ::= 64

maxnoofMBSSessions					INTEGER ::= 32

maxnoofMBSSessionsofUE				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofMRBs							INTEGER ::= 32

maxnoofMultiConnectivity			INTEGER ::= 4

maxnoofMultiConnectivityMinusOne	INTEGER ::= 3

maxnoofNeighPCIforMDT				INTEGER ::= 32

maxnoofNGAPIESupportInfo			INTEGER ::= 32

maxnoofNGConnectionsToReset			INTEGER ::= 65536

maxnoofNRCellBands					INTEGER ::= 32

maxnoofNSAGs						INTEGER ::= 256

maxnoofPagingAreas					INTEGER ::= 64

maxnoofPC5QoSFlows 					INTEGER ::= 2048

maxnoofPDUSessions					INTEGER ::= 256

maxnoofPLMNs						INTEGER ::= 12

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo	INTEGER ::= 8

maxnoofQosFlows						INTEGER ::= 64

maxnoofQosParaSets					INTEGER ::= 8

maxnoofRANNodeinAoI					INTEGER ::= 64

maxnoofRecommendedCells				INTEGER ::= 16

maxnoofRecommendedRANNodes			INTEGER ::= 16

maxnoofAoI							INTEGER ::= 64

maxnoofReportedCells				INTEGER ::= 256

maxnoofSensorName					INTEGER ::= 3

maxnoofServedGUAMIs					INTEGER ::= 256

maxnoofSliceItems					INTEGER ::= 1024

maxnoofSuccessfulHOReports			INTEGER ::= 64

maxnoofTACs							INTEGER ::= 256

maxnoofTACsinNTN						INTEGER ::= 12

maxnoofTAforMDT						INTEGER ::= 8

maxnoofTAIforInactive				INTEGER ::= 16

maxnoofTAIforMBS					INTEGER ::= 1024

maxnoofTAIforPaging					INTEGER ::= 16

maxnoofTAIforRestart				INTEGER ::= 2048

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofUEsforPaging					INTEGER ::=	4096

maxnoofWLANName						INTEGER ::= 4

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofCandidateCells				INTEGER ::= 32

maxnoofTargetS-NSSAIs				INTEGER ::= 8

maxNRARFCN							INTEGER ::= 3279165

maxnoofCellIDforQMC					INTEGER ::= 32

maxnoofPLMNforQMC					INTEGER ::= 16

maxnoofUEAppLayerMeas				INTEGER ::= 16

maxnoofSNSSAIforQMC					INTEGER ::= 16

maxnoofTAforQMC						INTEGER ::= 8

maxnoofThresholds					INTEGER ::= 64

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-SelectedPLMNIdentity									ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType											ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-AdditionalRedundantDL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 183

id-AdditionalRedundantDLQosFlowPerTNLInformation		ProtocolIE-ID ::= 184

id-AdditionalRedundantNGU-UP-TNLInformation				ProtocolIE-ID ::= 185

id-AdditionalRedundantUL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 186

id-CNPacketDelayBudgetDL								ProtocolIE-ID ::= 187

id-CNPacketDelayBudgetUL								ProtocolIE-ID ::= 188

id-ExtendedPacketDelayBudget							ProtocolIE-ID ::= 189

id-RedundantCommonNetworkInstance						ProtocolIE-ID ::= 190

id-RedundantDL-NGU-TNLInformationReused					ProtocolIE-ID ::= 191

id-RedundantDL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 192

id-RedundantDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 193

id-RedundantQosFlowIndicator							ProtocolIE-ID ::= 194

id-RedundantUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 195

id-TSCTrafficCharacteristics							ProtocolIE-ID ::= 196

id-RedundantPDUSessionInformation 						ProtocolIE-ID ::= 197

id-UsedRSNInformation									ProtocolIE-ID ::= 198

id-IAB-Authorized										ProtocolIE-ID ::= 199

id-IAB-Supported										ProtocolIE-ID ::= 200

id-IABNodeIndication									ProtocolIE-ID ::= 201

id-NB-IoT-PagingDRX										ProtocolIE-ID ::= 202

id-NB-IoT-Paging-eDRXInfo								ProtocolIE-ID ::= 203

id-NB-IoT-DefaultPagingDRX								ProtocolIE-ID ::= 204

id-Enhanced-CoverageRestriction							ProtocolIE-ID ::= 205

id-Extended-ConnectedTime								ProtocolIE-ID ::= 206

id-PagingAssisDataforCEcapabUE							ProtocolIE-ID ::= 207

id-WUS-Assistance-Information							ProtocolIE-ID ::= 208

id-UE-DifferentiationInfo								ProtocolIE-ID ::= 209

id-NB-IoT-UEPriority									ProtocolIE-ID ::= 210

id-UL-CP-SecurityInformation							ProtocolIE-ID ::= 211

id-DL-CP-SecurityInformation							ProtocolIE-ID ::= 212

id-TAI													ProtocolIE-ID ::= 213

id-UERadioCapabilityForPagingOfNB-IoT					ProtocolIE-ID ::= 214

id-LTEV2XServicesAuthorized								ProtocolIE-ID ::= 215

id-NRV2XServicesAuthorized								ProtocolIE-ID ::= 216

id-LTEUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 217

id-NRUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 218

id-PC5QoSParameters										ProtocolIE-ID ::= 219

id-AlternativeQoSParaSetList							ProtocolIE-ID ::= 220

id-CurrentQoSParaSetIndex								ProtocolIE-ID ::= 221

id-CEmodeBrestricted									ProtocolIE-ID ::= 222

id-EUTRA-PagingeDRXInformation							ProtocolIE-ID ::= 223

id-CEmodeBSupport-Indicator								ProtocolIE-ID ::= 224

id-LTEM-Indication										ProtocolIE-ID ::= 225

id-EndIndication										ProtocolIE-ID ::= 226

id-EDT-Session											ProtocolIE-ID ::= 227

id-UECapabilityInfoRequest								ProtocolIE-ID ::= 228

id-PDUSessionResourceFailedToResumeListRESReq			ProtocolIE-ID ::= 229

id-PDUSessionResourceFailedToResumeListRESRes			ProtocolIE-ID ::= 230

id-PDUSessionResourceSuspendListSUSReq					ProtocolIE-ID ::= 231

id-PDUSessionResourceResumeListRESReq					ProtocolIE-ID ::= 232

id-PDUSessionResourceResumeListRESRes					ProtocolIE-ID ::= 233

id-UE-UP-CIoT-Support									ProtocolIE-ID ::= 234

id-Suspend-Request-Indication							ProtocolIE-ID ::= 235

id-Suspend-Response-Indication							ProtocolIE-ID ::= 236

id-RRC-Resume-Cause										ProtocolIE-ID ::= 237

id-RGLevelWirelineAccessCharacteristics					ProtocolIE-ID ::= 238

id-W-AGFIdentityInformation								ProtocolIE-ID ::= 239

id-GlobalTNGF-ID										ProtocolIE-ID ::= 240

id-GlobalTWIF-ID										ProtocolIE-ID ::= 241

id-GlobalW-AGF-ID										ProtocolIE-ID ::= 242

id-UserLocationInformationW-AGF							ProtocolIE-ID ::= 243

id-UserLocationInformationTNGF							ProtocolIE-ID ::= 244

id-AuthenticatedIndication								ProtocolIE-ID ::= 245

id-TNGFIdentityInformation								ProtocolIE-ID ::= 246

id-TWIFIdentityInformation								ProtocolIE-ID ::= 247

id-UserLocationInformationTWIF							ProtocolIE-ID ::= 248

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 249

id-IntersystemSONConfigurationTransferDL				ProtocolIE-ID ::= 250

id-IntersystemSONConfigurationTransferUL				ProtocolIE-ID ::= 251

id-SONInformationReport									ProtocolIE-ID ::= 252

id-UEHistoryInformationFromTheUE						ProtocolIE-ID ::= 253

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 254

id-MDTConfiguration										ProtocolIE-ID ::= 255

id-PrivacyIndicator										ProtocolIE-ID ::= 256

id-TraceCollectionEntityURI								ProtocolIE-ID ::= 257

id-NPN-Support											ProtocolIE-ID ::= 258

id-NPN-AccessInformation								ProtocolIE-ID ::= 259

id-NPN-PagingAssistanceInformation						ProtocolIE-ID ::= 260

id-NPN-MobilityInformation								ProtocolIE-ID ::= 261

id-TargettoSource-Failure-TransparentContainer			ProtocolIE-ID ::= 262

id-NID													ProtocolIE-ID ::= 263

id-UERadioCapabilityID									ProtocolIE-ID ::= 264

id-UERadioCapability-EUTRA-Format						ProtocolIE-ID ::= 265

id-DAPSRequestInfo									ProtocolIE-ID ::= 266

id-DAPSResponseInfoList								ProtocolIE-ID ::= 267

id-EarlyStatusTransfer-TransparentContainer 			ProtocolIE-ID ::= 268

id-NotifySourceNGRANNode									ProtocolIE-ID ::= 269

id-ExtendedSliceSupportList								ProtocolIE-ID ::= 270

id-ExtendedTAISliceSupportList							ProtocolIE-ID ::= 271

id-ConfiguredTACIndication								ProtocolIE-ID ::= 272

id-Extended-RANNodeName									ProtocolIE-ID ::= 273

id-Extended-AMFName										ProtocolIE-ID ::= 274

id-GlobalCable-ID										ProtocolIE-ID ::= 275

id-QosMonitoringReportingFrequency						ProtocolIE-ID ::= 276

id-QosFlowParametersList									ProtocolIE-ID ::= 277

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour					ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-SourceTNLAddrInfo										ProtocolIE-ID ::= 284

id-ExtendedReportIntervalMDT							ProtocolIE-ID ::= 285

id-SourceNodeID											ProtocolIE-ID ::= 286

id-NRNTNTAIInformation									ProtocolIE-ID ::= 287

id-UEContextReferenceAtSource							ProtocolIE-ID ::= 288

id-LastVisitedPSCellList								ProtocolIE-ID ::= 289

id-IntersystemSONInformationRequest						ProtocolIE-ID ::= 290

id-IntersystemSONInformationReply						ProtocolIE-ID ::= 291

id-EnergySavingIndication								ProtocolIE-ID ::= 292

id-IntersystemResourceStatusUpdate						ProtocolIE-ID ::= 293

id-SuccessfulHandoverReportList							ProtocolIE-ID ::= 294

id-MBS-AreaSessionID 									ProtocolIE-ID ::= 295

id-MBS-QoSFlowsToBeSetupList							ProtocolIE-ID ::= 296

id-MBS-QoSFlowsToBeSetupModList							ProtocolIE-ID ::= 297

id-MBS-ServiceArea 										ProtocolIE-ID ::= 298

id-MBS-SessionID 										ProtocolIE-ID ::= 299

id-MBS-DistributionReleaseRequestTransfer				ProtocolIE-ID ::= 300

id-MBS-DistributionSetupRequestTransfer 				ProtocolIE-ID ::= 301

id-MBS-DistributionSetupResponseTransfer				ProtocolIE-ID ::= 302

id-MBS-DistributionSetupUnsuccessfulTransfer			ProtocolIE-ID ::= 303

id-MulticastSessionActivationRequestTransfer			ProtocolIE-ID ::= 304

id-MulticastSessionDeactivationRequestTransfer			ProtocolIE-ID ::= 305

id-MulticastSessionUpdateRequestTransfer				ProtocolIE-ID ::= 306

id-MulticastGroupPagingAreaList							ProtocolIE-ID ::= 307

id-MBS-SupportIndicator									ProtocolIE-ID ::= 309

id-MBSSessionFailedtoSetupList							ProtocolIE-ID ::= 310

id-MBSSessionFailedtoSetuporModifyList					ProtocolIE-ID ::= 311

id-MBSSessionSetupResponseList							ProtocolIE-ID ::= 312

id-MBSSessionSetuporModifyResponseList					ProtocolIE-ID ::= 313

id-MBSSessionSetupFailureTransfer						ProtocolIE-ID ::= 314

id-MBSSessionSetupRequestTransfer						ProtocolIE-ID ::= 315

id-MBSSessionSetupResponseTransfer						ProtocolIE-ID ::= 316

id-MBSSessionToReleaseList								ProtocolIE-ID ::= 317

id-MBSSessionSetupRequestList 							ProtocolIE-ID ::= 318

id-MBSSessionSetuporModifyRequestList 					ProtocolIE-ID ::= 319

id-MBS-ActiveSessionInformation-SourcetoTargetList		ProtocolIE-ID ::= 323

id-MBS-ActiveSessionInformation-TargettoSourceList		ProtocolIE-ID ::= 324

id-OnboardingSupport									ProtocolIE-ID ::= 325

id-TimeSyncAssistanceInfo								ProtocolIE-ID ::= 326

id-SurvivalTime											ProtocolIE-ID ::= 327

id-QMCConfigInfo											ProtocolIE-ID ::= 328

id-QMCDeactivation										ProtocolIE-ID ::= 329

id-PDUSessionPairID										ProtocolIE-ID ::= 331

id-NR-PagingeDRXInformation								ProtocolIE-ID ::= 332

id-RedCapIndication										ProtocolIE-ID ::= 333

id-TargetNSSAIInformation								ProtocolIE-ID ::= 334

id-UESliceMaximumBitRateList							ProtocolIE-ID ::= 335

id-M4ReportAmount										ProtocolIE-ID ::= 336

id-M5ReportAmount										ProtocolIE-ID ::= 337

id-M6ReportAmount										ProtocolIE-ID ::= 338

id-M7ReportAmount										ProtocolIE-ID ::= 339

id-IncludeBeamMeasurementsIndication					ProtocolIE-ID ::= 340

id-ExcessPacketDelayThreshold									ProtocolIE-ID ::= 341

id-PagingCause											ProtocolIE-ID ::= 342

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized									ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-MBSSessionModificationFailureTransfer				ProtocolIE-ID ::= 348

id-MBSSessionModificationRequestTransfer				ProtocolIE-ID ::= 349

id-MBSSessionModificationResponseTransfer				ProtocolIE-ID ::= 350

id-MBS-QoSFlowToReleaseList								ProtocolIE-ID ::= 351

id-MBS-SessionTNLInfo5GC								ProtocolIE-ID ::= 352

id-TAINSAGSupportList									ProtocolIE-ID ::= 353

id-SourceNodeTNLAddrInfo									ProtocolIE-ID ::= 354

id-NGAPIESupportInformationRequestList					ProtocolIE-ID ::= 355

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer						ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Appendix B - CR for TS 38.423

3GPP TSG-RAN WG3 Meeting #117-e	R3-22xxxx

# Online, 15th August – 24th August, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XnAP Corrections related to Excess Packet Delay based on RAN2 status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current Excess Packet Delay specifications in RAN3 are not in sync with the RAN2 agreements on the subject. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During the previous RAN3#115-e meeting it has been agreed to change the current M6 configuration IE by introducing the M6 Delay Threshold IE to enable the calculation of the PDCP Excess Packet Delay in the UL. Nevertheless, in RAN2, excess packet delay is viewed as a new measurement and is not directly coupled to M6. Namely, RAN2’s conclusions are that the M6 delay measurement should be provided as per legacy specifications, but additionally excess packet delay measurements may be added to the M6 measurement report. In 38.314, section 4.3.1.2, the PDCP Excess Packet Delay is defined as follows: PDCP Excess Packet Delay in the UL per DRB. It represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. The RAN2 agreement does not prohibit using the M6 configuration IE to communicate the Excess Packet Delay Threshold IE. Furthermore, RAN2 has agreed that for UL PDCP Excess Packet Delay (related to section 4.3.1.2 in TS 38.314) different delay threshold values for different DRBs are to be used. These different delay threshold values need to be provided by the CN/OAM while the RAN would be responsible for mapping the provided values to the relevant DRBs as these are under the RAN’s responsibility. Currently only a single delay threshold value is communicated to the RAN through the M6 Configuration. A solution to this problem it to enable multiple threshold parameters in the renamed Excess Packet Delay Threshold IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact with the previous version of the specification. There is limited impact in the ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN2 and RAN3 specifications are not in sync while RAN3 specification will fail to provide functionality already assumed by agreed RAN2 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.131, 9.2.3.x, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS38.413 CRxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.131	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120,ms240,ms480,ms640,ms1024, ms2048, ms5120, ms10240, ms20480,ms40960,min1,min6,min12,min30, …) |  | – |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  | – |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity …) | Number of reports. | YES | ignore |
| Excess Packet Delay Threshold | O |  | 9.2.3.x | Indicates that excess packet delay measurements need to be reported, as specified in TS 38.314 [42], additionally to M6 measurements | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	Excess Packet Delay Threshold

This IE defines the parameters for Excess Packet Delay Threshold configuration in order to enable the calculation of the PDCP Excess Packet Delay in the UL per DRB as specified in TS 38.314 [42].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Excess Packet Delay Threshold |  | 1..<maxnoofThresholds> |  |  |
| >Excess Packet Delay Threshold Value | M |  | ENUMERATED (ms0.25, ms0.5, ms1, ms2, ms4, ms10, ms20, ms50, ms100, ms500, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of Thresholds allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< End of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element Definitions

-- TEXT OMITTED –

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT 		PRESENCE optional		}|

{ ID id-ExcessPacketDelayThreshold	CRITICALITY ignore	EXTENSION ExcessPacketDelayThreshold		PRESENCE optional		},

...

}

ExcessPacketDelayThreshold ::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF ExcessPacketDelay-thresholdItem

ExcessPacketDelay-thresholdItem ::= SEQUENCE {

excessPacketDelay-thresholdValue			ExcessPacketDelay-thresholdValue,

iE-Extensions		ProtocolExtensionContainer { { ExcessPacketDelay-thresholdItem-ExtIEs} }	OPTIONAL,

...

}

ExcessPacketDelay-thresholdItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExcessPacketDelay-thresholdValue ::= ENUMERATED {

ms0.25, ms0.5, ms1, ms2, ms4, ms10, ms20, ms50, ms100, ms500,

...

}

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< End of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- TEXT OMITTED –

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs							INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation				INTEGER ::= 256

maxnoofTAIforMBS 							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN 						INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs 							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofThresholds							INTEGER ::= 64

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered																						ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-EUTRAPagingeDRXInformation																		ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-DirectForwardingPathAvailability																	ProtocolIE-ID ::= 253

id-SourceNG-RAN-node-ID																				ProtocolIE-ID ::= 254

id-SourceDLForwardingIPAddress																		ProtocolIE-ID ::= 255

id-SourceNodeDLForwardingIPAddress																	ProtocolIE-ID ::= 256

id-ExtendedReportIntervalMDT																			ProtocolIE-ID ::= 257

id-SecurityIndication																				ProtocolIE-ID ::= 258

id-RRCConnReestab-Indicator																			ProtocolIE-ID ::= 259

id-TargetNodeID																						ProtocolIE-ID ::= 260

id-ManagementBasedMDTPLMNList																		ProtocolIE-ID ::= 261

id-PrivacyIndicator																					ProtocolIE-ID ::= 262

id-TraceCollectionEntityIPAddress																	ProtocolIE-ID ::= 263

id-M4ReportAmount																					ProtocolIE-ID ::= 264

id-M5ReportAmount																					ProtocolIE-ID ::= 265

id-M6ReportAmount																					ProtocolIE-ID ::= 266

id-M7ReportAmount																					ProtocolIE-ID ::= 267

id-BeamMeasurementIndicationM1																		ProtocolIE-ID ::= 268

id-MBS-Session-ID																					ProtocolIE-ID ::= 269

id-UEIdentityIndexList-MBSGroupPaging																	ProtocolIE-ID ::= 270

id-MulticastRANPagingArea																			ProtocolIE-ID ::= 271

id-Supported-MBS-FSA-ID-List																		ProtocolIE-ID ::= 272

id-MBS-SessionInformation-List																		ProtocolIE-ID ::= 273

id-MBS-SessionInformationResponse-List																ProtocolIE-ID ::= 274

id-MBS-SessionAssociatedInformation																	ProtocolIE-ID ::= 275

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= 276

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= 277

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= 278

id-SCGUEHistoryInformation																			ProtocolIE-ID ::= 279

id-SSBOffsets-List																					ProtocolIE-ID ::= 280

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= 281

id-Coverage-Modification-List																		ProtocolIE-ID ::= 282

id-NR-U-Channel-List																				ProtocolIE-ID ::= 283

id-SourcePSCellCGI																					ProtocolIE-ID ::= 284

id-FailedPSCellCGI																					ProtocolIE-ID ::= 285

id-SCGFailureReportContainer																		ProtocolIE-ID ::= 286

id-SNMobilityInformation																			ProtocolIE-ID ::= 287

id-SourcePSCellID																					ProtocolIE-ID ::= 288

id-SuitablePSCellCGI																				ProtocolIE-ID ::= 289

id-PSCellChangeHistory																				ProtocolIE-ID ::= 290

id-CHOConfiguration																					ProtocolIE-ID ::= 291

id-NR-U-ChannelInfo-List																			ProtocolIE-ID ::= 292

id-PSCellHistoryInformationRetrieve																	ProtocolIE-ID ::= 293

id-NG-RANnode2SSBOffsetsModificationRange															ProtocolIE-ID ::= 294

id-MIMOPRBusageInformation																			ProtocolIE-ID ::= 295

id-F1CTrafficContainer																				ProtocolIE-ID ::= 296

id-IAB-MT-Cell-List																					ProtocolIE-ID ::= 297

id-NoPDUSessionIndication																			ProtocolIE-ID ::= 298

id-IAB-TNL-Address-Request																			ProtocolIE-ID ::= 299

id-IAB-TNL-Address-Response																			ProtocolIE-ID ::= 300

id-TrafficToBeAddedList																				ProtocolIE-ID ::= 301

id-TrafficToBeModifiedList																			ProtocolIE-ID ::= 302

id-TrafficToBeReleaseInformation																	ProtocolIE-ID ::= 303

id-TrafficAddedList																					ProtocolIE-ID ::= 304

id-TrafficModifiedList																				ProtocolIE-ID ::= 305

id-TrafficNotAddedList																				ProtocolIE-ID ::= 306

id-TrafficNotModifiedList																			ProtocolIE-ID ::= 307

id-TrafficRequiredToBeModifiedList																	ProtocolIE-ID ::= 308

id-TrafficRequiredModifiedList																		ProtocolIE-ID ::= 309

id-TrafficReleasedList																				ProtocolIE-ID ::= 310

id-IABTNLAddressToBeAdded																			ProtocolIE-ID ::= 311

id-IABTNLAddressToBeReleasedList																	ProtocolIE-ID ::= 312

id-nonF1-Terminating-IAB-DonorUEXnAPID																ProtocolIE-ID ::= 313

id-F1-Terminating-IAB-DonorUEXnAPID																	ProtocolIE-ID ::= 314

id-BoundaryNodeCellsList																			ProtocolIE-ID ::= 315

id-ParentNodeCellsList																				ProtocolIE-ID ::= 316

id-tdd-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 317

id-UL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 318

id-DL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 319

id-permutation																						ProtocolIE-ID ::= 320

id-IABTNLAddressException																			ProtocolIE-ID ::= 321

id-CHOinformation-AddReq																			ProtocolIE-ID ::= 322

id-CHOinformation-ModReq																			ProtocolIE-ID ::= 323

id-SurvivalTime																						ProtocolIE-ID ::= 324

id-TimeSynchronizationAssistanceInformation															ProtocolIE-ID ::= 325

id-SCGActivationRequest																				ProtocolIE-ID ::= 326

id-SCGActivationStatus																				ProtocolIE-ID ::= 327

id-CPAInformationRequest																			ProtocolIE-ID ::= 328

id-CPAInformationAck																				ProtocolIE-ID ::= 329

id-CPCInformationRequired																			ProtocolIE-ID ::= 330

id-CPCInformationConfirm																			ProtocolIE-ID ::= 331

id-CPAInformationModReq																				ProtocolIE-ID ::= 332

id-CPAInformationModReqAck																			ProtocolIE-ID ::= 333

id-CPC-DataForwarding-Indicator																		ProtocolIE-ID ::= 334

id-CPCInformationUpdate																				ProtocolIE-ID ::= 335

id-CPACInformationModRequired																		ProtocolIE-ID ::= 336

id-QMCConfigInfo																					ProtocolIE-ID ::= 337

id-ProtocolIE-ID338-NotToBeUsed																		ProtocolIE-ID ::= 338

id-Additional-Measurement-Timing-Configuration-List													ProtocolIE-ID ::= 339

id-PDUSession-PairID																					ProtocolIE-ID ::= 340

id-Local-NG-RAN-Node-Identifier																		ProtocolIE-ID ::= 341

id-Neighbour-NG-RAN-Node-List																		ProtocolIE-ID ::= 342

id-Local-NG-RAN-Node-Identifier-Removal																ProtocolIE-ID ::= 343

id-FiveGProSeAuthorized																				ProtocolIE-ID ::= 344

id-FiveGProSePC5QoSParameters																		ProtocolIE-ID ::= 345

id-FiveGProSeUEPC5AggregateMaximumBitRate															ProtocolIE-ID ::= 346

id-ServedCellSpecificInfoReq-NR																		ProtocolIE-ID ::= 347

id-NRPagingeDRXInformation																			ProtocolIE-ID ::= 348

id-NRPagingeDRXInformationforRRCINACTIVE																ProtocolIE-ID ::= 349

id-Redcap-Bcast-Information																			ProtocolIE-ID ::= 350

id-SDTSupportRequest																				ProtocolIE-ID ::= 351

id-SDT-SRB-between-NewNode-OldNode																	ProtocolIE-ID ::= 352

id-SDT-Termination-Request																			ProtocolIE-ID ::= 353

id-SDTPartialUEContextInfo																			ProtocolIE-ID ::= 354

id-SDTDataForwardingDRBList																			ProtocolIE-ID ::= 355

id-PagingCause																						ProtocolIE-ID ::= 356

id-PEIPSassistanceInformation																		ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList																			ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR																			ProtocolIE-ID ::= 359

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-ExcessPacketDelayThreshold																		ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>