3GPP TSG-RAN WG3 Meeting #117-e	R3-224961

# E-meeting, 15 – 25 August 2022

Agenda item:		12.2.2

Source:	ZTE

Title:	Discussion on the standard impacts of load prediction across multiple AI/ML based use cases

Document for:		Discussion and Decision

Introduction

The following is the objectives of the R18 AI-RAN WI:

Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows:

•	Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

Note: On security impacts, coordination with SA3 when needed. On OAM aspects, coordination with SA5 when needed.

Note: Specify new UE measurements when needed if any.

Note: Specify MDT procedure enhancements when needed if any.

In this contribution, we focus on the standard impacts of the common predicted information across multiple use cases, (e.g., load prediction, UE trajectory prediction), and provide the analysis of two options for standard impacts.

Discussion

Load prediction

Following are input, output, and feedback information involved in the each AI/ML based use cases:

|  | Network Energy Saving | Load Balancing | Mobility Optimization |
| --- | --- | --- | --- |
| Input | From local node: - UE mobility/trajectory prediction - Current/Predicted Energy efficiency - Current/Predicted resource status From the UE: - UE location information - UE measurement report From neighbouring NG-RAN nodes: - Current/Predicted energy efficiency - Current/Predicted resource status - Current energy state | From the local node: - Current and predicted own resource status - UE trajectory prediction - Current and predicted UE traffic - Predicted resource status information of neighbouring NG-RAN node(s) From the UE: - UE location information - UE Mobility History Information - UE measurement report From neighbouring NG-RAN Nodes: - Current and predicted resource status - UE performance measurement at traffic offloaded neighbouring cell | From the UE: - UE location information - Radio measurements related to serving cell and neighbouring cells - UE Mobility History Information. From the neighbouring RAN nodes: - UE’s history information from neighbour - Position, QoS parameters and the performance information of historical HO-ed UE - Current/predicted resource status - UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism. From the local node: - UE trajectory prediction - Current/predicted resource status - Current/predicted UE traffic |
| Output | - Energy saving strategy, such as recommended cell activation/deactivation. - Handover strategy, including recommended candidate cells for taking over the traffic - Predicted energy efficiency - Predicted energy state (e.g., active, high, low, inactive) - Model output validity time | - Selection of target cell for load balancing - Predicted own resource status information - Predicted resource status information of neighbouring NG-RAN node(s) - Model output validity time - The predicted UE(s) selected to be handed over to target NG-RAN node | - UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time) - Estimated arrival probability in CHO and relevant confidence interval - Predicted handover target node, candidate cells in CHO - Priority, handover execution timing, predicted resource reservation time window for CHO. - UE traffic prediction - Model output validity time |
| Feedback | - Resource status of neighbouring NG-RAN nodes - Energy efficiency - UE performance affected by the energy saving action - System KPIs | - UE performance information from target NG-RAN - Resource status information updates from target NG-RAN - System KPIs | - QoS parameters - Resource status information updates from target NG-RAN. - Performance information from target NG-RAN. |

Based on the yellow-highlighted information, load prediction is commonly used for input information, output information, and feedback information. That is to say, to define the load prediction as input information/output information/feedback information over interface needs to be specified.

Proposal 1: Load prediction should be considered as the common information for AI/RAN to be specified.

There are two options to specify the load prediction:

Option 1: Define the new unified procedures for AI/ML related information.

Option 2: Enhance the existing signaling procedure for AI/ML related information.

We provide our analysis of option1 and option2 for each scenarios for load prediction.

Scenario 1: The NG-RAN nodes perform model training/model inference for load prediction locally (NG-RAN node1 collects predicted load from other NG-RAN nodes).

Figure 1. Each NG-RAN nodes perform load prediction locally

In the scenario1, the information transferred between NG-RAN node1 and NG-RAN node2 is predicted load information.

Step1. NG-RAN node1 collects the historical load information locally.

Step2. NG-RAN node1 request the predicted load information of NG-RAN node2.

Step3~Step4. NG-RAN node2 performs model inference to achieve predicted load information.

Step 5. NG-RAN node2 response the predicted load information through response message.

Step 6~Step 7. NG-RAN node1 performs model training/model inference through the local load information and predicted load information. And after the model inference, each NG-RAN nodes evaluate model performance locally.

For the predicted information, we can define a new predicted information request message as shown in the Figure1. NG-RAN node1 send the request message to NG-RAN node2 to indicate the predicted load information from target RAN node, and NG-RAN node1 perform the model training and model inference. Or enhance the existing procedure to include the predicted load information.

Proposal 2: For the scenario1, two options could be chosen:

- Define the AI/ML prediction information procedure to include predicted load information.

- Enhance existing resource status procedure to include predicted load information.

Scenario 2: The NG-RAN node1 performs model training/model inference for load prediction locally, and collect historical load information from other NG-RAN nodes.

Figure 2. Scenario2: NG-RAN node1 performs load prediction including predicted load of the neighbor NG-RAN nodes

Step1. NG-RAN node1 collects the historical load information locally.

Step2. NG-RAN node1 requests the historical load information of neighbor NG-RAN nodes.

Step3~Step4. NG-RAN node2 collects the historical load information locally, and response to the NG-RAN node1.

Step 5. NG-RAN node1 performs model training/model inference through the local load information and predicted load information.

Step 6~Step 7. NG-RAN node1 request feedback information through request/response message.

In the scenario2, NG-RAN node1 needs to collect the historical load information from neighbor NG-RAN nodes to perform model training/model inference to obtain the predicted load information of neighbor NG-RAN nodes. As we know, AI/ML model (e.g., time-series related model, LSTM) needs to leverage the historical information over a period information and analyze/learn the distribution of historical information to predict information. However, current resource status procedure cannot support to report the historical information, since the current procedure adopts one-time reporting type to carry current load information, or periodic reporting type. It needs to be specified that the NG-RAN node is able to request the historical load information from neighbor NG-RAN nodes.

In order to involve the historical load information, following are two options:

- Define the AI/ML data collection information procedure

- Enhance the existing resource status procedure

Proposal 3: NG-RAN node should be able to request the historical load information from neighbor NG-RAN nodes through new procedure (AI/ML data collection information procedure) or the existing procedure.

In addition to the data collection procedure, since NG-RAN node1 performs model inference and achieve the predicted load information of neighboring NG-RAN nodes. NG-RAN node1 should be able to request the actual load information to evaluate the model performance, so the feedback information include the actual load information should be considered.

In order to involve the actual load information, following are two options:

- Define the AI/ML feedback information procedure

- Reuse the existing resource status procedure

Proposal 4: NG-RAN node should be able to request the feedback information, including actual load information, from neighbor NG-RAN nodes through new procedure (AI/ML feedback information procedure) or the existing procedure.

Above all, compared with Option2 (enhance existing procedure), Option1 (define new procedure) seems much clearer and simpler. We can clearly differentiate data collection procedure, predicted information procedure and feedback procedure for the AI/ML function, which corresponds to input, output, and feedback information respectively. These procedure could be leveraged across multiple AI/ML based use cases.

And, for the future extension, we just needs to identify the common predicted information, and involve these prediction information into this unified AI/ML procedure, instead of finding a proper existing procedure to involve a new identified information for AI/ML based use cases.

Proposal 5: Prefer to adopt Option1 to transfer load prediction related information:

- AI/ML data collection procedure

- AI/ML predicted information procedure

- AI/ML feedback information procedure

In addition to the impacts to Xn interface, load prediction related information over F1 interface also needs to be considered. Following are the scenarios in the case of CU-DU split architecture:

Scenario 3: The gNB-CU performs Model Inference and the gNB-CU needs to collect the historical load information from the gNB-DU.

Figure 3. Scenario3: CU performs Model Training/Inference

Step1. The gNB-CU requests the historical load information of gNB-DU(s).

Step2~3. The gNB-DU(s) collect(s) the historical load information locally, and response(s) to the gNB-CU.

Step4. The gNB-CU performs Model training or Model inference.

Step 5~Step 6. NG-RAN node1 request feedback information to retrieve actual load information through the request/response message.

In order to involve the historical load information in the case of CU/DU split architecture, following are two options:

- Define the AI/ML data collection information procedure

- Enhance the existing resource status procedure

Proposal 6: The gNB-CU should be able to request the historical load information from gNB-DU(s) through new procedure (AI/ML data collection information procedure) or the existing procedure.

In order to involve the actual load information in the case of CU/DU split architecture, following are two options:

- Define the AI/ML feedback information procedure

- Reuse the existing resource status procedure

Proposal 7: The gNB-CU should be able to request the feedback information, including actual load information, from gNB-DU(s) through new procedure (AI/ML feedback information procedure) or the existing procedure.

Scenario 4: The gNB-DU performs Model Inference and the gNB-CU can request the predicted load information from gNB-DU(s).

Figure 4. Scenario4: DU performs Model Training/Inference

Step1. The gNB-CU requests the predicted load information of the gNB-DU(s).

Step2~Step3. The gNB-DU(s) collect(s) historical load information locally and performs model inference to achieve predicted load information.

Step 4. The gNB-DU(s) responses the predicted load information through response message.

Step 5. The gNB-DU(s) collect(s) actual load information and evaluates model performance locally.

Proposal 8: Following are options for the predicted load information in the case of CU/DU architecture:

- Define the AI/ML prediction information procedure to include predicted load information.

- Enhance existing resource status procedure to include predicted load information.

Network Energy Saving

During the SI phase, AI/ML based Network Energy Saving is the high-prioritized use cases. The following are output information of AI/ML based Network Energy Saving:

-	Energy saving strategy, such as recommended cell activation/deactivation.

-	Handover strategy, including recommended candidate cells for taking over the traffic

-	Predicted energy efficiency

-	Predicted energy state (e.g., active, high, low, inactive)

-	Model output validity time will be discussed during R18 normative work per inference output.

Since another topic on network energy saving will be discussed in RAN during R18, it may discussed the details of energy efficiency and energy state. Hence, for the AI/ML based network energy saving, we proposed to focus on the energy saving strategy per cell level in RAN3.

Moreover, network energy saving is a system-level use case, it will dynamically configure energy saving parameters according to the related information (e.g., predicted load information, predicted UE trajectory prediction, etc.). We suggest to focus on the AI/ML procedure to transfer these information for energy saving. And recommended cell activation/deactivation could be one kind of information transferred over Xn/F1 interface.

Proposal 9: For AI/ML based energy saving, RAN should focus on the energy saving strategy per cell level (e.g., cell activation/cell deactivation).

Conclusion

We propose the following observations and proposals:

Proposal 1: Load prediction should be considered as the common information for AI/RAN to be specified.

Proposal 2: For the scenario1, two options could be chosen:

- Define the prediction information procedure to include predicted load information.

- Enhance existing resource status procedure to include predicted load information.

Proposal 3: NG-RAN node should be able to request the historical load information from neighbor NG-RAN nodes through new procedure (AI/ML data collection information procedure) or the existing procedure.

Proposal 4: NG-RAN node should be able to request the feedback information, including actual load information, from neighbor NG-RAN nodes through new procedure (AI/ML feedback information procedure) or the existing procedure.

Proposal 5: Prefer to adopt Option1 to transfer load prediction related information:

- AI/ML data collection procedure

- AI/ML predicted information procedure

- AI/ML feedback information procedure

Proposal 6: The gNB-CU should be able to request the historical load information from gNB-DU(s) through new procedure (AI/ML data collection information procedure) or the existing procedure.

Proposal 7: The gNB-CU should be able to request the feedback information, including actual load information, from gNB-DU(s) through new procedure (AI/ML feedback information procedure) or the existing procedure.

Proposal 8: Following are options for the predicted load information in the case of CU/DU architecture:

- Define the AI/ML prediction information procedure to include predicted load information.

- Enhance existing resource status procedure to include predicted load information.

Proposal 9: For AI/ML based energy saving, RAN should focus on the energy saving strategy per cell level (e.g., cell activation/cell deactivation).

- References

- TR37.817, Study on enhancement for Data Collection for NR and EN-DC