3GPP TSG-RAN WG3 Meeting #117-e	R3-224963

# E-meeting, 15 – 25 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0896 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unified AI/ML procedure across multiple use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR37.817, it can be acknowledged that load prediction and UE trajectory prediction is the common information as input information so that these two prediction could be considered as one kind of tool box, which be used for the multiple use cases, not limited to three use cases (e.g., Energy Saving, Load Balancing, and Mobility Optimization). Hence, it is proposed to define the unfied procedure across multiple AI/ML use cases, e.g, AI/ML data collection procedure, AI/ML predicted procedure, and AI/ML feedback procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the AI/ML Data Collection Transfer procedure - Introduce the AI/ML Predicted Information Transfer procedure - Introduce the AI/ML Feedback Information Transfer procedure - Introduce Historical Resource Status List IE within the Data Collection Transfer procedure - Introduce Predicted UE trajectory List IE and Predicted resource status IE within the AI/ML Predicted Information Transfer procedure - Introduce Actual UE trajectory IE and Actual resource status IE within the AI/ML Feedback Information Transfer procedure - Introduce Start Time and Duration IE within the AI/ML request message Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML funtction can not be supported over XN interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<Unchanged Text Omitted>

XnAP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Data Collection Transfer | AI/ML Data Collection Request | AI/ML Data Collection Response | AI/ML Data Collection FAILURE |
| AI/ML Predicted Information Transfer | AI/ML Predicted Information Request | AI/ML Predicted Information Response | AI/ML Predicted Information Failure |
| AI/ML Feedback Information Transfer | AI/ML FEEDBACK INFORMATION REQUEST | AI/ML FEEDBACK INFORMATION RESPONSE | AI/ML FEEDBACK INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI/ML Data Collection Report | AI/ML DATA COLLECTION REPORT |
| AI/ML Predicted Information Report | AI/ML PREDICTED INFORMATION REPORT |
| AI/ML Feedback Information Report | AI/ML Feedback Information Report |

<Unchanged Text Omitted>

8.x	AI/ML Procedures

8.x.1	AI/ML Data Collection Transfer

8.x.1.1	General

This procedure is used by an NG-RAN node to request the reporting of collected AI/ML data used for AI/ML funtion to another NG-RAN node.

8.x.1.2	Successful Operation

Figure 8.x.1.2.1: AI/ML Data Collection Transfer Procedure: Successful Operation

NG-RAN node1 initiates the procedure by sending the AI/ML DATA COLLECTION REQUEST message to NG-RAN node2 to request the AI/ML information.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node2 node shall return the corresponding historical measurement results in the AI/ML DATA COLLECTION RESPONSE message, and the NG-RAN node1 shall consider that this reporting has been terminated by the NG-RAN node2. If the Report Characteristics IE is set to "Periodic", the NG-RAN node2 shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node2 shall then periodically initiate the AI/ML DATA COLLECTION REPORT procedure for the corresponding measurements, with the requested reporting periodicity.

If the Report Information Type IE is included in the AI/ML DATA COLLECTION REQUEST message, the NG-RAN node2 shall, if supported, include the requested information in the AI/ML DATA COLLECTION RESPONSE message.

8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3.1: AI/ML Data Collection Transfer Procedure: Unsuccessful Operation

If any of the requested measurements cannot be initiated or configured, NG-RAN node2 shall send the AI/ML DATA COLLECTION FAILURE message with an appropriate cause value.

8.x.1.4	Abnormal Conditions

Not applicable

8.x.2	AI/ML Data Collection Report

8.x.2.1	General

This procedure is initiated by an NG-RAN node2 to report the result of measurements admitted by the NG-RAN node1 following a successful AI/ML DATA COLLECTION REPORT procedure.

8.x.2.2	Successful Operation

Figure 8.x.2.2.1: AI/ML Data Collection Report Procedure: successful operation

NG-RAN node2 shall report the results of the admitted measurements in AI/ML DATA COLLECTION REPORT message. The admitted measurements are the measurements that were successfully initiated during the AI/ML Data Collection Information Transfer procedure.

8.x.2.3	Unsuccessful Operation

Not applicable.

8.x.2.4	Abnormal Conditions

Void

8.x.3	AI/ML Predictied Imformation Transfer

8.x.3.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML prediction information used for AI/ML funtion to another NG-RAN node.

8.x.3.2	Successful Operation

Figure 8.x.3.2.1: AI/ML Prediction Information Procedure: Successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML PREDICTION INFORMATION REQUEST message to NG-RAN node2 to request the AI/ML prediction information.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node2 node shall return the corresponding historical measurement results in the AI/ML DATA COLLECTION RESPONSE message, and the NG-RAN node1 shall consider that this reporting has been terminated by the NG-RAN node2. If the Report Characteristics IE is set to "Periodic", the NG-RAN node2 shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node2 shall then periodically initiate the AI/ML DATA COLLECTION REPORT procedure for the corresponding measurements, with the requested reporting periodicity.

If the Report Information Type IE is included in the AI/ML DATA COLLECTION REQUEST message, the NG-RAN node2 shall, if supported, include the requested information in the AI/ML DATA COLLECTION RESPONSE message.

8.x.3.3	Unsuccessful Operation

Figure 8.x.3.3.1: AI/ML Prediction Information Procedure: Unsuccessful operation

If any of the requested measurements cannot be initiated or configured, NG-RAN node2 shall send the AI/ML PREDICTION INFORMATION FAILURE message with an appropriate cause value.

8.x.3.4	Abnormal Conditions

Not applicable

8.x.4	AI/ML Predictied Information Report

8.x.4.1	General

This procedure is initiated by an NG-RAN node2 to report the result of measurements admitted by the NG-RAN node1 following a successful AI/ML PREDICTION INFORMATION REPORT procedure.

8.x.4.2	Successful Operation

Figure 8.x.5.2-1: AI/ML Prediction Information Report: successful operation

NG-RAN node2 shall report the results of the admitted measurements in AI/ML PREDICTION INFORMATION REPORT message. The admitted measurements are the measurements that were successfully initiated during the AI/ML Prediction Information Transfer procedure.

8.x.4.3	Unsuccessful Operation

Not applicable.

8.x.4.4	Abnormal Conditions

Void

8.x.5	AI/ML Feedback Imformation Transfer

8.x.5.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML feedback information used for AI/ML funtion to another NG-RAN node.

8.x.5.2	Successful Operation

Figure 8.x.5.2.1: AI/ML Feedback Imformation Transfer Procedure: Successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML FEEDBACK INFORMATION REQUEST message to NG-RAN node2 to request the AI/ML feedback information.

8.x.5.3	Unsuccessful Operation

Figure 8.x.5.3.1: AI/ML Feedback Imformation Transfer Procedure: Unsuccessful operation

If any of the requested measurements cannot be initiated or configured, NG-RAN node2 shall send the AI/ML FEEDBACK INFORMATION FAILURE message with an appropriate cause value.

8.x.5.4	Abnormal Conditions

Not applicable

8.x.6	AI/ML Feedback Information Report

8.x.6.1	General

This procedure is initiated by an NG-RAN node2 to report the result of measurements admitted by the NG-RAN node1 following a successful AI/ML FEEDBACK INFORMATION REPORT procedure.

8.x.6.2	Successful Operation

Figure 8.x.11.2-1: Resource Status Reporting, successful operation

NG-RAN node2 shall report the results of the admitted measurements in AI/ML FEEDBACK INFORMATION REPORT message. The admitted measurements are the measurements that were successfully initiated during the AI/ML Feedback Information Transfer procedure.

8.x.6.3	Unsuccessful Operation

Not applicable.

8.x.6.4	Abnormal Conditions

Void

<Unchanged Text Omitted>

9.1.Y	Messages for AI/ML Procedures

9.1.Y.1	AI/ML DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| Report Characteristics | M |  | ENUMERATED (on-demand, Periodic, …) |  | YES | reject |
| Report Information Type | M |  | ENUMERATED (resource status, …) |  | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Start time and Duration | O |  | 9.2.3.X |  | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.Y.2	AI/ML DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested measurement is successfully initiated, or to report the requested AI/ML measurments.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Resource Status Measurement Response List |  | 0..1 |  |  | YES | reject |
| > Resource Status Measurement Response Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Historical Resource Status Item |  | 1 .. <maxnoofResMeas> |  |  | – |  |
| >>> Radio Resource Status |  |  | 9.2.2.50 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofResMeas | Maximum no. of histroical resource status that can be configured and reported with one response message. Value is 16384. |

9.1.Y.3	AI/ML DATA COLLECTION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.Y.4	AI/ML DATA COLLECTION REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.Y.5	AI/ML PREDICTIED REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| Report Characteristics | M |  | ENUMERATED (on-demand, Periodic, …) |  | YES | reject |
| Report Prediction Information Type | M |  | ENUMERATED (resource status, UE trajectory…) |  | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Start time and Duration | O |  | 9.2.3.X |  | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.Y.6	AI/ML PREDICTIED RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested prediction information is successfully initiated, or to report the requested AI/ML prediction information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Resource Status Prediction List |  | 0..1 |  |  | YES | reject |
| > Resource Status Prediction Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| UE Trajectory Prediction |  | 0..1 |  |  |  |  |
| >Predicted UE Trajectory Item |  | 1..<maxnoofCellsUEMovingTrajectory> |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofCellsUEMovingTrajectory | Maximum no. of cells of UE moving trajectory. Value is 16. |

9.1.Y.7	AI/ML PREDICTIED FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested prediction information cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.Y.8	AI/ML PREDICTIED REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested prediction information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Resource Status Prediction List |  | 0..1 |  |  | YES | ignore |
| > Resource Status Prediction Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.Y.9	AI/ML FEEDBACK REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested feedback information according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| Report Characteristics | M |  | ENUMERATED (on-demand, Periodic, …) |  | YES | reject |
| Report Feedback Information Type | M |  | ENUMERATED (resource status, UE trajectory…) |  | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Start time and Duration | O |  | 9.2.3.X |  | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.Y.10	AI/ML FEEDBACK RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested feedback information is successfully initiated, or to report the requested AI/ML feedback information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Resource Status Measurement List |  | 0..1 |  |  | YES | reject |
| > Resource Status Measurement Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| Actual UE Trajectory |  | 0..1 |  |  |  |  |
| >Actual UE Trajectory Item |  | 1..<maxnoofCellsUEMovingTrajectory> |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofCellsUEMovingTrajectory | Maximum no. of cells of UE moving trajectory. Value is 16. |

9.1.Y.11	AI/ML FEEDBACK FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested feedback information cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.Y.12	AI/ML FEEDBACK REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested prediction information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG-RAN node1 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated by NG-RAN node2 | YES | reject |
| Resource Status Measurement List |  | 0..1 |  |  | YES | ignore |
| > Resource Status Measurements Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<Unchanged Text Omitted>

9.2	Information Element definitions

<Unchanged Text Omitted>

9.2.3	General IE definitions

9.2.3.X	Start Time and Duration

This IE contains the start time and/or duration for the validity time for AI/ML related information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Start Time | O |  | Relative Time 1900 9.2.36 |  |
| Duration | O |  | INTEGER (0..90060, …) | Unit: seconds |

Change Ends