3GPP TSG-RAN WG3 Meeting #116-e	R3-224977

# E-meeting, 15 Aug - 24 Aug 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1718 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TS 36.423 on UHI in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2022-7-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In 8.7.7.2, it is stated that if the SCG UE History Information IE is included in the SN modification required message, the MN shall use this information as specified in TS 37.340. However, in 8.7.8.2, 8.7.9.2 and 8.7.10.2, it is described that if the SCG UE History Information IE is included in the SN change required, SN release request acknowledge and SN release required messages, the MN shall use the information to update UE history information. - The descriptions in 8.7.8.2, 8.7.9.2 and 8.7.10.2 are not complete, since the MN shall not only use the information to update the SN UHI but also correlates the MN and SN UHI as specified in TS 37.340. Thus it is required to align the description in 8.7.8.2, 8.7.9.2 and 8.7.10.2 with the description in 8.7.7.2. - In 9.2.177, the description is not aligned with that of XnAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 8.7.8.2, 8.7.9.2 and 8.7.10.2, clarify the behavior of MN after receiving the SN UHI information by aligning the behavior description with the description in 8.7.7.2. - Change the description In 9.2.177 to keep align with that of XnAP. Impact assessment towards the previous version of the specification (same release): This CR has no impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The MN behavior description is missing and not aligned with the description of the other messages including the SCG UE History Information IE. - The description in 9.2.177 is not aligned with that of XnAP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.8.2, 8.7.9.2, 8.7.10.2, 9.2.177 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

#### 8.7.8.2	Successful Operation

Figure 8.7.8.2-1: SgNB Change, successful operation.

The en-gNB initiates the procedure by sending the SGNB CHANGE REQUIRED message to the MeNB including the Target SgNB ID Information IE. When the en-gNB sends the SGNB CHANGE REQUIRED message, it shall start the timer TDCoverall.

The SGNB CHANGE REQUIRED message may contain

-	the SgNB to MeNB Container IE.

If the MeNB is able to perform the change requested by the en-gNB, the MeNB shall send the SGNB CHANGE CONFIRM message to the en-gNB. For each E-RAB configured with the PDCP entity in the en-gNB, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer.

The en-gNB may start data forwarding and stop providing user data to the UE and shall stop the timer TDCoverall upon reception of the SGNB CHANGE CONFIRM message.

If the SCG UE History Information IE is included in the SGNB CHANGE REQUIRED message, the MeNB node shall, if supported, use this information as specified in TS 37.340 [32].

If the Conditional PSCell Change Information Required IE is included in the SGNB CHANGE REQUIRED message, the MeNB shall, if supported, consider that the requirement concerns CPAC, as described in TS 37.340 [32]. The SgNB to MeNB Container IE within the Conditional PSCell Change Information Required IE contains at least the suggested PSCell list for each candidate target SgNB. Accordingly, the MeNB may include the Conditional PSCell Change Information Confirm IE in the SGNB CHANGE CONFIRM message.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the SGNB CHANGE REQUIRED message, the MeNB shall, if supported, forward this information to the candidate target en-gNB, then the candidate target en-gNB may use the information to allocate necessary resources for the incoming CPAC procedure.

Interaction with MeNB initiated SgNB Release:

If the MeNB receives the SGNB CHANGE REQUIRED message releasing target en-gNB and cancelling all the prepared PSCells in the target en-gNB(s), the MeNB shall, if supported, trigger the MeNB initiated en-gNB release procedure to the target en-gNB(s) and cancel all the prepared PSCells at the target en-gNB(s).

-----------------------------------------------------------Next change-----------------------------------------------------

#### 8.7.9.2	Successful Operation

Figure 8.7.9.2-1: MeNB initiated SgNB Release, successful operation

The MeNB initiates the procedure by sending the SGNB RELEASE REQUEST message. Upon reception of the SGNB RELEASE REQUEST message the en-gNB shall stop providing user data to the UE. The SgNB UE X2AP ID IE shall be included if it has been obtained from the en-gNB.

If the bearer context in the en-gNB was configured with the PDCP entity in the en-gNB, for E-RAB for which the MeNB requests forwarding of uplink/downlink data, the MeNB includes the UL Forwarding GTP Tunnel Endpoint/ DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB RELEASE REQUEST message to indicate that the en-gNB should perform data forwarding of uplink/downlink packets for that E-RAB.

Upon reception of the SGNB RELEASE REQUEST message containing UE Context Kept Indicator IE set to "True", the en-gNB shall, if supported, only initiate the release of the resources related to the UE-associated signalling connection between the MeNB and the en-gNB.

If the en-gNB confirms the request to release en-gNB resources it shall send the SGNB RELEASE REQUEST ACKNOWLEDGE message to the MeNB.

If the RLC Mode IE is included for an E-RAB within the E-RABs Admitted To Be Released List IE (for E-RABs hosted at the en-gNB) in the SGNB RELEASE REQUEST ACKNOWLEDGE message, it indicates the mode that the en-gNB used for the E-RAB when it was hosted at the en-gNB.

If the MeNB did not include the SgNB UE X2AP ID IE in the SGNB RELEASE REQUEST message, the MeNB shall ignore the SgNB UE X2AP ID IE in the SGNB RELEASE REQUEST ACKNOWLEDGE message.

Upon successful completion of the procedure, the MeNB shall start counting time, so that information regarding time since Secondary Node Release may be transferred towards the MME as specified in TS 36.413 [4].

If the SCG UE History Information IE is included in the SGNB RELEASE REQUEST ACKNOWLEDGE message, the MeNB node shall, if supported, use this information as specified in TS 37.340 [32].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the E-RABs transferred to MeNB IE are included in the SGNB RELEASE REQUEST message, then the en-gNB shall, if supported, include the uplink/downlink PDCP SN and HFN status for the listed E-RABs, as specified in TS 37.340 [32].

Interaction with Data Forwarding Address Indication procedure:

If the SGNB RELEASE REQUEST message concerns UE for which a CPC has been triggered, the en-gNB shall, if supported, consider that the triggered Conditional PSCell Change has been executed, and act as specified in TS 37.340 [32].

-----------------------------------------------------------Next change-----------------------------------------------------

#### 8.7.10.2	Successful Operation

Figure 8.7.10.2-1: SgNB initiated SgNB Release, successful operation.

The en-gNB initiates the procedure by sending the SGNB RELEASE REQUIRED message to the MeNB.

Upon reception of the SGNB RELEASE REQUIRED message, the MeNB replies with the SGNB RELEASE CONFIRM message. For each E-RAB configured with the PDCP entity in the en-gNB, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer.

If the RLC Mode IE is included for an E-RAB within the E-RABs To Be Released List IE (for E-RABs hosted at the en-gNB) in the SGNB RELEASE REQUIRED message, it indicates the mode that the en-gNB used for the E-RAB when it was hosted at the en-gNB.

If the SgNB to MeNB Container IE is included in the SGNB RELEASE REQUIRED message, the MeNB may use the contained information to apply delta configuration.

The en-gNB may start data forwarding and stop providing user data to the UE upon reception of the SGNB RELEASE CONFIRM message.

Upon successful completion of the procedure, the MeNB shall start counting time, so that information regarding time since Secondary Node Release may be transferred towards the MME as specified in TS 36.413 [4].

If the SCG UE History Information IE is included in the SGNB RELEASE REQUIRED message, the MeNB node shall, if supported, use this information as specified in TS 37.340 [32].

-----------------------------------------------------------Next change-----------------------------------------------------

9.2.177	SCG UE History Information

This IE contains information about the PSCells served by the secondary node in an active state.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerSN> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. |
| >Last Visited PSCell Information | M |  | OCTET STRING | The PSCell related information. Defined in TS 36.413 [4]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerSN | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8 |

-----------------------------------------------------End of the change-------------------------------------------------------