3GPP TSG-RAN WG3 Meeting #117-e	R3-224978

E-meeting, 15 – 24 Aug, 2022

Agenda item:	12.2

Source:	Xiaomi

Title:	Data collection via MDT in split architecture

Document for:	Discussion and Decision

1	Introduction

The R18 AI RAN WID introduced the following objectives in the core part:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

In TR 37.817, it states that “RAN3 shall reuse the existing framework (including MDT and RRM measurements)” and “MDT procedure enhancements should be discussed during the normative phase.”, which means the MDT measurement can be used for AI model training/inference/evaluation.

As indicated in the objective, the split architecture should be considered in R18, while we find there may be an issue for m-based MDT in split architecture to support data collection for AI purposes.

In this contribution, we’d like to discuss the issue and possible solutions that data collection via MDT for AI purposes in split architecture.

2	Discussion

As specified in TS 32.422, “In the case of split RAN architecture, the management system may send the MDT activation directly to gNB-CU-CP, gNB-DU, or gNB-CU-UP.” If the EM send the MDT directly to gNB-DU or gNB-CU-UP, and the gNB-DU or gNB-CU-UP can perform the UE selection and assign TRSR without notifying gNB-CU-CP, after finish MDT measurements, the gNB-DU or gNB-CU-UP will sent the MDT report directly to TCE without gNB-CU-CP involvement. The gNB-CU-CP is not aware of the whole measurement and reporting process in gNB-DU and gNB-CU-UP. The following Figure 1 is an example for gNB-DU when the EM directly sent the MDT activation to gNB-DU.

Figure 1 direct m-based MDT activation in gNB-DU

Observation 1, EM can directly send the m-based MDT activation to gNB-DU or gNB-CU-UP without gNB-CU-CP involvement, and the gNB-DU or gNB-CU-UP can send the MDT report to TCE without gNB-CU-CP involvement.

Observation 2, the AI function in gNB-CU-CP cannot collect the MDT measurements in gNB-DU or gNB-CU-UP that is directly activated by EM.

To solve the issue, two possible options can be considered.

- Option1, gNB-CU-CP indicate the gNB-DU or gNB-CU-UP it has AI function or it want to collect data for AI purposes, once the gNB-DU or gNB-CU-UP is configured MDT measurements directly by EM, it will notify the gNB-CU-CP the MDT configuration, so that the gNB-CU-CP can collect the measurements in gNB-DU or gNB-CU-UP

- Option 2, specify that the gNB-DU or gNB-CU-UP should notify the gNB-CU-CP the MDT configuration, then the gNB-CU-CP can consider it to decide whether to request the MDT measurement for AI purposes.

We think both options can be further discussed.

Proposal 1, RAN3 to discuss how the gNB-CU-CP obtains the MDT measurement that is directly activated to gNB-DU or gNB-CU-UP by EM.

3	Conclusion

In this contribution, we discussed the possible scenarios and general principle of dynamic PCI/RACH (re)configuration to minimize the interference due to the movement of mobile IAB, and we had the following observations and proposals:

Observation 1, EM can directly send the m-based MDT activation to gNB-DU or gNB-CU-UP without gNB-CU-CP involvement, and the gNB-DU or gNB-CU-UP can send the MDT report to TCE without gNB-CU-CP involvement.

Observation 2, the AI function in gNB-CU-CP cannot collect the MDT measurements in gNB-DU or gNB-CU-UP that is directly activated by EM.

Proposal 1, RAN3 to discuss how the gNB-CU-CP obtains the MDT measurement that is directly activated to gNB-DU or gNB-CU-UP by EM.

4	References