3GPP TSG-RAN WG3 #117-e	R3-225083

Online, 15. – 24.08 2022

Agenda Item:	14.3

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CHO with NR-DC 
(CB #MobilityEnh2_CHO)

Document for:	Approval

# Introduction

CB: # MobilityEnh2_CHO

- Focus on scenarios and potential issue, e.g. if forwarding the same data over multiple paths to the same target SN.

- If CHO is executed first, whether the associated CPAC config should be maintained or released?

- Whether to support early data forwarding, CHO in NR-DC, CHO with C-MN initiated CPA/CPC, and identify RAN3 impacts?

- Capture agreements and open issues

# For the Chairman’s Notes (2nd round)

WA: RAN3 agrees to create a separate chapter in TS 37.340 related to CHO with DC, and to do it as part of the work on the Rel.18 Mobility Enhancements. The new chapter will also address two existing scenarios: DC at source (as described currently in TS37340 chapter 10.8) and DC at target (as described currently in TS37340 chapter 10.7).

# Discussion (1st round)

## CHO with SCG at the target (cont. of Rel.17 work)

There are three separate threads related to the continuation of the work on CHO with a classic DC operation at the target side, as started in Rel.17:

- In [4251], it is proposed to continue the work and enhance the data forwarding from the source to the target SN.

- In [4519], it is proposed to describe the Rel.17 signalling in more detailed form in stage-2.

- In [4510], it is proposed to complete the Rel.17 signalling so that possible SCG reconfiguration before CHO execution is addressed (based on a decision in RAN2).

Question 1: Please, which of the above points should further be addressed in Rel.18.

| Company | Points (a, b, c) | Please, explain why some topics are preferred to be excluded from the Rel.18 WI. |
| --- | --- | --- |
| Nokia | (a) | We consider (b) and (c) as corrections to the mechanism agreed in Rel.17, so we are fine to discuss these as corrections (point (c) is already addressed in CB # 10). However, data forwarding has not been addressed yet, so it should be addressed as a new feature in Rel.18. |
| E/// | a, b | Regarding the signaling flows proposed in (b), we would continue addressing here and then as indicated in objectives, i.e., whether it belongs to R17 will be reviewed in next RP. We are open to discuss data forwarding in (a). For c, agree with Nokia. |
| CATT | a | Agree with Nokia. For b and c, we may wait for the WID updating after RP meeting |
| ZTE | (a) | For (a), in previous release, the legacy mechanism is that early data forwarding is handled during conditional reconfiguration preparation phase. For R18 CHO with CPAC, if it is one-step execution, then we think it is similar to legacy, however, if it is two-step execution, the early data forwarding can be handled after CHO execution but before CPAC execution phase. So, R18 early data forwarding procedure is depended on the down selection of one-step/two-step execution. (b) We think (as CR R3-224520) it shall be belonged to R17, and it can extend to R18. However, we are not sure of the need of new signalling. (c) We agree with both Nokia and Ericsson. |
| China Telecom | a | We agree to enhance data forwarding form source SN to target SN, because it much benefit to relieve the network load. For b) and c), we share similar view with Nokia. |
| Huawei | a |  |
| Google | a |  |
| Lenovo | a, c | For c), we propose two failure cases to be considered in [4436] that will impact the signalling. We are also fine to discuss this point in R17 this meeting. For a), we agree with Nokia. For b), we think it belongs to R17. |
| Qualcomm | (a) | Agree with Nokia. |
| vivo | c |  |
| Samsung | (a) | Data forwarding aspect needs to be discussed. We can discuss (b) and (c) in Rel-17 correction (CB #10) before the RP’s decision. |
| Intel | (a) | Agree with Nokia. And not sure about (b) but (c) is currently being discussed in CB #10. |

## CHO with multiple SCGs at the target

In numerous papers it is acknowledged to work on CHO with multiple SCGs prepared at the target side ([4252, 4269, 4322, 4343, 4394, 4794, 4834, 4436, 4509, 4793]). However, most of them list issues that are either purely RAN2’s domain or depend on RAN2’s progress.

Question 2: Please, comment if RAN3 can start considering particular signalling solutions without waiting for RAN2’s progress.

| Company | Comment |
| --- | --- |
| Nokia | At this moment, we don’t see much space for discussing principles of signalling. We could agree some general principles like that we’ll try to reuse existing procedures defined for CHO and CPAC, but we can’t really move much further. |
| E/// | Fine to wait for RAN2’s progress. |
| CATT | Fine to wait for RAN2 |
| ZTE | We are not sure which RAN group to decide either one-step execution or two-step execution. Can RAN3 decide it? |
| China Telecom | Prefer to wait for RAN2 progress. |
| Huawei | Agree with Nokia. |
| Google | Fine to wait for RAN2 progress |
| Lenovo | We can start the working on some scenarios and general principles, e.g., Xn/F1 signalling for the CHO including target MCG and target SCG should be used as baseline. |
| Qualcomm | Fine to wait for RAN2 progress. |
| vivo | We agree most issues listed need input from RAN2 to make progress. |
| Samsung | We are also fine to wait for RAN2 progress. |
| Intel | Agree with the majority view. |

In [4394], [4794] and [4834], it is proposed to start considering early data forwarding for the CHO with multiple SCGs prepared at the target side.

Question 3: Please, comment if RAN3 can start working on the early data forwarding for CHO with multiple SCGs prepared at the target side.

| Company | Comment |
| --- | --- |
| Nokia | Yes, we agree, we can start discussing principles for early data forwarding. |
| E/// | Agree that this is a valid point. |
| CATT | We can start the working on this point. |
| ZTE | Of course, early data forwarding is a good point. However, in our view, it is depended on either one-step or two-step execution. If it is one-step execution, then we think it is similar to legacy, however, if it is two-step execution, the early data forwarding can be handled after CHO execution but before CPAC execution phase. |
| China Telecom | Agree to start working on the early data forwarding. |
| Huawei | ok |
| Google | ok |
| Lenovo | Agree to start the working on early data forwarding with some general principles. |
| Qualcomm | Yes, we agree. |
| vivo | Yes, some initial discussion on early data forwarding is possible. |
| Samsung | Yes, we agree. |
| Intel | Fine with the proposal. |

# Discussion (2nd round)

## Separate stage-2 description

In [4520], it is proposed to open a separate chapter in TS 37.340 to describe conditional aspects of the MCG and SCG mobility. During the online discussion, this seemed to be widely supported approach, similarly like CHO is described separately in 3x.300.

The first question is therefore to confirm this preference, as there are two options:

- YES, the conditional MR-DC scenarios enabled already in Rel.17 shall also be described separately and thus the existing descriptions transferred to the new chapters (concerns the notes in 10.7, as agreed at this meeting, and in 10.8).

- NO, the existing notes that concern Rel.17 scenarios, i.e. CHO with DC at the source (chapter 10.8) and at the target (chapter 10.7), shall be kept as they were described in Rel.17. Only new Rel.18 scenarios may be described separately (up to the discussion in the Rel.18 WI).

Question 4: Please, provide your preference concerning one of the options above.

| Company | Yes / No | Possible comments |
| --- | --- | --- |
| Nokia | Yes |  |
| ZTE | No | I wish R17 spec shall be keep stable. I support it in the R18. If my remember is correct, for R17 CPAC, at the beginning, RAN3 wishes to introduce a separate procedure in TS37.340, but it is not agreed until RAN2 changed their view. So, without RAN2 involved, we shall not consider it in R17. |
| Qualcomm | No | We were leaning towards “Yes” but changed our mind after considering ZTE’s comment regarding possible RAN2 involvement, and the comment by Nokia in response to Question 6 which indicates the possibility of further corrections needed to the Rel-17 specification if a more thorough review at RAN3 #117-bis reveals other issues due to the change. The current specification is quite stable, and we prefer it this way. We support it for Rel-18. |
| CATT | No. | We prefer to keep R17 as it is. |
| China Telecom | No | Agree with ZTE. |
| Lenovo | Yes | It is nice to have overall signalling flows for the coexistence of CHO and MR-DC, which could be used as baseline for R18 discussion. |
| E/// | Yes | CHO+MR-DC has been discussed for several meetings, and we can see the notes are continued being added as a patch, so we are not able to keep a “world-level” standard. Therefore it is important to have a clean section in stage-2 to avoid any future inconsistency of updates between R17 and R18 onwards. |
| Samsung | No | We support it in Rel-18. The current specification is stable and Rel-17 CHO with DC could been referred as one case of Rel-18 CHO with DC, so the complete set of signaling flows for CHO with DC could be given in Rel-18. |
| Intel | No | Tend to agree with ZTE. |
| Huawei | No | do not see the need to do it for now, can wait for further progress of the Rel-18 technical discussion. |

If the Rel.17 CHO with MR-DC scenarios are to be transferred to the new chapter (answer ‘yes’ above), it needs to be clarified which scenarios we mean (the proposal in [4520] does not remove any existing text):

- The Rel.17 CHO with DC at the source is described in the new chapter (and thus the CHO-related notes in 10.8 are removed).

- The Rel.17 CHO with DC kept at the target is described in the new chapter (and thus the CHO-related newly added notes in 10.7 are removed).

Question 5: If your answer was ‘yes’ in Q4, please, provide your preference concerning what scenarios are to be transferred to the new chapter.

| Company | 1 / 2 / both | Possible comments |
| --- | --- | --- |
| Nokia | Both |  |
| Lenovo | Both |  |
| E/// | Both |  |

Finally, if the change is to affect any of the Rel.17 scenarios, the question is, since when the change is to be introduced:

- We introduce the new chapter and needed shifting of the text in Rel.17 (as a Rel.17 correction).

- We leave Rel.17 as is and introduce the new chapter and related changes in the existing ones in Rel.18, as part of the work on the Mobility Enhancement WI.

Question 6: If your answer was ‘yes’ in Q4, please, provide your preference concerning since when the change is to be introduced.

| Company | 1 or 2 | Possible comments |
| --- | --- | --- |
| Nokia | 2 or neutral | If majority prefers to correct the Rel.17 specification, we will accept it; however, we can’t accept the correction at this meeting yet, we need time until RAN3 #117-bis to review the Rel.17 functionality to make sure something is not broken. |
| Lenovo | 1 | We prefer to do it as R17 correction. We also agree with Nokia that the correction could be done at next meeting. |
| E/// | 1 | We understand Nokia’s concern. And a starting point will be needed, i.e., considering the new sections as agreement and reviewing the detailed changes afterwards, e.g., how to mitigate the existing notes to the new sections. |

# Conclusion, Recommendations [if needed]

If needed

# References

- [4251]	R3-224251, Continuation of the work on CHO with DC and optimisation of the data forwarding (Nokia, Nokia Shanghai Bell)

- [4252]	R3-224252, CHO with multiple candidate SCGs (Nokia, Nokia Shanghai Bell)

- [4269]	R3-224269, Discussion on CHO with CPA (ZTE)

- [4270]	R3-224270, New procedure for support of CHO with CPA feature to TS37.340 (ZTE)

- [4322]	R3-224322, Consideration on CHO related aspects (Huawei)

- [4343]	R3-224343, Support of CHO with CPAC (vivo)

- [4394]	R3-224394, Consideration on support of CHO including target MCG and SCGs (China Telecommunication)

- [4436]	R3-224436, Discussion on CHO in NR-DC (Lenovo)

- [4509]	R3-224509, CHO including target MCG and candidate SCGs (Qualcomm Incorporated)

- [4510]	R3-224510, CHO with SCG configuration (Qualcomm Incorporated)

- [4519]	R3-224519, Outstanding issues for CHO + MR-DC (Ericsson)

- [4520]	R3-224520, Introduction of signaling flows for CHO+MR-DC (Ericsson)

- [4793]	R3-224793, Discussion on the scenarios of CHO with multiple candidate SCGs (CATT)

- [4794]	R3-224794, Discussion on the procedure of CHO with multiple candidate SCGs (CATT)

- [4834]	R3-224834, (TP to TS37.340 on Mobility Enhancements)Considerations on CHO+CPAC procedure (Samsung)

- [4835]	R3-224835, (TP to TS38.423 on Mobility Enhancements) Considerations on CHO+CPAC configuration (Samsung)